| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G908 |
| --- | --- |

| Title: | Cross-check of BBC’s transform skip mode (JCTVC-G575) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution confirms the results presented in contribution JCTVC-G575 on transform skip mode.

- Introduction

Software implementing JCTVC-G575 [1] on top of HM4.0 basis was released by BBC as part of Adhoc Group 19 on Transform skipping. BD-rate provided by BBC match with the obtained ones.

This cross-check focuses on the TSM mode-dependant scans of NSQT in JCTVC-G575 (section 2.2). The code introduces horizontal and vertical scans for the NSQT blocks dependant on the TSM mode. This code is straightforward.

- Results

Coding results using common conditions (High-Efficiency settings) are summarized in the tables below. HM4.0 is the anchor. Note that decoding times are not reliable since different types of machines were used to decode. TSM is applied to inter frames.

|  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.08% | -0.23% | -0.28% |  |  |  |  |  |  |
| Class B | -0.25% | 0.07% | 0.27% | -0.28% | 0.86% | 1.26% | -0.39% | 0.68% | 1.11% |
| Class C | -0.37% | -0.16% | 0.17% | -0.62% | 0.23% | 0.67% | -0.80% | 0.16% | 0.52% |
| Class D | -0.70% | -0.11% | -0.02% | -1.13% | 0.96% | 0.94% | -1.41% | 0.55% | 0.32% |
| Class E |  |  |  | -0.21% | 1.61% | 2.39% | -0.33% | 2.38% | 1.99% |
| Overall | -0.31% | -0.10% | 0.05% | -0.57% | 0.87% | 1.24% | -0.73% | 0.83% | 0.93% |
| Enc Time[%] | 139% |  |  | 134% |  |  | 143% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  | 102% |  |  |

Results for class F are the following:

| Class F | -1.91% | -1.04% | -1.10% | -4.64% | -3.04% | -2.86% | -4.97% | -3.26% | -3.40% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- M. Mrak, A. Gabriellini, D. Flynn, “Transform skip mode”, JCTVC-G575, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.