| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G909 WG11 number: m224954 |
| --- | --- |

| Title: | Non-CE1: Cross-check of JCTVC-G833 on adaptive initialization for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-checks performed by the Institute for Infocomm Research on Qualcomm’s contribution JCTVC-G833 which proposes to perform adaptive initialization of CABAC when using fixed probability contexts. The cross-check performance results reportedly match those provided by Qualcomm. The implemented is also reportedly in line with Qualcomm’s proposal.

# Introduction

In JCTVC-G833, Qualcomm is proposing adaptively initializing CABAC when context model updates are disabled, i.e., fixed probability contexts are used [1]. In particular, some adaptation based on sequence and/or frame statistics can still be done by having the encoder send CABAC initialization parameters to the decoder.

With CABAC, the initial probability state is computed by using:

InitState = Clip(1,126,(m*QP>>4)+n)

where m is the slope of the InitState-QP relationship, and n is the intercept. A pair of (m,n) is used to initialize each context.

In the implementation provided by Qualcomm, context updates are disabled. A flag in the slice header indicates if the proposed adaptive initialization of CABAC states should be used. If so, a map is transmitted to indicate for each context if the default CABAC initialization parameters (m,n) are to be used, or if an adaptive initial state value is to be used; a ‘0’ entry in the map indicates the former while a ‘1’ entry in the map indicate the latter. In transmitting the map, run-length coding is used to code the number of ‘0’s before each ‘1’. For each context that is indicated to use an adaptive initial state value, the initial state value to be used is signaled. This initial state value is quantized using a step size of 2.

During encoding, statistics regarding each syntax element is collected, which estimates the probability distribution of each syntax element. After a slice is encoded, its statistics are analyzed to determine what should be done for the next slice with the same slice type and QP. For each context, a decision is made as to whether the default initialization should be used, or if an adaptive initial state based on statistics from the current slice should be used. The decision is based on two checks. The first check is whether the default initialization and the adaptive state differ by more than or equal to a threshold (2 in this case). The second check is whether the scaled number of bits spent sending the adaptive initial state (the scaling used is 1.2) is less than the bits saved by using the adaptive initial state. If both checks pass, the difference between the adaptive state to be used and the state computed from the default (m,n) would be stored. For the next slice encoded with the same slice type and QP, the stored decisions used for encoding and are also sent to the decoder, which can then reconstruct the adaptive initial state to use.

# Simulation Results

We have studied and run the software provided by Qualcmom, and have verified the results provided by them. Tests were conducted on Intra, Random Access, Low Delay and Low Delay (P) configurations for the both high-efficiency and low-complexity setting [2]. Note that in both high-efficiency and low-complexity setting, CABAC is used as the entropy code (“SymbolMode=1”), and context updating is turned off.

Two cases were tested. In the first case, existing (m,n) parameters in HM4 were used. In the second case, a different set of (m,n) parameters trained for the case of no context updating is used.

## Case 1 – HM4 CABAC initialization

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.9% | -0.5% | 0.1% | 0.1% | 0.1% |
| Class B | 0.7% | -0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.0% | 0.1% | 0.4% | 0.3% | 0.3% |
| Class D | 0.1% | 0.7% | 1.1% | 1.1% | 0.9% | 1.0% |
| Class E | 0.7% | -0.7% | -0.9% | 0.5% | 0.4% | 0.4% |
| Overall | 0.3% | -0.2% | 0.0% | 0.4% | 0.4% | 0.4% |
|  | 0.3% | -0.1% | 0.1% | 0.4% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.6% | 4.6% | 4.0% | -7.0% | 11.4% | 10.2% |
| Class B | 2.7% | 2.8% | 2.8% | -5.1% | 5.5% | 7.2% |
| Class C | 2.4% | 3.4% | 3.6% | -3.4% | 5.0% | 5.5% |
| Class D | 3.0% | 4.7% | 4.7% | -2.1% | 7.5% | 7.9% |
| Class E |  |  |  |  |  |  |
| Overall | 2.4% | 3.8% | 3.7% | -4.5% | 7.2% | 7.7% |
|  | 2.8% | 3.9% | 3.8% | -4.1% | 7.2% | 7.7% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.5% | 3.1% | 3.2% | -5.1% | 8.8% | 13.1% |
| Class C | 2.2% | 3.7% | 3.6% | -3.7% | 7.8% | 8.8% |
| Class D | 3.1% | 6.5% | 7.1% | -2.1% | 13.7% | 14.2% |
| Class E | 3.7% | -0.1% | -1.1% | -3.9% | 5.6% | 5.9% |
| Overall | 2.8% | 3.5% | 3.5% | -3.8% | 9.2% | 10.9% |
|  | 2.8% | 3.6% | 3.6% | -3.7% | 9.2% | 10.9% |
| Enc Time[%] | 99% |  |  | 104% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.3% | 1.5% | 1.7% | -5.5% | 8.7% | 12.9% |
| Class C | 2.0% | 2.2% | 2.1% | -4.1% | 6.7% | 7.7% |
| Class D | 2.6% | 3.9% | 4.5% | -2.8% | 12.2% | 12.2% |
| Class E | 4.0% | -0.8% | -3.1% | -4.2% | 5.4% | 5.5% |
| Overall | 2.6% | 1.9% | 1.6% | -4.3% | 8.4% | 10.0% |
|  | 2.6% | 2.0% | 1.6% | -4.1% | 8.6% | 10.1% |
| Enc Time[%] | 99% |  |  | 106% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |

## Case 2 – Alternative CABAC initialization

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.6% | -0.3% | -6.0% | 0.2% | 0.7% |
| Class B | 0.5% | -0.1% | 0.1% | -5.4% | 0.6% | 1.8% |
| Class C | 0.3% | -0.1% | 0.0% | -4.4% | 0.4% | 0.6% |
| Class D | 0.0% | -1.3% | -1.1% | -4.0% | -0.1% | 0.1% |
| Class E | 0.2% | -0.9% | -1.1% | -7.3% | 5.1% | 5.2% |
| Overall | 0.1% | -0.6% | -0.4% | -5.3% | 1.0% | 1.5% |
|  | 0.1% | -0.5% | -0.4% | -5.3% | 1.0% | 1.6% |
| Enc Time[%] | 100% |  |  | 107% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 4.0% | 3.2% | -8.1% | 11.4% | 10.0% |
| Class B | 1.3% | 1.1% | 1.2% | -6.2% | 4.3% | 6.1% |
| Class C | 1.0% | 1.2% | 1.2% | -4.4% | 3.2% | 3.9% |
| Class D | 1.4% | 1.5% | 1.4% | -3.3% | 5.6% | 5.3% |
| Class E |  |  |  |  |  |  |
| Overall | 1.0% | 1.9% | 1.7% | -5.5% | 6.0% | 6.3% |
|  | 1.4% | 2.0% | 1.8% | -5.1% | 6.0% | 6.2% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | 0.9% | 1.0% | -6.1% | 6.9% | 11.5% |
| Class C | 0.8% | 1.6% | 1.3% | -4.7% | 5.7% | 6.4% |
| Class D | 1.2% | 2.8% | 2.1% | -3.6% | 11.4% | 11.6% |
| Class E | 2.1% | -0.7% | -0.4% | -4.8% | 5.8% | 6.4% |
| Overall | 1.3% | 1.3% | 1.1% | -4.9% | 7.5% | 9.3% |
|  | 1.3% | 1.3% | 1.2% | -4.8% | 7.4% | 9.3% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | 1.5% | 1.7% | -6.2% | 9.1% | 13.6% |
| Class C | 0.9% | 2.0% | 1.7% | -4.7% | 6.5% | 7.7% |
| Class D | 1.3% | 3.7% | 3.3% | -3.6% | 12.5% | 12.7% |
| Class E | 1.8% | -1.3% | -1.8% | -5.3% | 7.6% | 7.3% |
| Overall | 1.3% | 1.7% | 1.5% | -5.0% | 9.0% | 10.7% |
|  | 1.3% | 1.7% | 1.5% | -4.9% | 9.0% | 10.8% |
| Enc Time[%] | 100% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 95% |  |  |

# Conclusions

We have cross-checked Qualcomm’s proposal JCTVC-G833 on adaptive initialization of CABAC contexts when no context updates are done, studied the algorithm and verified their coding simulation results.

# References

[1] L. Guo et al., “Non-CE1: Adaptive initialization for CABAC with fixed probability contexts,” in JCTVC-G833, Geneva, Nov. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.