| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G911 WG11 Number: m22497 |
| --- | --- |

| Title: | Cross verification of KDDI’s intra Chroma prediction based on residual luma samples (JCTVC-G346) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang, Lidong Xu, Yu Han, 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 wenhao.zhan, lidong.xu, yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of KDDI’s proposed new intra chroma prediction mode, i.e., RM mode, based on residual luma samples. In RM mode, the chroma residuals are produced in the first step by using the prediction mode derived from Luma mode, and then are predicted from reconstructed Luma residuals (down-sampled in vertical direction and sub-sampled in horizontal direction) by applying a linear model. The parameter alpha is derived on encoder side and then coded for decoder use, and the parameter beta is implicitly coded in DC coefficient.

The software package obtained from the proponent was successfully compiled and tested under the common test conditions for AI_HE and AI_LC cases. The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations. Detailed performance data are shown in Table 1.

Table 1 Crosscheck results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -12.0% | -14.9% | -0.1% | -11.1% | -13.7% |
| Class B | 0.0% | -2.6% | -2.5% | -0.1% | -2.2% | -1.7% |
| Class C | -0.1% | -2.2% | -2.7% | -0.1% | -1.7% | -2.0% |
| Class D | -0.1% | -2.1% | -2.3% | -0.1% | -1.5% | -1.7% |
| Class E | -0.1% | -1.1% | -0.9% | 0.0% | -0.8% | -0.7% |
| Class F | -0.3% | -2.8% | -3.4% | -0.3% | -2.3% | -2.6% |
| Overall | -0.1% | -3.9% | -4.5% | -0.1% | -3.3% | -3.8% |
|  | -0.1% | -3.8% | -4.4% | -0.1% | -3.3% | -3.7% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |