| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G915r2 |
| --- | --- |

| Title: | Coding and selection of SAO parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Baylon and Koohyar Minoo 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2285 david.baylon@motorola.com kminoo@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In HM4.0, the determination of the SAO offset value is based upon only distortion considerations.  This contribution proposes to determine the offset based on RD considerations.  In addition, this contribution proposes to increase the number of EO classes to eight, and change the coding of the SAO type for HE.  Simulations reportedly show no significant loss in luma coding for HE and LC, while a bit rate savings of up to 1.1% for chroma.  For encoder offset optimization only, simulations reportedly show no significant change in luma coding performance, while a bit rate savings of up to 0.5% for chroma.

# Introduction

In SAO, the offset per class is determined as the average distortion per pixel, where the total distortion per class is divided by the number of pixels in the class.  With more classes, there is the opportunity to further reduce the distortion but at the cost of increased bit rate from sending the additional offsets.  Section 2 describes the proposed encoder RD optimization for the offset, and Section 3 proposes its use when the number of EO classes is increased from four (transmitted offsets) to eight.  Section 4 presents simulation results relative to HM4.0, and Section 5 summarizes the contribution.

# Encoder offset optimization

In HM4.0, the SAO offset for a given class of a given type is computed by minimizing the distortion.  On the other hand, the distortion and rate for coding and using these offsets are used to compute the Lagrangian cost J for each type.  The minimum total cost over all the types is then used to select the best SAO type.  This contribution proposes to select individual SAO offsets for each type, by minimizing a Lagrangian cost function which includes both rate and distortion.  Note that currently HM4.0 minimizes the distortion to assign the offsets.

# Increase in number of EO classes

In HM4.0, there are four EO types and five classes per EO type. For the four EO types, the offsets for four (out of five) classes per type are coded and transmitted to the decoder.  This contribution proposes to split up the existing five classes into nine classes, where each class represents when the current pixel (being classified) is greater than, equal to, or less than its neighboring pixels (along the EO type direction).  Offsets for eight of the nine classes for a given EO type are then transmitted to the decoder.  By combining the increased number of EO classes with the encoder offset optimization described in Section 2, there is a potential for improved performance.

# Simulation results

The proposed methods were implemented on HM4.0.  Results of encoder offset optimization only are reported in Table 1.  Table 2 reports the results with encoder offset optimization combined with the increase in EO classes to eight.  It should be noted that in these latter results, the coding of the SAO type in the HE case was modified so as to reduce the bits for coding the BO types.  Also, encoder and decoder timing was not provided as the results were not accurate.

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | Class B | 0.0% | -0.2% | -0.4% | 0.0% | -0.1% | -0.1% |
|  | Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
|  | Class D | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | Class E | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | 0.0% |
|  | Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
|  |  | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
|  |  | Random Access HE |  |  | Random Access LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.0% | -0.4% | -0.5% | 0.0% | 0.0% | 0.0% |
|  | Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.1% |
|  | Class C | 0.0% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% |
|  | Class D | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.3% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |
|  |  | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.0% | -0.5% | -0.7% | -0.1% | -0.3% | -0.2% |
|  | Class C | 0.0% | -0.3% | -0.4% | -0.1% | -0.1% | -0.4% |
|  | Class D | -0.1% | 0.2% | -0.5% | 0.0% | -0.3% | -0.4% |
|  | Class E | -0.1% | -0.4% | -0.1% | -0.1% | -0.3% | -0.1% |
|  | Overall | -0.1% | -0.3% | -0.5% | -0.1% | -0.2% | -0.3% |
|  |  | -0.1% | -0.3% | -0.5% | -0.1% | -0.3% | -0.3% |
|  |  | Low delay P HE |  |  | Low delay P LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.1% | -0.3% | -0.4% | 0.0% | 0.1% | -0.2% |
|  | Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.3% |
|  | Class D | 0.0% | -0.4% | -0.4% | 0.0% | -0.3% | -0.7% |
|  | Class E | -0.1% | -0.8% | -1.0% | -0.1% | -0.4% | -0.6% |
|  | Overall | -0.1% | -0.4% | -0.5% | 0.0% | -0.2% | -0.4% |
|  |  | -0.1% | -0.5% | -0.5% | 0.0% | -0.2% | -0.4% |

Table 1.  Performance using encoder offset optimization only.

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.0% | -0.9% | -0.8% | 0.0% | -0.7% | -0.8% |
|  | Class B | 0.1% | -0.8% | -1.5% | 0.1% | -0.8% | -1.4% |
|  | Class C | 0.0% | -0.5% | -0.6% | 0.0% | -0.3% | -0.4% |
|  | Class D | 0.0% | -0.5% | -0.5% | 0.0% | -0.4% | -0.4% |
|  | Class E | 0.0% | -1.5% | -1.2% | 0.0% | -1.5% | -1.4% |
|  | Overall | 0.0% | -0.8% | -1.0% | 0.0% | -0.7% | -0.9% |
|  |  | 0.0% | -0.8% | -1.0% | 0.0% | -0.7% | -0.9% |
|  |  | Random Access HE |  |  | Random Access LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -0.1% | -1.8% | -1.6% | 0.0% | -0.7% | -0.9% |
|  | Class B | -0.1% | -1.0% | -1.8% | 0.0% | -0.9% | -1.3% |
|  | Class C | 0.0% | -0.5% | -0.5% | 0.0% | -0.3% | -0.2% |
|  | Class D | 0.0% | -0.5% | -0.4% | 0.0% | -0.6% | -0.6% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.0% | -1.0% | -1.1% | 0.0% | -0.6% | -0.8% |
|  |  | 0.0% | -0.9% | -1.1% | 0.0% | -0.6% | -0.8% |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.1% | -1.2% | -1.7% | -0.1% | -1.5% | -1.7% |
|  | Class C | 0.0% | -0.4% | -0.7% | 0.0% | -0.5% | -0.7% |
|  | Class D | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.6% |
|  | Class E | 0.0% | -2.6% | 0.0% | -0.1% | -1.3% | -1.4% |
|  | Overall | 0.0% | -1.0% | -0.7% | 0.0% | -0.9% | -1.1% |
|  |  | 0.0% | -1.0% | -0.8% | 0.0% | -0.9% | -1.1% |
|  |  | Low delay P HE |  |  | Low delay P LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.0% | -0.9% | -1.5% | 0.1% | -0.6% | -1.1% |
|  | Class C | 0.0% | -0.2% | -0.5% | 0.0% | -0.3% | -0.4% |
|  | Class D | 0.0% | -0.1% | -0.5% | 0.0% | 0.1% | -0.1% |
|  | Class E | 0.0% | -1.7% | 0.1% | 0.1% | -1.2% | 0.1% |
|  | Overall | 0.0% | -0.7% | -0.7% | 0.1% | -0.5% | -0.5% |
|  |  | 0.0% | -0.7% | -0.7% | 0.1% | -0.5% | -0.5% |

Table 2.  Performance using encoder offset optimization, eight EO classes, and a change in the coding of SAO type for the HE case.

# Summary

This contribution proposes improvements to SAO coding.  In particular, the contribution proposes encoder offset optimization, increasing the number of SAO Edge classes to eight, and a change in the coding of the SAO type for the HE case.  For encoder offset optimization only, simulations reportedly show no significant change in luma coding for HE and LC, while a bit rate savings of up to 0.5% for chroma.  For all three proposed changes, simulations reportedly show no significant loss in luma coding for HE and LC, while a bit rate savings of up to 1.1% for chroma.  It is suggested that the proposed methods be considered for adoption in HEVC.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).