| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G917 |
| --- | --- |

| Title: | On significance map coding in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Hun-Seok Kim Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes removal of neighboring dependencies for context selection of significant_coeff_flag located at high frequencies. The neighbors used for context selection in significant_coeff_flag were carefully chosen such that context selection does not depend on recently decoded bins.  However, when reverse scan (from high frequency to low frequency) is used, which was adopted in Turin for HM-4.0, this dependency exists for the high frequency positions (lower right corner at the start of the scan). This contribution proposes for removing dependency for 3 and 6 high frequency positions.  No additional contexts are required.  Negligible impact on coding efficiency was observed in HM-4.0 under common conditions and low QP (0.0 % AI-HE, 0.0% RA-HE, 0.0% LD-HE).

# Introduction

For the significant_coeff_flag of 16x16 and 32x32 TU, the context selection depends on neighboring values as show in Fig. 1.  The neighbors used for context selection in significant_coeff_flag were carefully chosen such that context selection does not depend on recently decoded bins.  However, when reverse scan (from high frequency to low frequency) is used, which was adopted in Turin for HM-4.0, this dependency exists for the high frequency positions (lower right corner at the start of the scan) as shown in Fig. 2.   This work proposes that high frequency positions shown in white in Fig. 2 don’t depend on neighbors for context selection and instead use a fix context (i.e. assume all neighbors are zero). No additional contexts are required.

B

E

F

H

I

X

Fig.1 – Simplified context selection for significant map in HM-4.0.  Context for position X depends on B, E, F, H and I.

Fig.2 – When processing the significant_coeff_flag with reverse diagonal scan, the blue position depends on positions highlighted in pink.  This will make it difficult to do 4 or more bins in parallel.

6 high frequency positions (L=3)

3 high frequency positions (L=2)

X

X

Fig. 3 –Proposed context selection of significant_coeff_flag. High frequency positions highlighted in red do not depend on neighbors.

# Experiment Results

The HM-4.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  |  | L=3 |  |  | L=2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | All Intra LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  | 101% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 96% |  |  |

Table 1 Average loss from removing neighboring dependency for context selection of significant_coeff_flag at high frequency positions under common conditions. Positive numbers in the table means coding loss.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104.8% |  |  | 96.2% |  |  |
| Dec Time[%] | 104.4% |  |  | 98.3% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98.9% |  |  | 99.9% |  |  |
| Dec Time[%] | 97.7% |  |  | 100.9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100.9% |  |  | 99.2% |  |  |
| Dec Time[%] | 103.6% |  |  | 98.0% |  |  |

Table 2 Average loss from removing neighboring dependency for context selection of significant_coeff_flag at high frequency positions for lowQP (QP=12,17,22,27). Positive numbers in the table means coding loss.

# Text for Working Draft

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise if xC + yC is greater than (( 1 << ( log2TrafoSize + 1) ) -3) , sigCtx is derived as follows.

sigCtx  =   34

- Otherwise (xC  +  yC is greater than 4 and less than (( 1 << ( log2TrafoSize + 1) ) -2)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-11)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

# Summary

This contribution proposes the removal neighboring dependency for context selection of significant_coeff_flag at high frequency positions.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).