| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011 | Document: JCTVC-G918 WG11 Number: m22508 |
| --- | --- |

| Title: | Non-CE8: Cross-check of JCTVC-G445 on removing DC component of ALF filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for Non-CE8 JCTVC-G445 on removing DC component of ALF filter coefficients. Rate-PSNR numbers match exactly to the proponent’s results. Enc/dec time measurements are within minor 1%~2% differences, which is normal considering cross-check is done on Linux cluster. The removal of DC components is done by not estimating the DC component in the MMSE filter design process at encoder and not transmitting them in the bitstream.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | Results by the proponent ( JCTVC-G445 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | -0.1 | -0.1 | 0.0 | -- | -0.1 | -0.1 | 0.0 | -0.1 |
| Enc. Time | 96% | 99% | 99% | -- | 96% | 99% | 99% | 99% |
| Dec. time | 101% | 101% | 102% | -- | 101% | 103% | 103% | 104% |

# Findings in software study

The removal of DC component in ALF is done by:

- Not estimating the DC component in the MMSE filter design process at encoder.

- Setting the DC component to 0 and not transmitting it in the bitstream.

# References

- “Removing DC component of ALF filter coefficients”, JCTVC-G445, Geneva, Switzerland, Nov. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).