| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011 | Document: JCTVC-G919 WG11 Number: m22509 |
| --- | --- |

| Title: | Non-CE8: Cross-check of JCTVC-G146 on CU-based ALF control re-design process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for Non-CE8 JCTVC-G146 on CU-based ALF control re-design process. The number of ALF re-design iteration under each CU depth for on/off control is reduced, simply by setting ALF_NUM_OF_REDESIGN to 2, 1, and 0 (default value in HM4.0 is 3). Rate-PSNR numbers match exactly to the proponent’s results. Encoding time reduction is confirmed with less re-design iterations, while decoding time has normal < 3% variations due to Linux cluster setup.

The cross-check also reports comparisons between reduced iteration of ALF re-design (JCTVC-G146) and the updated “2-pass” and “1-pass” ALF reported in CE8 Summary Report (JCTVC-G038). The results indicate it is possible to improve ALF design trade-off simply by reducing the re-design iteration, rather than designing “2-pass” and “1-pass” methods which introduce additional process.

It is recommended to consider reduced ALF_NUM_OF_REDESIGN in common test conditions.

# Cross-check results

The simulations were run in the 64-bit Linux cluster environment.

(1)	ALF_NUM_OF_REDESIGN set to 2

|  | Results by the proponent ( JCTVC-G146 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | 0.0 | 0.0 | 0.1 | -- | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc. Time | 92% | 98% | 99% | -- | 92% | 98% | 99% | 98% |
| Dec. time | 100% | 100% | 100% | -- | 101% | 102% | 101% | 102% |

(2)	ALF_NUM_OF_REDESIGN set to 1

|  | Results by the proponent ( JCTVC-G146 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | 0.0 | 0.1 | 0.2 | -- | 0.0 | 0.1 | 0.2 | 0.2 |
| Enc. Time | 83% | 96% | 97% | -- | 83% | 96% | 97% | 96% |
| Dec. time | 100% | 100% | 100% | -- | 101% | 103% | 101% | 102% |

(3)	ALF_NUM_OF_REDESIGN set to 0

|  | Results by the proponent ( JCTVC-G146 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | 0.0 | 0.2 | 0.4 | -- | 0.0 | 0.2 | 0.4 | 0.7 |
| Enc. Time | 74% | 93% | 95% | -- | 74% | 93% | 96% | 93% |
| Dec. time | 99% | 100% | 101% | -- | 101% | 103% | 102% | 105% |

# Software study

- The number of ALF re-design iteration under each CU depth for on/off control is reduced, simply by setting ALF_NUM_OF_REDESIGN to 2, 1, and 0 (default value in HM4.0 is 3).

- Thus, there is only one-line change of the code!

- HM4.0 contains additional code for “2-pass” and “1-pass”, which has been updated [2] for the newly adopted ALF shapes and the associated shape selection process [3]. This cross-check also compared the reduced number of re-design iteration to the updated 2-pass and 1-pass. The results below indicate it is possible to achieve better design trade-off simply by reducing the re-design iteration, rather than designing “2-pass” and “1-pass” methods which introduce additional process.

(1)	“ALF_NUM_OF_REDESIGN 2” v.s “Updated 2-pass”

|  | ALF_NUM_OF_REDESIGN 2 (Samsung cross-check G146) |  |  |  | Updated 2-pass ALF [2] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc. Time | 92% | 98% | 99% | 98% | 95% | 99% | 99% | 99% |
| Dec. time | 101% | 102% | 101% | 102% | 101% | 99% | 99% | 99% |

(2)	ALF_NUM_OF_REDESIGN set to 1

|  | ALF_NUM_OF_REDESIGN 1 (Samsung cross-check G146) |  |  |  | Updated 1-pass ALF [2] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | LDP | AI | RA | LD | LDP |
| BD-rate | 0.0 | 0.1 | 0.2 | 0.2 | 0.0 | 0.2 | 0.2 | 0.3 |
| Enc. Time | 83% | 96% | 97% | 96% | 95% | 99% | 99% | 99% |
| Dec. time | 101% | 103% | 101% | 102% | 101% | 99% | 99% | 99% |

# References

- “Non-CE8: Information on ALF coefficient re-designing process”, JCTVC-G146, Geneva, Switzerland, Nov. 2011

- “CE8: Summary report of Core Experiment on non-deblocking loop filtering”, JCTVC-G038, Geneva, Switzerland, Nov. 2011

- “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients”, JCTVC-F303, Torino, Italy, Jul. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).