| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G925 |
| --- | --- |

| Title: | Cross-check of JCTVC-G550 on TU_size based dQP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-3481 vseregin@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a cross-check report of JCTVC-G550 related to implicit derivation of dQP based on transform size.

# Introduction

Software of JCTVC-G550 [1], implemented on top of HM4.0, was released by a proponent.

It was verified that the implementation related to dQP signaling is consistent with G550 description, and the following files were modified:

TypeDef.h, TComRom.h, TComRom.cpp, TComTrQuant.h, TComTrQuant.cpp, TEncSearch.cpp, TDecCU.cpp, TDecGop.cpp

Proposed method is applied only to Inter slices and consists of dQP modification according to the TU size by the following rule.

| TU size | 4x4 | 8x8 | 16x16 | 32x32 |
| --- | --- | --- | --- | --- |
| Implicit dQP | 2 | 1 | 0 | -1 |

# Results

Simulation results under common test conditions with HM4.0 as an anchor are summarized in the below tables. Encoding and decoding time may have inaccuracy since software was executed on non-uniform cluster. PSNR and bitrate are matched with proponent’s data.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 1.5% | 1.3% | 0.2% | 1.6% | 1.5% |
| Class B | 0.1% | 0.5% | 0.2% | 0.1% | 0.4% | 0.2% |
| Class C | -0.2% | -0.4% | -0.3% | -0.4% | -0.6% | -0.8% |
| Class D | -0.1% | -0.8% | -1.0% | -0.3% | -0.9% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.1% | -0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.4% | 0.2% | -0.2% | 0.2% | 0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.9% | 1.2% | -0.1% | 1.2% | 1.5% |
| Class C | -0.5% | -0.3% | -0.2% | -0.9% | -0.2% | -0.5% |
| Class D | -0.5% | -0.1% | -0.6% | -0.8% | 0.4% | -0.1% |
| Class E | -0.1% | 1.4% | 1.2% | -0.2% | -0.4% | -0.3% |
| Overall | -0.3% | 0.4% | 0.4% | -0.5% | 0.4% | 0.3% |
|  | -0.3% | 0.4% | 0.4% | -0.5% | 0.5% | 0.5% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |

- References

- C. Kim, Y. Park, K.P. Choi and J.H. Park, “TU_size based dQP”, JCTVC-G550, 7th JCT-VC Meeting, Geneva, CH, November 2011.