| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G926 |
| --- | --- |

| Title: | Non-CE13: Cross-check of JCTVC-G787 on MV dependent offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-3481 vseregin@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a cross-check report of JCTVC-G787 part related to motion vector dependent offset, using instead of constant offset as it is in CE13, for merger/skip modes.

# Introduction

Software of JCTVC-G787 [1] was released by a proponent, implemented on top of HM4.0+MRG_ENC_FIX in combination with the CE13 test 3.5.1 with macro CE13_3_5_CANON_TEST_1.

It was verified that the implementation under MV_DEP_OFFSET macro, related to motion vector (MV) offset modification, is consistent with G787 description, and the following files were modified:

TypeDef.h, TComDataCU.h

Proposed method consists of replacing constant offset used in CE13 by offset of 8, 4, 2, and 1, depending on absolute value of motion vector component of the first MV candidate.

# Results

Simulation results under common test conditions with HM4.0+MRG_ENC_FIX as an anchor are summarized in the below table1. Encoding and decoding time may have inaccuracy since software was executed on non-uniform cluster. PSNR and bitrate are matched with proponent’s data.

Since proposed method is implemented on top of HM4.0+MRG_ENC_FIX+CE13.3.5.1, comparison should be done against CE13 3.5.1 test as HM4.0+MRG_ENC_FIX+CE13.3.5.1. CE13 3.5.1 test result is provided for reference in the following table2. Low delay P configuration was run during cross-check due to absence of this data in CE report.

Table1. Proposed method on top of HM4.0+MRG_ENC_FIX+CE13.3.5.1 against HM4.0+MRG_ENC_FIX anchor

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.26% | -0.12% | -0.14% | -0.16% | -0.42% |
| Class B | -0.21% | -0.03% | 0.03% | -0.34% | -0.15% | -0.25% |
| Class C | -0.15% | -0.11% | -0.10% | -0.22% | -0.10% | -0.25% |
| Class D | -0.22% | 0.02% | -0.11% | -0.28% | -0.22% | -0.29% |
| Class E |  |  |  |  |  |  |
| Overall | -0.15% | -0.09% | -0.07% | -0.25% | -0.16% | -0.30% |
|  | -0.16% | -0.07% | -0.05% | -0.26% | -0.17% | -0.28% |
| Enc Time[%] | 103.4% |  |  | 103.7% |  |  |
| Dec Time[%] | 100.0% |  |  | 100.2% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.16% | 0.43% | 0.09% | -0.32% | 0.06% | -0.22% |
| Class C | -0.31% | -0.35% | -0.26% | -0.24% | 0.16% | 0.10% |
| Class D | -0.27% | 0.34% | -0.04% | -0.33% | -0.25% | 0.16% |
| Class E | 0.35% | 0.71% | 0.41% | 0.27% | 1.13% | 0.29% |
| Overall | -0.13% | 0.26% | 0.03% | -0.19% | 0.21% | 0.05% |
|  | -0.12% | 0.29% | -0.01% | -0.19% | 0.15% | 0.05% |
| Enc Time[%] | 103.2% |  |  | 102.8% |  |  |
| Dec Time[%] | 100.3% |  |  | 99.5% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.41% | 0.14% | -0.11% | -0.87% | -0.24% | -0.38% |
| Class C | -0.32% | -0.24% | -0.44% | -0.64% | -0.31% | -0.52% |
| Class D | -0.58% | -0.39% | -0.49% | -0.77% | -0.47% | -0.71% |
| Class E | 0.14% | 0.69% | 1.07% | -0.38% | 1.72% | 1.05% |
| Overall | -0.33% | 0.01% | -0.06% | -0.70% | 0.05% | -0.23% |
|  | -0.33% | 0.01% | -0.05% | -0.69% | 0.00% | -0.21% |
| Enc Time[%] | 105.2% |  |  | 105.4% |  |  |
| Dec Time[%] | 101.0% |  |  | 99.8% |  |  |

Table2. CE13 3.5.1 test result against HM4.0+MRG_ENC_FIX [2]

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.10% | -0.40% | -0.31% | -0.13% | -0.32% | -0.62% |
| Class B | -0.21% | -0.16% | -0.09% | -0.27% | -0.18% | -0.26% |
| Class C | -0.10% | -0.10% | -0.07% | -0.14% | -0.17% | -0.25% |
| Class D | -0.10% | -0.16% | 0.08% | -0.14% | -0.41% | -0.28% |
| Class E |  |  |  |  |  |  |
| Overall | -0.13% | -0.20% | -0.10% | -0.18% | -0.26% | -0.35% |
|  | -0.14% | -0.20% | -0.09% | -0.20% | -0.27% | -0.34% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.37% | -0.27% | -0.31% | -0.47% | -0.27% | -0.47% |
| Class C | -0.30% | -0.57% | -0.41% | -0.22% | -0.03% | -0.05% |
| Class D | -0.24% | -0.09% | -0.36% | -0.29% | -0.43% | 0.04% |
| Class E | 0.15% | 1.12% | 0.16% | 0.06% | 1.14% | 0.59% |
| Overall | -0.22% | -0.04% | -0.26% | -0.26% | 0.01% | -0.04% |
|  | -0.22% | -0.04% | -0.27% | -0.26% | -0.05% | -0.06% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.36% | -0.19% | -0.31% | -0.64% | -0.42% | -0.49% |
| Class C | -0.20% | -0.35% | -0.21% | -0.38% | -0.55% | -0.58% |
| Class D | -0.26% | -0.27% | -0.78% | -0.27% | -0.47% | -0.78% |
| Class E | 0.42% | 0.39% | 1.14% | 0.13% | 1.10% | 0.60% |
| Overall | -0.15% | -0.14% | -0.13% | -0.34% | -0.18% | -0.38% |
|  | -0.15% | -0.08% | -0.12% | -0.34% | -0.20% | -0.35% |
| Enc Time[%] | 108% |  |  | 107% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

- References

- T. Lee, J. Chen, J. H. Park, G. Laroche, P. Onno, J.-L. Lin, Y.-W. Chen, Y.-W. Huang and S. Lei, “Non-CE13: Additional merge candidates with MV dependent offsets”, JCTVC-G787, 7th JCT-VC Meeting, Geneva, CH, November 2011.

- G. Laroche, T. Poirier and P. Onno, “CE13: Experiment regarding section 3.5”, JCTVC-G240, 7th JCT-VC Meeting, Geneva, CH, November 2011.