| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G929 |
| --- | --- |

| Title: | Cross check of Qualcomm`s deblocking filter simplifications (JCTVC-G290) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Kotra Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com +49 6103 766-1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

Cross check results of Qualcomm`s deblocking deblocking filter simplifications (JCTVC-G290) is reported. The source codes correspond to the proposal descriptions. The BD-rate results match perfectly and the execution times are approximately similar to the HM4.0 anchor times.

# Deblocking weak filter simplifications

The calculation of  = ( 9*(q0-p0) - 3*(q1-p1) + 8 )/16 in HM-4.0 weak filter is  changed to                                                                           =  ( 3*(q0-p0) - (q1-p1) + 4 )/8

The BD-rate results are obtained according to the HM4 common test conditions [2]. The execution times were measured in a heterogeneous computing environment.

The objective results are as follows:

# Strong/Weak Decision Simplification

d<β>>2 and p3i−p0i+q0i−q3i<β>>3 and p0i−q0i<5⋅tc+1>>1Decision                                                                                                                                                                  is performed for line=2 and line =5. The decision result of line i=2 is used for first 4 lines/columns of 8-sample edge segment and the decision result of line i=5 is used for second 4 lines/columns of 8-sample edge segment. As a result the number of strong/weak filter decisions for an edge segment is reduced from eight to two.

The BD-rate results are obtained according to the HM4 common test conditions [2]. The execution times were measured in a heterogeneous computing environment.

The objective results are as follows:

# References

- HM 4.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).