| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G930r1 WG11 Number: m22523 |
| --- | --- |

| Title: | CE7: Cross check report of “On secondary transforms for intra prediction residual (G108)” |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

The contribution reports cross-verification results on mode-dependent secondary transforms (MD-ST) for intra prediction residual. The tool is unified tool of F224 and F554 in Torino meeting. And it is applied to 8x8 and higher DCT coefficients without any additional signaling. We tested hardware friendly implementation of 8x8 MD-ST. The verification task has been done successfully and the results were identical to provided ones by proponents.

# Mode dependent secondary transform

This tool is secondary transform depending on the intra prediction mode for an 8x8 and larger TUs. The secondary transform is applied to lower frequency coefficients of transformed TUs. In the case of 8x8 TUs, it was identical to mode dependent DCT/DST in theory. The experiment is conducted with 8x8 secondary transform toward 8x8, 16x16 and 32x32 TUs. The conducted software is designed with regards to the hardware implementation.

# Cross verification

## Test conditions

The source code, which was integrated MD-ST into HM4.0, was provided from proponents. Because this proposal applies to only intra TUs, the cross-verification conditions are AI (All Intra) and RA (Random Access) cases. The all verifications were performed on Linux 64 bit OS with gcc-4.1.2 with the same computer systems. Default conditions were according to JCEVC-F900 [2].

## Test results

The test results are shown in Table 1 and 2.

## Complexity analysis

We counted the operations number. The secondary transform is calculated by using matrix multiplier. Therefore the number of multiplier is derived from 2N3, the number of adder is derived from 2N2(N-1+1) and the number of shift is derived from 2N2 for 2D transform. The appearance frequency of intra prediction mode was assumed as equal probability for calculating average cost. The counting results were identical to the result of proponent.

Additionally, we counted the operation number of simplified G108. The simplification is only on the 8x8 TUs. In the case of 8x8 TU, the secondary transform are applied only horizontal or vertical, when the intra prediction mode is the one of VER-1, VER-2, … VER-8 or HOR-1, HOR-2, … HOR-7 and Planar. The counting results of this simplified case were also identical to the result of proponent.

# Summary

We confirmed the implementation of MD-ST and cross-checked the coding results. The results show 0.7% gain with AIHE and 0.7% gain with AILC. These results are identical to the result of proponent. The process time was increased 3% with encoding time and 4% with decoding time. MDST is implemented clear and the results were identical to the results of proponent.

# References

- A. Saxena, Y. Shibahara, F. C. Fernandes and T. Nishi, “CE7: On secondary transforms for intra prediction residual,” Doc. JCTVC-G108, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, Nov. 2011

- “Common test conditions and software reference configurations.” Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, July 2011