| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G931 |
| --- | --- |

| Title: | Non CE3: Cross-check for memory band-width reduction from Toshiba (G-770) by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for memory band-width reduction proposal from Toshiba. Performance matches with data reported by proponent. We can confirm 0.3% BD-rate drop (all color components) with 25%-31% (depending on memory architecture) memory band-width reduction in the worst case.

# Introduction

Toshiba proposes non-normative restriction for memory band-width reduction in the worst case. An encoding method that at least one motion vector of L0 or L1 is restricted to the integer position for both luma and chroma when the block size of bi-prediction is less than 8x8 is introduced. The worst case from view point of memory band with after proposed modification will be 4x8 Luma bi-predictive block with 2D interpolation on one reference frame and integer MV on opposite side or 8x8 bi-predictive block with 21D interpolation on both references (depends on memory architecture)..

# Test results

We run test using test conditions [1] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [2]. There is no performance change compare to anchor in LD(P) test since bi-prediction only was modified.

The summary test results is shown in Table 1 (in APPENDIX). In average across only common test cases RA-HE/LC, LD(B)-HE/LC proposed non-normative change shows 0.3%(Y), 0.3%(V), 0.3%(U) BD-rate drop without noticeable change of encoding decoding time. Complexity measurement used in CE3 indicates 1% computations reduction and 2% memory band-width reduction in average.

Proposed change reduces the worst case memory access at least 25% but some MVs restriction on encoder side. We would like to notice that some combinations of MS in bi-prediction are over-restricted. For example, the worst case for memory access after proposed change with be 2D-I bi-prediction (one reference use 2D interpolation, but an opposite uses integer pel MC). This MC requires 6.88 pixels access in memory per of output. MV combination 1D-1D which needs 5.50 pixels per on pixel output is restricted. If we’ll keep this possibility for bi-prediction then performance drop will be less, but the worst case memory access will not change.

CE3 studied memory band-width for several different memory architectures. Table 2 shows memory band-width for restricted in G770 1D-1D case compare to the worst case memory band width in G770. For all memory architectures if 1D-1D will not be restricted the worst case memory band-width will not change (but performance drop will b e definitely less).

As it proposed in [3] some mechanism for memory band-width restriction is needed. Proposed by Toshiba modification can be one of the possible solutions. With more accurate restriction losses can be minimized.

Table 2. Memory band-width for 1D-1D bi-prediction restricted in G770) compare to the worst case.

| Bi-pred | MemBand(2D 4x2) | MemBand(2D 8x2) | MemBand(1D 4x1) | MemBand(1D 8x1) |
| --- | --- | --- | --- | --- |
| 1D-1D | 13.5 | 24.0 | 16.0 | 22.0 |
| Worst case G770 | 15.0 | 24.0 | 16.5 | 22.5 |

There are at least two more related contributions on memory band-width reduction in CE3 and we recommend compare them together.

# Conclusions

We confirm performance reported by Toshiba for encoder only modification reducing the worst case memory band-width:

0.3%(Y), 0.3%(V), 0.3%(U) BD-rate drop

with 1% computations reduction and 2% memory band-width reduction in average.

We recommend studying further with interesting proposal and try to minimize losses by more accurate MV restriction.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- T.Chujoh, E. Alshina, “CE3: interpolation MC”, JCTVC-F903, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- Hui Yong Kim, Kyung Yong Kim, Gwang Hoon Park, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi “MC Complexity Reduction for Bi-prediction”, JCTVC-G095, Geneva, Switzerland, November 2011

- M. Zhou (TI) “Virtual motion compensation memory bandwidth verifier (VMBV)”, JCTVC-G415, Geneva, Switzerland, November 2011

APPENDIX

Table 1. Summary of solid interpolation filter tests.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | -0.2% | 0.2% | 0.4% | 0.2% |
| Class B | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
| Class C | 0.3% | 0.3% | 0.3% | 0.4% | 0.4% | 0.3% |
| Class D | 0.6% | 0.4% | 0.6% | 0.5% | 0.4% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
|  | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 6% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.0% | -0.2% | 0.2% | 0.2% | 0.0% |
| Class C | 0.4% | 0.4% | 0.5% | 0.4% | 0.4% | 0.1% |
| Class D | 0.7% | 1.0% | 0.8% | 0.7% | 0.6% | 0.5% |
| Class E | 0.2% | 0.0% | 0.5% | 0.1% | 0.2% | 0.1% |
| Overall | 0.4% | 0.3% | 0.4% | 0.3% | 0.4% | 0.2% |
|  | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -1% |  |  | -1% |  |  |
| Adds[%] | -1% |  |  | -1% |  |  |
| Access[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 3% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 3% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 0% |  |  | 0% |  |  |

| Gain Ave.[%] | 0.22% | 0.20% | 0.18% |
| --- | --- | --- | --- |
| Gain Min. [%] | -0.01% | -0.66% | -1.44% |
| Gain Max.[%] | 1.16% | 1.43% | 1.40% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
| Mult Ave.[%] | -0.45% |  |  |
| Mult Worst[%] | 0.00% |  |  |
| Adds Ave.[%] | -0.45% |  |  |
| Adds Worst[%] | 0.00% |  |  |
| Access[%] | -0.75% |  |  |
| MemBand(2D) Ave.[%] | -1.17% |  |  |
| MemBand(2D) Max.[%] | 2.62% |  |  |
| MemBand(2D) Worst[%] | -25.00% |  |  |
| MemBand(1D) Ave.[%] | -1.10% |  |  |
| MemBand(1D) Max.[%] | 2.46% |  |  |
| MemBand(1D) Worst[%] | -31% |  |  |