| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G932 |
| --- | --- |

| Title: | CE1: Cross-check for test 12 in subset C from HHI (JCTVC-G633) by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

Test 12 in CE1 subset C was originally announced by Cisco. HHI has found and corrected the bug in code provided for test 12. This report is cross-check for corrected by HHI version. In average -5.2% (Y)/ -0.5% (U) /0.1%(V) BD-rate change is confirmed by our measurement.

# Description

In test probability up-date (PMU) in CABAC should be disabled during RDO mode selection in order to simulate realistic encoder. For this test RDOQ was disabled both for anchor and modification. We also set CABAC as entropy coder in LC configuration and tested vs HM4.0 LC as anchor.

We run test for AI-LC, RA-LC, LD(B)-/LC and LD(P)-LC according to [1]. Test results are shown in Table 1. Anchor for this test was provided by HHI (match with the same test made by Cisco) so we tested only modification.

If anchor uses CALVC then tested modification provided -5.2% (Y), -0.5% (U) ,0.1%(V) BD-rate change.

If anchor uses CABAC then ested modification provided 0.5% (Y), 0.8% (U) ,1.0%(V) BD-rate change (there results are reported in Table 2).

Table 1. Performance test summary: RDOQ off, Anchor uses CAVLC, modification uses CABAC but PMU is disabled during ROD search.

| All Intra LC. Y; U; V. Class A; -4.8%; -2.9%; -4.2%. Class B; -5.8%; -3.8%; -3.1%. Class C; -4.2%; -2.8%; -2.5%. Class D; -3.4%; -2.7%; -2.4%. Class E; -7.1%; -1.2%; -1.0%. Overall; -5.0%; -2.8%; -2.7%. -5.0%; -2.8%; -2.7%. | Random Access LC. Y; U; V. -7.2%; 3.2%; 2.0%. -6.4%; -0.9%; -0.3%. -4.4%; -1.3%; -0.6%. -3.7%; -1.0%; -0.1%. -5.5%; -0.1%; 0.2%. -5.0%; -0.2%; 0.1%. |
| --- | --- |
| Low delay B LC. Y; U; V. Class A. Class B; -6.0%; 1.5%; 3.3%. Class C; -4.4%; 1.2%; 2.2%. Class D; -3.6%; 3.1%; 4.0%. Class E; -6.7%; -0.2%; 2.1%. Overall; -5.1%; 1.5%; 3.0%. -5.1%; 1.4%; 2.9%. | Low delay P LC. Y; U; V. -6.4%; 2.2%; 4.2%. -4.7%; 1.2%; 2.4%. -3.9%; 2.7%; 4.5%. -7.1%; 0.6%; 2.1%. -5.5%; 1.8%; 3.4%. -5.5%; 1.7%; 3.2%. |

Table 2. Performance test summary: RDOQ off, Anchor uses CABAC, modification uses CABAC but PMU is disabled during ROD search.

| All Intra LC. Y; U; V. Class A; 0.5%; 0.9%; 0.8%. Class B; 0.7%; 1.7%; 2.1%. Class C; 0.6%; 0.9%; 1.0%. Class D; 0.5%; 0.8%; 0.8%. Class E; 0.5%; 1.3%; 1.1%. Overall; 0.6%; 1.1%; 1.2%. 0.6%; 1.1%; 1.2%. | Random Access LC. Y; U; V. 0.5%; 0.6%; 0.7%. 0.3%; 1.2%; 1.7%. 0.3%; 0.6%; 1.0%. 0.2%; 0.5%; 0.5%. 0.3%; 0.8%; 1.0%. 0.3%; 0.8%; 1.0%. |
| --- | --- |
| Low delay B LC. Y; U; V. Class A. Class B; 0.5%; 1.0%; 1.2%. Class C; 0.5%; 0.8%; 1.2%. Class D; 0.4%; 0.1%; 0.3%. Class E; 0.3%; 0.4%; 0.0%. Overall; 0.4%; 0.6%; 0.8%. 0.4%; 0.7%; 0.7%. | Low delay P LC. Y; U; V. 0.4%; 1.3%; 1.5%. 0.5%; 0.7%; 1.2%. 0.4%; -0.5%; 0.2%. 0.3%; 0.0%; 0.0%. 0.4%; 0.4%; 0.8%. 0.4%; 0.4%; 0.8%. |

More details can be found in excel sheet attached to this report.

# Conclusions

We confirm performance reported by HHI in CE1 subset C test 12:

-5.2% (Y)/ -0.5% (U) /0.1%(V) BD-rate change.

# References

- R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema, “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, IT, July 2011.