| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G936 |
| --- | --- |

| Title: | Cross verification of NEC's proposal JCTVC-G213 on TMVP performance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin Jianhua Zheng | Email: | yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of NEC's proposal JCTVC-G213 on TMVP performance. The test cases are RA and LB, both with HE and LC configurations. The tests are based on HM-4.0 according to common test conditions. The cross-check has been completed successfully and the results match those provided by proponent.

# Test description and simulation results

In CTVC-G213, the performance of temporal MV prediction (TMVP) is evaluated for the comparison purpose. In our test, TMVP is disabled by using the following macro definitions in TypeDef.h

#define MRG_NEIGH_COL 0

#define AMVP_NEIGH_COL 0

Four cases of RA-HE, RA-LC, LB-HE and LB-LC are tested under common test conditions [1]. The test results are compared with HM4.0. The BD performance is shown in Table 1. Noticed that the reference in Table 1 is HM4.0 with TMVP disabled. It is observed these results match that reported in JCTVC-G213. The decoding and encoding run time has the similar characteristics as those of proponent.

Table 1 TMVP disabled v.s TMVP enabled

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -1.9% | -2.0% | -2.6% | -2.3% | -2.4% |
| Class B | -2.1% | -1.5% | -1.4% | -2.3% | -1.4% | -1.4% |
| Class C | -2.2% | -1.9% | -2.0% | -2.3% | -1.9% | -2.0% |
| Class D | -2.6% | -2.0% | -2.0% | -2.8% | -1.9% | -2.1% |
| Class E |  |  |  |  |  |  |
| Overall | -2.4% | -1.8% | -1.8% | -2.5% | -1.9% | -2.0% |
|  | -2.4% | -1.8% | -1.8% | -2.5% | -1.9% | -2.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.2% | -2.7% | -2.3% | -4.1% | -2.7% | -3.1% |
| Class C | -3.4% | -3.3% | -3.4% | -3.9% | -3.3% | -3.4% |
| Class D | -3.1% | -2.7% | -2.4% | -3.5% | -2.9% | -2.8% |
| Class E | -3.4% | -3.1% | -3.6% | -4.1% | -2.7% | -3.0% |
| Overall | -3.2% | -2.9% | -2.8% | -3.9% | -2.9% | -3.1% |
|  | -3.2% | -2.9% | -2.8% | -3.9% | -2.9% | -3.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# References

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.