| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G941 WG11 Number: m22535 |
| --- | --- |

| Title: | AHG19: Cross-check of JCTVC-G663, modifications to HE transform coefficient coding for transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-G663, “AHG19: Modifications to HE transform coefficient coding for transform skip mode.” A brief analysis of the software implementation is followed by a summary of experimental results. The simulation results performed for the cross-verification match those provided by the proponents, and examination of the software confirms that for High Efficiency configurations, it operates with a transform skip mode while using a fixed number of bits to code the position of the last non-zero coefficient when skip mode is used.

# Introduction and examination of software

Two flag are used to determine whether to use transform skip mode along the rows and columns. When the transform is enabled for a given direction, the existing HM transform is used in that direction. When the transform is disabled, i.e. transform skip mode is enabled, only a scaling and transposing is performed as follows:

int j,k;

int add = 1<<(shift-1);

int scale = NTScale[(int)g_aucConvertToBit[width]];

for (j=0; j<width; j++) {

for (k=0; k<height; k++) {

dst[j*height+k] = (src [k*width+j] * scale + add)>>shift;

}

}

For coding the position of the last significant coefficient, HM uses a string of 0s, ending with a termination 1 when the X or Y coordinate is less than half the block width, as specified in Working Draft 4 (WD4) [1]. Any additional distance past the half-width point is signaled with a fixed number of bits. When the transform is applied in both directions, as is the case in HM, the last nonzero coefficient can frequently be encountered very early in the scan order, especially for high QP values. In these cases, only a few bits would be needed to signal the X and Y positions of the last nonzero coefficients. An example code segment from HM 4.0 for signaling positions less than half the block width is as follows:

for( uiCtxLast = 0; uiCtxLast < uiPosX; uiCtxLast++ )

{

m_pcBinIf->encodeBin( 0, *( pCtxX + puiCtxIdx[ uiCtxLast ] ) );

}

if( uiPosX < uiMaxWidth - 1 )

{

m_pcBinIf->encodeBin( 1, *( pCtxX + puiCtxIdx[ uiCtxLast ] ) );

}

When transform skip mode is used, there is no energy compaction in that direction, meaning that the last nonzero coefficient may occur quite late in the scan order. The method proposed in JCTVC-G663 [2] instead codes this position using a fixed number of bits. The corresponding segment of code is as follows:

for (Int z=g_aucConvertToBit[uiWidth]+1; z >=0; z--)

{

m_pcBinIf->encodeBinEP((uiPosX0>>z) & 0x01);

}

Thus, the number of signaled bits is a function of the block size.

# Performance of proposed method

The proposed method from JCTVC-G663 was implemented on top of HM 4.0. For this cross verification, simulations using software provided by the proponents were compared to a reference coded with HM 4.0, using the common conditions described in [3]. Results without Class F sequences are shown in Table 1, and results including Class F are shown in Table 2. More detailed results can be found in the accompanying spreadsheets.

Table 1. Performance of proposed method, without Class F sequences

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.8% | -0.3% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | -0.4% | -0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | -0.4% | -0.3% | 0.0% |
| Enc Time[%] | 101% |  |  | 141% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 1.1% | 1.3% | -0.5% | 0.7% | 1.3% |
| Class C | -0.8% | 0.1% | 0.6% | -1.0% | 0.1% | 0.6% |
| Class D | -1.2% | 1.2% | 0.9% | -1.5% | 0.7% | 0.5% |
| Class E | -0.3% | 2.6% | 1.7% | -0.4% | 3.0% | 1.6% |
| Overall | -0.7% | 1.2% | 1.1% | -0.9% | 1.0% | 1.0% |
|  | -0.7% | 1.0% | 1.0% | -0.9% | 0.9% | 0.9% |
| Enc Time[%] | 133% |  |  | 148% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

Table 2. Performance of proposed method, including Class F sequences

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.8% | -0.3% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | -2.2% | -1.4% | -1.4% |
| Overall | 0.0% | 0.0% | 0.0% | -0.7% | -0.4% | -0.3% |
|  | 0.0% | 0.0% | 0.0% | -0.7% | -0.5% | -0.3% |
| Enc Time[%] | 101% |  |  | 140% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 1.1% | 1.3% | -0.5% | 0.7% | 1.3% |
| Class C | -0.8% | 0.1% | 0.6% | -1.0% | 0.1% | 0.6% |
| Class D | -1.2% | 1.2% | 0.9% | -1.5% | 0.7% | 0.5% |
| Class E | -0.3% | 2.6% | 1.7% | -0.4% | 3.0% | 1.6% |
| Class F | -5.1% | -3.7% | -3.5% | -5.5% | -3.8% | -3.7% |
| Overall | -1.6% | 0.2% | 0.2% | -1.8% | 0.0% | 0.1% |
|  | -1.6% | 0.1% | 0.1% | -1.8% | 0.0% | 0.0% |
| Enc Time[%] | 133% |  |  | 147% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# Conclusions

The quantitative results obtained during this cross verification match those provided by the proponents. Examination of the software confirms that it is indeed performing transform skip mode, while coding the position of the last nonzero coefficient using a fixed number of bits dependent upon the block size. Run times for the proposed tool match, allowing for expected variations due to the compute platform.

# References

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803_d5, Torino, IT, July 2011.

- R. Joshi et al., “AHG19: Modifications to HE transform coefficient coding for transform skip mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G663, Geneva, CH, November 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, Torino, IT, July 2011