| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G944 |
| --- | --- |

| Title: | Cross-check report on additional tests of reordering of merge candidate (JCTVC-G396) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check on additional tests of reordering of merge candidate (JCTVC-G396)

In this document, report on additional tests of reordering of merge candidate is provided. In the experiments, simplification and proposed methods are evaluated. For simplification test, 2Nx2N emulation is avoided for 2NxN, Nx2N and AMP prediction units. Proposed methods are evaluated compared to simplification in order to verify benefits when the simplification is applied. Brief description of each test is given in Table 1.

Table 1. Brief description of tests

| Simplification | HM4.0 MrgEncFix | HM4.0 MrgEncFix + ‘simplification’ | Avoiding 2Nx2N emulation |
| --- | --- | --- | --- |
| Simplification + Proposed method 2 + Proposed method 1 | HM4.0 MrgEncFix + ‘simplification’ | HM4.0 MrgEncFix + ‘simplification’ + Proposed method 2 + Proposed method 1 | Coding efficiency test based on simplification |

Software was provided by the proponent of JCTVC-G396. It is confirmed that the source code has modified according to the description and the results under common test condition [1] are same with the results of JCTVC-G396.

Table 2. Experimental results of simplification

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| Class D | 0.1% | 0.4% | 0.6% | 0.0% | 0.0% | 0.4% |
| Class E | 0.1% | -0.2% | 0.3% | -0.1% | 0.2% | -0.1% |
| Overall | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3. Experimental results of simplification + proposed methods 2 + proposed method 1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -0.2% |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% |
| Class D | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% |
| Class C | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | -0.1% |
| Class D | -0.2% | -0.2% | -0.7% | -0.2% | -0.1% | -0.3% |
| Class E | -0.3% | -0.5% | -0.9% | -0.2% | -0.2% | 0.3% |
| Overall | -0.2% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.