INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 M22556

November 2011, Geneva, Switzerland

| Source: | WG11 AHG on Study of HEVC Extensions |
| --- | --- |
| Purpose: | Information and Discussion |
| Title: | Draft Requirements and Discussion on the scalable enhancement of HEVC |
| Chair: | Ajay Luthra |
| Co-Chairs: | Jens-Rainer Ohm, Jörn Ostermann |

Abstract

This document captures various items discussed in the WG11 adhoc group on Study of HEVC Extension (N12195) and also provides the draft of the requirements and the use cases associated with the scalable enhancement of HEVC

# Recommended Requirements and Test Points for Scalable Coding Extensions

Three main use cases - 2D TV distribution, Video Conferencing and 3D video (including 3DTV, 3D Packaged Media, 3D Video Conferencing and 3D Video Games) – were discussed. They are briefly described in Annex A. Following are the requirements and the issues identified so far with regard to the scalability extension of HEVC for those use cases.

# Spatial Scalability

# The spatial scalability extension shall support 2 or more layers, each with different spatial resolution.

Under Consideration: Combining Spatial Scalability with Coding Standard Scalability (see Annex A for use case and Annex D for further details).

- Comment: For the video captured in progressively scanned formats and same aspect ratios (see section 1.4 on Aspect Ratio) at the two layers, factors of 1.5 and 2 (per layer) increase in resolution between the base and enhancement layers in each direction are the two key operating points. Coding efficiency with two Spatial Scalable layers with these resolution scaling factors should be used for testing, comparisons and standard development. (Note for CfP Phase 1: For CfP 1.5x and 2.0x for progressively scanned content will be test points)

- Other (including non-symmetrical) scale factors besides 1.5x and 2.0x should also be supported (Note for CfP Phase 1: non 1.5x and 2.0 x will not be tested in CfP Phase 1).

- Support of Interlaced scanned video (TBD – to be revisited after that decision is made for HEVC base layer)

- Switching points should be supported, for efficient receiver-based switching of number of spatial layers decoded at intermediate points (pictures) in a rate constrained network without adding latency. (Profile Specific). (Note for CfP Phase 1: include this testing in phase 1 – TBD)

# SNR scalability

# The scalability extension shall support layers with same spatial and temporal resolutions but different bit rates.

# (Note for CfP Phase 1: Key operating region is 25% of total bit rate  (per layer) bit rate reduction. Coding efficiency with two scalable layers and 25% bit rate reduction to be used for testing, comparisons and standard development. )

# Bit rate variation can be achieved via adjusting Quantization parameter, Quantization matrix and/or truncation of frequency coefficients.

# Temporal scalability

The standard shall support layers with different frame rates in increasing or decreasing factors of multiple of 2.

- (Note for CfP Phase 1: ) Key operating point is a factor of 2 in the frame rates at the two layers (frame rate at the Enhancement layer is twice the frame rate at the Base layer). It should be used for testing, comparisons and standard development.

# Aspect Ratio: (See Annex A for some of the definitions)

# The standard should be able to support different picture aspect ratios (PARs) across the layers. (Note for CfP Phase 1: this should be tested in CfP Phase 1.)

  - The standard should be able to support different picture sample aspect ratios (PSARs) across the layers. (Note for CfP Phase 1: Keep PSARs to be the same in the base and enhancement layers in CfP)

  - Comment: For example, base layer could be VGA (PAR of 4:3) and the enhancement layer could be 720p (PAR of 5.33:3 aka 16:9). A subset includes where the vertical resolutions are the same at two layers. For example, the base layer is VGA and the enhancement layer is 16:9 version of WVGA with 480 lines.

  - Comment: there may be two different categories of applications – one that need to support different PSARs across the layers and others who do not. If the complexity of supporting different PSARs across the layers comes out to be high, then different Profiles may be considered.

  - The need for standard to support pan-scan such that the lower layer can be placed at different locations relative to enhancement layers is not yet identified.  A simpler, Active Format Description type, approach is preferred. Under Considerations (to be further discussed in the Geneva meeting): The standard should be able to support spatially varying (non-linear) PSAR (e.g. the image may be stretched more on the edges than in the center). The information about how linear to non-linear mapping is done and non-linearly sub-sampled video may be generated outside the encoder and provided as inputs to the encoder. Open Issues: Lack of industry standard for the generation of non-linear mapping process and the data format. How to interface with the post-production system that generates the non-linear PSAR map?

# Robustness to transmission errors and congestion (Error Resilience)

The scalability extension should enhance the transmission over error-prone or congested channels compared to the transmission of single-layer HEVC.

  - Focus on packet losses with varying statistics across various networks.

  - Include mechanisms that can take advantage of advanced system layer features such as:

    - Unequal protection of parts of the scalable bitstream

      - Unequal protection of layers

      - Unequal protection of especially important NAL units in a layer (parameter sets, intra slices, …)

  - Allow the following options:

    - Graceful degradation

    - Sending additional information layer to help in concealing losses

    - Low delay source coding based error resiliency, using scalable layers (note: needs further explanation)

    - Low delay error resiliency

    - Resiliency against burst losses

    - Allow the option of two layers to be independently decodable.

# Combined scalability and complexity

# The scalability extension shall support the combination of multiple scalabilities within one bitstream. Key combinations identified so far consist of various combinations of Spatial, Temporal, Aspect Ratio and SNR scalabilities among two or three layers of scalability.

# Allow flexibility to have coding tools with different complexity across various layers. For example,

# base layer can be compliant with one or more of possible Low Complexity Profiles and the enhancement layer(s) can be compressed using coding tools associated with one or more High Efficiency Profiles.

# Or, base layer can be compliant with one or more possible High Efficiency Profiles and the enhancement layer can be compressed using low complexity coding tools.

Note for CfP Phase 1: What combinations to require for CfP? TBD.

# End-to-end delay and latency

The scalability extension shall enable at least the same end-to-end delay and latency as single-layer HEVC.

# Random access

The scalability extension shall allow for at least the same random access properties as single-layer HEVC.

Note: How many maximum numbers of layers are allowed – TBD. That may be part of Profile and Level discussion and may need to be postponed until things progress to that stage

# Coding Efficiency/Gain: (To be further discussed)

# Embedded bitstreams, providing capability to decode different resolutions, shall have significantly less bitrate than simulcast at same perceived quality of same resolutions, and shall not have significantly higher bitrate than a single-layer stream of the highest resolution, while pertaining comparable quality of the lower resolutions (to be further discussed, what significant means)

  - Comment: Based on the past experience with MVC, AVC and others with regard to the acceptance of new and more complex compression technology, the Coding Gain in comparison to Simulcast, as defined in Annex B, should be targeted to be high.

  - Open Issues: How is Coding Efficiency Defined: Equation B3 or B5? What should be the requirement of the minimum coding efficiency? (Note for CfP Phase 1: Minimum required Coding Efficiency number(s) will be included in CfP)

    - Should the Coding Gain as defined in equation (B3) be more than 2 and as defined in equation (B5) should be more than 1.35? The Base layer cost, as defined in equation (B6), for similar visual quality, should be within 10%? With the base layer cost of less than 10%, should we approximate it to 0 and use the coding gain defined in equation (B3); otherwise, will it be hard to convey the benefit/gain of scalability?

    - Another approach can be that equation (B5) is used only during the standard development process and comparing different spatial scalability algorithms. Once the standard is developed, we use the base layer of the final scalable standard as also the base layer for Simulcast and then use Case 1 of Annex B and equation (B3) to measure and convey the CG of the standard.

# Complexity: (To be further discussed)

# Scalable coding shall enable low complexity implementations for encoding as well as decoding. ( Comment: See Annex C for Complexity Discussion)

# (Note for CfP Phase 1: Proponents will be requested to provide some complexity measures? – TBD. Should recommended complexity measures be included in CfP Phase 1 – TBD?)

  - Both software (CPU utilization) and hardware implementation complexity (in particular, DRAM bandwidth requirement) should be measured and taken into account.

  - Open Issues:

    - In comparison to single layer higher resolution decoding, for spatial scalability with 2x scale factor, should the total complexity should be required to be below around 125%, and for a 1.5x scale factor be below around 150%?

    - In comparison to transcoding based systems, higher (2 to 3x of single higher layer decoding?) complexity can also be acceptable in some systems if it provides better coding gain.

- In comparison to Simulcast: TBD (110 to 125%?).

# System Level Considerations

Scalable coding technology shall specify efficient and easy ways to carry the bitstreams over widely used protocols and networks.

- Comment: It is important to keep in mind the System level considerations. The compressed video will get transported using MPEG-2 Systems, MPEG DASH and various Internet Protocols. It should be kept in mind that video may be multiplexed, re-multiplexed, statistically multiplexed, transrated and transcoded in the chain from the source to destination. Efficiency and ease of carriage over various networks should be a part of the design.

- Comment: The compressed video will also be transported using IP networks.  Impact of network switching elements should also be considered. (Detail to be added).

- (Note for CfP Phase 1: This will be taken into consideration in evaluating Cfp? TBD)

Annex A

Some Example Use Cases

A.1 (2D) Digital Video Distribution

A.1.1 Brief descriptions of various use cases

The key driving force from the requirement point of view is that the environment of next generation of digital TV distribution is expected to be quite heterogeneous on both the client as well as the network side. On the client side, the multiple (three) screen scenario is expected to dominate. The three main screens are: Smart Phone, Tablet and TV. The PC/Laptop screen is not ignored here. That is also covered by the three-screen scenario.

Even though, unlike digital TV screen, there is no standard with regard to the number of pixels in the glass in mobile displays (it varies a lot among various smart phones and tablets), the hot spots of the video resolutions associated with the 60Hz systems are coming out to be : HVGA (480x320)p30, VGA (640x480)p30, 720 (1280x720)p30, 720 (1280x720)p60, and potentially 1080 (1920x1080)p60 in future. There are also 1080i30 (aka 1080i60, here in this document the numbers after letters ‘p’ or ‘i’ are frames per sec. In case of interlaced format 30 frames second consists of 60 fields per sec) and 480i30 (aka 480i60) formats.

(Note: there are multiple definitions of HVGA, but lately 480x320 seems to be more popular definition. Although the ratio of 480 to 320 is not exactly 4:3, but in many cases it can be assumed that the video maintains 4:3 aspect ratio inside that format).

The numbers 30 and 60 fps get replaced by 25 and 50 fps respectively in the 50 Hz systems.

Another digital TV application includes extending the resolution beyond current HD resolutions in backwards compatible way. For example, in some use case scenarios, it is desirable to extend resolution from 1920x1080 to 3840x2160 such that the base layer has 1920x1080 resolution and enhancement layer is 3840x2160.

Looking at those video resolutions, the spatial resolutions jump in the steps of factors of 1.5 or 2. Therefore, one of the requirements for the scalability is to (efficiently) support the spatial scalability with a factor of 1.5 or 2.0 in each axis. Supporting both VGA and 720p will require the discussion of aspect ratio.

Supporting both 720p30 and 1080p60 will require not only the spatial scalability but also temporal scalability at the same time.

The requirements for SNR scalability is mainly driven by the network. As an example, in the home environment, a video can be sent to a set top box / DVR for viewing on a TV and/or also to a Tablet where the network between a Home Gateway/DVR/Access Point and Tablet may be a Wi-Fi network. The network capacity can change from home to home as well as it will vary with the distance between the gateway (the term "gateway" is used in very broad and generic sense here) and the Tablet. An example of bit rate ranges used with AVC codec (note: all the example bit rates in this paragraph correspond to AVC) are 1 to 2 Mbps for VGA, 2 to 4 Mpbs for 720p30 and 4 to 8 Mbps for 720p60. If the bit rate needs to be changed by a factor of 2, the spatial/temporal resolution will probably also need to be changed. However, if the bit rate range is less than factor of 2 then spatial and temporal resolutions can be kept the same and only the bit rate can be changed. This can either be achieved via transrating/transcoding and/or via inserting a layer of SNR scalability. Picking the highest and the center numbers in the range, the SNR scalability that allows one to reduce the bit rate by about 25% is useful (for example higher quality layer at 4 Mbps 720p30 and lower quality at 3 Mbps 720Mbps). To go to the next lower (by 50%) rate, spatial and/or temporal resolution is reduced.

For other scenarios, for example streaming video over the internet, Over The Top (OTT) digital TV distribution etc., where the network either can not guarantee quality of service or varies with time where the guarantee of the network bandwidth is time dependent, the requirements on scalability are similar as above.

A.1.2  Interlace Scanned Video

Majority (if not all) of the displays have become progressive only. So, there does not appear to be a need to support 1080i30 displays going forward. However, there still exists content and new content is created that is captured in interlaced scanning format.

So, the main question here is, if the content is captured in interlaced format what to do with it. An example problem can now be described as follows – support the base layer that is progressive (e.g. HVGAp30 or 720p30) and the enhancement layer that is interlaced (e.g. 480i30 or 1080i30)

Although HEVC has so far not formally looked into addressing how to handle interlaced scanned video, it also appears that the intention of the group is to not add any interlace scan specific coding tools. So, for now, there appear to be following options:

(1)  Let the interlaced video in the enhancement layer be sent as fields with some appropriate signaling to inform the receiver. The spatial resolution of each field can also be modified to match more efficiently with base layer’s resolution.

(2) Let the enhancement video be sent as frames. Frames can be created in various possible ways:

  - Both the fields are merged together to create 1080p30 version of 1080i30 with proper signaling to the receiver informing it that was done

  - Encoder uses advanced de-interlacing to convert 1080i30 to 1080p60

  - Encoder uses advanced de-interlacing to convert 1080i30 to 60 or 30 frames per second and also modify the spatial resolution to match more efficiently with base layer’s resolution.

A.1.3  Aspect Ratio

Definitions:

picture sample aspect ratio (PSAR): the ratio between the horizontal distance between the columns and the vertical distance between the rows of the luma sample array in a picture. Picture sample aspect ratio is expressed as h:v, where h is horizontal width and v is vertical height (in arbitrary units of spatial distance).

picture aspect ratio (PAR): The ratio width:height of the captured picture where width and height are measured in the same length (spatial measurement) units. Picture aspect ratio is expressed as X:Y, where X is horizontal width and Y is vertical height (in arbitrary units of spatial distance).

Example use cases:

- The base layer is VGA and the enhancement layer is 720p. In this case PARs at the two layers are different but PSARs are the same (square pixels).

- The base layer is VGA and the enhancement layer is 16:9 WVGA. In this case PARs at the two layers are different but PSARs are the same (square pixels). But, here the vertical resolutions at both the layers are the same.

A.1.4  Error Resilience

Next generation of digital TV distribution will include IP networks with UDP/IP based protocols using MPEG-2 TS or RTP based encapsulation. Packet loss statistics will be different across different networks.

Both unidirectional (unicast as well as multicast) and bi-directional systems are expected to be in use. In bi-directional systems there can be a feedback of information, associated with packet loss, from the receiver/network that can impact the behavior of the transmission side. 
In one to one bi-directional systems that can tolerate longer delays, TCP (or other) re-transmission schemes can be used. However, to achieve lower delays, providing resilience at the video layer can be more attractive.

In multicast, a system can not be optimized or designed for an individual receiver but a more general optimization can be done for general distribution system. For example, a different resilience for multicast over wireless channels (similar to mobile version of various web sites) than wired, e.g., one could be optimized for nG (n = 3,4, ...) or Wi-Fi.

A.1.5  Bit Depth

Next generation of consumer displays, specifically TVs, are expected to be able to support more than 8 bits of resolution. This will create diversity of displays along the bit depth dimension.

A.1.6  Chroma Resolution

Next generation of consumer displays, specifically TVs, are expected to be able to support 4:4:4 Chroma resolution. This will create diversity of displays along the chroma resolution dimension.

A 1.7 Scalability Combinations:

A.1.7.1 Spatial + SNR Scalability

A combination of SNR scalability and other scalability may also be needed. For example, layers corresponding to 720p30 at 3Mbps, 720p30 at 4 Mbps and 720p60 at 6 Mbps, or, in another scenario, layers corresponding to 720p60 at 6 Mpbs, 1080p60 at 9 Mbps and 1080p60 12 Mbps may be provided by a service provider. Similarly, one can think of scenarios where all three (spatial, temporal and SNR) scalability tools are needed and provided. The quality of video in those combination cases need to be acceptable and the coding efficiency also needs to be acceptable. If it comes out that the efficiency need can not be satisfied then what and how many combinations are allowed may be limited.

A 1.8 Color space

Changes in resolution in digital video distribution often result in a change in color space.  For example, in a digital TV application that includes the support of standard definition and high definition content, the standard definition content is represented with color primaries specified in ITU-T Rec. BT601.  Similarly, the high definition content may be represented with color primaries specified from ITU-T Rec. BT709.  In this application, it is desirable to support different color spaces in the standard definition and high definition layers.

Another relevant digital TV application is that of extending the resolution beyond current HD resolutions in a backwards compatible way (described above). For example, in some use case scenarios, it is desirable to extend resolution from 1920x1080 to 3840x2160 such that the base layer has 1920x1080 resolution and enhancement layer is 3840x2160.  In these applications, the HD resolution version is represented with ITU-T Rec. BT709 color primaries for backwards compatibility.  However, it is desirable for the enhancement layer to be represented with color primaries that provide a larger color gamut.  For example, one set of enhancement primaries are:

It is noted that currently available consumer displays typically support a larger gamut.  Similarly, devices in-market can acquire content with this larger gamut.

Requirements for the applications above are: (i) support of different color spaces in base and enhancement layers and (ii) support of tristimulus values in the enhancement layer color space that can not be represented in the base layer color space.

A.2 Video Conferencing

Video conferencing systems are also rapidly moving towards multi-screen environment where the display screen could as small as HVGA to as large as HDTV (and potentially 4kx2k in future). Video conferencing systems are very delay sensitive.  Video conferencing frequently takes place over networks with dynamically changing conditions, and needs tools to allow fast adaptation to changing conditions which do not require transmission of I frames.

In multipoint video conferencing, the display resolution of an individual video sequence may change frequently, for example when the person speaking is displayed at a larger resolution than the additional participants in the video conference.

In addition, Video Conferencing systems have two way communication that can be used to characterize the channel.

Otherwise, the requirements of this use case are similar to the 2D Digital Video Distribution briefly described in A.1.

A.3 3D Video

A.3.1 Overview of 3D Video Use Case

A stereoscopic effect can be obtained from a flat two-dimensional display by employing some form of filtering to ensure that information representing a different perspective is presented to each eye.  Such plano-stereoscopic “3D” video is becoming increasingly used in various applications, such as 3DTV, 3D Packaged Media, 3D Video Conferencing and 3D Video Games. The filtering process may either rely on glasses (spatial filtering by polarized glasses or temporal filtering by shutter glasses) or else it may be inherent in the display itself (an auto-stereoscopic display).

In the 3D video use case, an additional layer associated with View Scalability comes into play. Although only Stereo-views based 3D Video systems are currently deployed, interest in Multi (more than 2) views is increasing. In both Stereo and Multi View cases, there can be two categories of approach:

- 2 or more views are captured and all views in addition to the Base view are compressed by exploiting redundancy across the views (e.g. MVC extension of AVC),

- 2 or more views are captured, depth maps and other related information are extracted (in some cases that may be a part of the video capturing process), and the bit stream corresponding to the depth maps and other information is sent as a layer in addition to the base view. 2D to 3D conversion techniques are also becoming increasingly popular. In that case, the depth maps and other related information are generated as a part of 2D to 3D conversion process.

A.3.2 3DTV

Several major pay TV broadcasters have launched dedicated 3DTV channels, whilst free-to-air broadcasters have so far tended to restrict their 3D broadcasting to the occasional special event.  Real-time camera-captured content (e.g. sports events) is a commercially important genre for 3D broadcasting. For most of the current 3D broadcasters, a critical factor in the business plan is the ability to use an already deployed HDTV transmission infrastructure to transmit the 3D signal and an already deployed population of HDTV set-top boxes to decode the 3D signal (with appropriate software updates).  For this reason, the vast majority of current 3DTV systems are encoded using a frame-compatible, half-resolution approach within AVC (or in some cases MPEG-2) video.

The ease of migration offered by the frame-compatible approach will be an important factor for some of future 3D broadcasting systems based on HEVC.  In other systems, future HEVC-coded TV channels may be planned to be launched as 3D-capable, in which case the consideration of issues such as compression efficiency and 2D compatibility may favour an MVC-like approach.   At present, it is more difficult to use the alternative 2D plus depth approach when encoding real-time content such as sports.

A.3.3 3D Packaged Media

Inspired by the commercial success of Avatar, an increasing number of movies are released theatrically in 3D, including those which have a significant amount of computer generated imagery.  The Blu-ray 3D specification was developed to allow packaged media to give a 3D movie experience in the home, when a 3D-capable Blu-ray player is connected via HDMI to a 3D-capable TV.  An important commercial consideration was backwards compatibility to allow the 3D-capable disc to be played, as 2D, on a legacy 2D player.  This was achieved using the MVC extension of AVC.

Future HEVC-coded packaged media are likely to continue to have the requirement to be backwards compatible with 2D players and hence they may continue to use an MVC-like approach to coding.  Alternatively, a 2D plus depth approach could offer 2D compatibility whilst also providing better coding efficiency and display-related or user-controlled variation of viewing preferences, since the content is intrinsically non real-time.

A.3.4 3D Video Conferencing

By its nature, 3D Videoconferencing involves real-time encoding of camera-captured content with minimal time delay.  This would probably suit an MVC-like approach, although the restricted nature of a typical video-conferencing scene may also facilitate the capture of a real-time depth map.

A.3.5 Video Games with 3D Video

Video Games with capability to support 3D video are becoming available using all three of the main categories of gaming hardware:

- PC  typically using appropriate graphics card connected via DVI or HDMI to a high frame-rate PC monitor (with shutter glasses)

- Games console - typically connected via HDMI to a 3D-capable TV (with shutter glasses or polarized glasses)

- Hand-held including Tablets and Smart Phones - typically using integral auto-stereoscopic display (including depth control)

If the video content is generated by the gaming hardware from an internal abstract model, the most appropriate format for the connected display should be able to be created with no major compatibility constraints.  In this case, the extra degree of user control offered by a 2D plus depth type of approach may be attractive.

A. 4 Cloud based (on line) video games

The video gaming market place is expanding to include on-line games. The ever increasing presence of broadband connection is paving the way for acceptance of increasingly sophisticated cloud based (on line) video games. The on line video gaming business is expected to be a fast growing sub-set of video games over next few years. In this model the gaming hardware is located in the network/cloud. The server streams the content using audio/video compression to a thin client in the home or to a mobile device. The server and clients communicate via IP network. The latency of communication is expected to be low – less than about 100 msec. As the communication technology improves with time, this delay is expected to reduce. The gaming hardware further adds the latency that is in the similar range. Similar to the entertainment TV distribution environment, this environment is also expected to be heterogeneous on both the client as well as the network side. Some of those games are also expected to provide 3D video. Requirements for this use case include: video coding with low latency and high efficiency (especially for computer generated content), to be able to support clients with resolutions ranging from VGA to HD, to be able to support distribution over wired and wireless networks. The bit depth of 8 bits is quite satisfactory. In near term, the View Scalability will be required to provide the 3D video support. As video is also expected to be distributed via the networks that may not have guaranteed quality of service, error resilience of the encoding scheme will also be desirable in those cases. In longer term, as the on line video gaming environment grows to include multi-users, other scalability tools may also be needed to satisfy the needs of diverse set of clients and networks.

A.5 Coding Standard Scalability:

It is expected that over the next few years AVC based encoding and decoding devices will be broadly deployed and AVC will become a dominant video coding standard used worldwide for the distribution of digital video. It will help significantly in deploying next scalable standard if a migration path can be provided such that the AVC based deployed system can still decode and process some of the layers. Some of the examples are: (1) in stereo-view scalability for 3DTV, the left eye view pictures can be compressed using AVC and the right eye view pictures can be compressed using HEVC tools as well as using the left eye view pictures as references. This way existing AVC based 2DTV system can still play back the 2D video. (2) Similarly, to provide a migration path to 1920x1080x60p, if good coding efficiency can be achieved, the base layer could be of 1280x720x60p resolution with pictures compressed using AVC standard and the enhancement layer could extend the resolution to 1920x1080x60p by using HEVC compression tools and 1280x720x60p base layer as reference, (3) the 1280x720x60p resolution in the previous example could also consist of two temporal layers with base layer at 30 frames per sec frame rate providing interoperability among existing AVC based tablets, AVC based TVs/STBs and new HEVC systems.

Annex B

Coding Efficiency

There are three competing approaches to Scalability:

- Simulcast

- Transcoding from Single layer

- Interpolation from Single layer

The coding efficiency and complexity of the scalable systems need to be compared to the above cases as well as to single layer systems with no scalability. This provides the user an understanding of what the cost and bit rate overhead is associated with various scalability options, and how they compare against other available options that can provide similar functionalities. For now, the discussion is focused only on two layers of scalability to keep it simple.

B.1 Simulcast

In these systems two layers are sent simultaneously. Let the base layer be sent using bit rate R and the enhancement layer be sent at α*R rate. The total required bit rate is (1 + α)*R bps. The coding efficiency of the scalable system is measured at the enhanced layer subject to the constraint that the visual quality at the base layer be similar to that of the Simulcast system. (Comment: The filters used to create the base layers for Simulcast should be such that sufficiently high frequencies, in comparison to the sampling rate, are significantly preserved. Filter response and cut off frequencies TBD). Let the bit rate used by the Scalable systems at the base layer be β*R and at the enhancement layer be γ*R. So, the total bit rate used by the scalable system is (β + γ)*R.

The coding efficiency is measured by obtaining the reduction in the total bit rate for the same quality at the enhancement layer. It can be measured by keeping the PSNR at the same level for both cases and comparing the bit rate (BD rate reduction measurement method can also be used). The bit rate reduction BR is given by

BR = (1 + α - β - γ)*R.						(B1)

There are two different ways to define the coding efficiency.

Case 1

In this case the base layers for the simulcast and the scalable systems can be same (e.g. in MVC), β = 1 and the percentage BR can be defined (as is done in MVC) by

(BR/ (α*R)) *100% = 100*(α - γ)/ α % .					(B2)

Coding gain (CG) can be defined by

CG = 1/(1- BR/( α*R)) = α/ γ .					(B3)

Case 2

In this case the base layers for the simulcast and the scalable systems may not be the same (e.g. various spatial scalability approaches may want to use different filtering to create the base layers). The value of (1 - β) shows the saving or cost of scalability at the lower layer to achieve the similar quality. Similar quality in this case should be established by visual inspection. The bit rate reduction, as a percentage, now needs to take into account the base layer bit rate as well and can be defined as

BR /((1 + α)*R)*100%  = ((1 + α - β - γ) /(1 + α)*100%

= (1 – (β + γ)/ (1 + α)) *100% .		(B4)

Following equation (B3), the coding gain can now be defined as

CG	=  1/(1- BR/ ((1+α)*R))

= (1 + α)/(β + γ) .					(B5)

There may be some interest in having the same definition of CG for the two cases above. However, the CG is already defined in the past (e.g. MVC) for Case 1 and making changes in that definition may create more confusion.

The percentage Base Layer cost (BC) can be defined as

BC = (β – 1)*100% .						(B6)

B.2 Transcoding

In these systems, one single higher quality / resolution layer is sent and the content is transcoded to achieve base layer. As the scale factor increases, the efficiency of the transcoding method increases. However, the transcoding tends to be significantly more compute intensive than simulcast or scalable decoding. As transcoding based systems use only one layer, the coding efficiency of such system is expected to be better than that of scalable systems. A system designer needs to take into account the bandwidth requirement versus the complexity into account while designing a particular system. It will be helpful to understand how much increase in bit rate is required to achieve similar visual quality as obtained via transcoding, and how much reduction in the complexity is achieved by scalability tools.

B.3 Single layer with Interpolation

In these systems, one single lower resolution layer is sent and the content is up-converted to higher resolution. We are not considering comparison of scalability systems to these approaches as it applies in limited cases, and the performance cannot improve over a certain limit irrespective of the bit rates used and at present there are no standard ways to up-convert the video.

Annex C

Complexity

High level measure of complexity based on CPU Utilization and Memory Bandwidth will be defined and inserted here.

ANNEX D

Requirements of the HEVC Extensions Under Consideration

# D.1 View Scalability for 3D Video

The scalability extension shall support View Scalability in order to enable stereoscopic and multi-view display.  There shall be mechanisms of supporting View Scalability that are appropriate for both real-time encoding of live content as well as non real-time encoding with a mechanism which enables user control of viewing preferences.These mechanisms may or may not be the same.

# D.1.1 Pixel based View Scalability

There shall be a mechanism of supporting View Scalability that is capable of:

- Exploiting inter-view redundancy of two views suitable for direct plano-stereoscopic display in order to substantially reduce the total bit-rate required, relative to the bit-rate required for independent HEVC coding of the two views.

- Exploiting inter-view redundancy of multiple views in order to substantially reduce the total bit-rate required, relative to the bit-rate required for independent HEVC coding of each view.

- Enabling a legacy 2D HEVC decoder to extract and decode a single view from the encoded stereoscopic or multi-view bitstream.

# D.1.2 Depth map and other data based View Scalability

There shall be a mechanism of supporting View Scalability based on Depth map and other data.

It shall be capable of:

- Enabling display-related or user control of viewing preferences, such as the degree of depth perception.

- Exploiting inter-view redundancy of two views suitable for direct plano-stereoscopic display in order to very substantially reduce the total bit-rate of the data required to enable the generation of the two views (e.g. using 2D video and a depth map).

- Exploiting inter-view redundancy of multiple views in order to very substantially reduce the total bit-rate required to enable the generation of many high-quality views in a manner suitable for multi-view display (e.g. on an autostereoscopic display).

- Enabling a legacy 2D HEVC decoder to extract and decode a single view from the encoded stereoscopic or multi-view bitstream.

# D 1.3 Backward compatibility with Half Resolution (Frame Compatible) 3D system

- MPEG is studying if Full Resolution per eye 3D video needs to be compressed in a scalable fashion with Frame Compatible (Half Resolution per eye) 3D video as base layer. If that mode of scalability is needed, HEVC will be extended to support that mode of scalability.

D 1.4 Combinations of View Scalability with other Scalabilities

D 1.4.1 Combined Spatial Scalability and View Scalability

The scalability extension shall allow combination of spatial scalability with view scalability where

- the base view includes video layers with different resolutions but the enhancement layers, corresponding to other views, are of single resolution that is same as the highest resolution of the base view.

- the base view as well as enhancement layers include video layers with different resolutions. The spatial and temporal resolutions in the base view and enhancement layers are the same

D 1.4.2 Combined Coding Standard Scalability and View Scalability

The scalability extension should include a mode where it is able to operate with a base layer that is compliant with other MPEG standards (e.g. AVC).  The mechanism used to achieve compatibility with other MPEG standards shall not cause unacceptable increase in complexity and reduction in the performance of the HEVC extension when it is operating in a mode (Profile) with an HEVC-compliant base layer. The enhancement layer, corresponding to other views, may consist of pixels corresponding to those views (MVC style scalability) or HEVC based representation of 2D Depth and other data.

D.2 Combined Coding Standard Scalability and Spatial Scalability

The scalability extension should include one or more modes (Profiles) where it is able to operate with a base layer that is compliant with AVC Standard’s Constrained Baseline Profile, Main Profile and High Profile.  The Base layer, for now, is constrained to be progressively scanned video. The enhancement layer is also restricted to be progressively scanned video until single layer HEVC work concludes how to address interlaced scanned video (revisit this after the single layer HEVC work concludes how to address interlaced scanned video). The base layer resolution specifications are the same as that for HEVC base layer. The mechanism used to achieve compatibility with AVC standard shall not cause unacceptable increase in complexity and reduction in the performance of the HEVC extension when it is operating in a mode (Profile) with an HEVC-compliant base layer.

# D.3 Bit Depth (Revisit after HEVC develops single layer solution to support higher than 8 bits depth)

  - Provide support for bit-depth scalability with a change of at least 2 bits between layers.  For example, an 8-bit base layer and a 10-bit enhancement layer.

  - Open Issue: Where is the crossover point/region (in terms of quality vs bit rate) for HEVC between not using pre-processing for down conversion to 8 bits and directly using 10 (or higher) bits vs pre-processing to 8 bits, is not known at this stage.

  - Open Issue: Will the entertainment TV distribution channels support the bit rates required for a viewer to see the benefits of increased bit depth?

# D.4 Chroma Resolution (Revisit after HEVC develops single layer solution to support 4:4:4 resolution)

  - Provide support for scalability from 4:2:0 to 4:4:4 chroma resolution. The 4:4:4 chroma resolution extension may also be accompanied with the bit depth scalability.

  - Requirements to support wider color gamut are not clear at this stage.

  - Open Issue – Where is the crossover point/region (in terms of quality vs bit rate) for HEVC between not using pre-processing for down conversion to 4:2:0 and directly using 4:4:4 vs pre-processing to 4:2:0, is not known at this stage.

  - Open Issue – Will the entertainment TV distribution channels support the bit rates required for a viewer to see the benefits of increased chroma resolution?

D.5 Spatially Varying (non-linear) PSAR

  - The standard should be able to support spatially varying (non-linear) PSAR (e.g. the image may be stretched more on the edges than in the center). The information about how linear to non-linear mapping is done and non-linearly sub-sampled video may be generated outside the encoder and provided as inputs to the encoder. Open Issues: Lack of industry standard for the generation of non-linear mapping process and the data format. How to interface with the post-production system that generates the non-linear PSAR map?

D.6 Support (Signaling) of Interlaced Video

  - To be revisited after decision related to the support (signaling) of Interlaced video is made for the single layer case.

D.7 Shoot and Protect Area (revisit after HEVC single layer support is defined)

- The standard should be able to signal a “shoot and protect” or “safe” area within the video image, which contains the critical video content which is intended to be retained after a cropping operation (e.g. for aspect ratio conversion).   The shoot and protect aspect ratio will generally be different from the picture aspect ratio (e.g. there may be a 14:9 shoot and protect area within a 16:9 picture).

# D.8 Color space

-	The scalability extension shall support a base and enhancement layer with different color spaces

-	The scalability extension shall support a combination of color space scalability and spatial scalability.

-	The scalability extension shall support tristimulus values in the enhancement layer that can not be represented in the base layer color space.

  - Open Issue: Should we support different color space across the layers in Phase 1. If yes then should we include this in CfP for Phase 1?