INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 M22557

November 2011, Geneva, Switzerland

| Source: | WG11 AHG on Study of HEVC Extensions |
| --- | --- |
| Purpose: | Information and Discussion |
| Title: | Draft requirements for the scalable enhancement of HEVC |
| Chair: | Ajay Luthra |
| Co-Chairs: | Jens-Rainer Ohm, Jörn Ostermann |

Abstract

This document provides the draft of the requirements associated with the scalable enhancement of HEVC as discussed by the WG11 adhoc group on Study of HEVC Extension (N12195).

# Introduction

# Requirements

# Spatial Scalability

# The spatial scalability extension shall support 2 or more layers, each with different spatial resolution.

For the video captured in progressively scanned formats and same aspect ratios  at the two layers, factors of 1.5 and 2 (per layer) increase in resolution between the base and enhancement layers in each direction are the two key operating points.

Other (including non-symmetrical) scale factors besides 1.5x and 2.0x should also be supported.

Key spatial resolutions of interest are 4kx2k, 1920x1080, 1280x720, 854x480 (a.k.a. WVGA), 640x480 (a.k.a. VGA)

# Aspect Ratio:

Definitions:

picture sample aspect ratio (PSAR): the ratio between the horizontal distance between the columns and the vertical distance between the rows of the luma sample array in a picture. Picture sample aspect ratio is expressed as h:v, where h is horizontal width and v is vertical height (in arbitrary units of spatial distance).

picture aspect ratio (PAR): The ratio width:height of the captured picture where width and height are measured in the same length (spatial measurement) units. Picture aspect ratio is expressed as X:Y, where X is horizontal width and Y is vertical height (in arbitrary units of spatial distance).

Requirements:

# The standard shall support different picture aspect ratios (PARs) across the layers. Key two aspect ratios of interest are 16:9 and 4:3.

# The standard should be able to support different picture sample aspect ratios (PSARs) across the layers. Note: In Phase 1 PSARs will be the same (1:1) in the base and enhancement layers in CfP. For example, the base layer could be VGA and the enhancement layer could be 1280x720. Or, the enhancement layer could be WVGA and the base layer could be VGA.

Example use cases:

- The base layer is VGA and the enhancement layer is 720p. In this case PARs at the two layers are different but PSARs are the same (square pixels).

- The base layer is VGA and the enhancement layer is 16:9 WVGA. In this case PARs at the two layers are different but PSARs are the same (square pixels). But, here the vertical resolutions at both the layers are the same.

Under Consideration: Combining Spatial Scalability with Coding Standard Scalability (see Annex A for further details).

# SNR scalability

# The scalability extension shall support 2 or more layers with same spatial and temporal resolutions but different bit rates.

# Key operating region is 25% of total bit rate (per layer) bit rate reduction.

# Bit rate variation can be achieved via adjusting Quantization parameter, Quantization matrix and/or truncation of frequency coefficients.

# Temporal scalability

The standard shall support 2 or more layers with different frame rates in increasing or decreasing factors of multiple of 2.

# Combined Scalability

# The standard shall support a mechanism that enables combined SNR, Spatial and Temporal scalability.

# The standard shall allow flexibility to have coding tools with different complexity across various layers. For example,

# base layer can be compliant with one or more of possible Low Complexity Profiles and the enhancement layer(s) can be compressed using coding tools associated with one or more High Efficiency Profiles.

# Or, base layer can be compliant with one or more possible High Efficiency Profiles and the enhancement layer can be compressed using low complexity coding tools.

Under Consideration: Combining Spatial Scalability with Coding Standard Scalability (see Annex A for further details).

# Robustness to transmission errors and congestion (Error Resilience)

The scalability extension should enhance the transmission over error-prone or congested channels compared to the transmission of single-layer HEVC. Primary focus is on packet losses with varying statistics across various networks.

The standard shall allow the mechanisms that can take advantage of advanced system layer features to allow unequal protection of parts of the scalable bitstream:

    - Unequal protection of layers

    - Unequal protection of especially important NAL units in a layer (parameter sets, intra slices, …)

The standard shall allow the mechanisms for the following options:

    - Graceful degradation

    - Sending additional information layer to help in concealing losses

    - Low delay source coding based error resiliency, using scalable layers (note: needs further explanation)

    - Low delay error resiliency

    - Resiliency against burst losses

    - Allow the option of two layers to be independently decodable.

# End-to-end delay and latency

The scalability extension shall enable at least the same end-to-end delay and latency as single-layer HEVC.

# Random access

The scalability extension shall allow for at least the same random access properties as single-layer HEVC.

Note: How many maximum numbers of layers are allowed – TBD. That may be part of Profile and Level discussion and may need to be postponed until things progress to that stage

# System Level Considerations

Scalable coding technology shall specify efficient and easy ways to carry the bitstreams over widely used protocols and networks. The compressed video will get transported using MPEG-2 Systems, MPEG DASH and various Internet Protocols. The video may be multiplexed, re-multiplexed, statistically multiplexed, transrated and transcoded in the chain from the source to destination. Efficiency and ease of carriage over various networks should be a part of the design.

ANNEX A

Requirements of the HEVC Extensions Under Consideration

# A.1 View Scalability for 3D Video

The scalability extension shall support View Scalability in order to enable stereoscopic and multi-view display.    There shall be mechanisms of supporting View Scalability that are appropriate for both real-time encoding of live content as well as non real-time encoding with a mechanism which enables user control of viewing preferences.These mechanisms may or may not be the same.

# A.1.1 Pixel based View Scalability

There shall be a mechanism of supporting View Scalability that is capable of:

- Exploiting inter-view redundancy of two views suitable for direct plano-stereoscopic display in order to substantially reduce the total bit-rate required, relative to the bit-rate required for independent HEVC coding of the two views.

- Exploiting inter-view redundancy of multiple views in order to substantially reduce the total bit-rate required, relative to the bit-rate required for independent HEVC coding of each view.

- Enabling a legacy 2D HEVC decoder to extract and decode a single view from the encoded stereoscopic or multi-view bitstream.

# A.1.2 Depth map and other data based View Scalability

There shall be a mechanism of supporting View Scalability based on Depth map and other data.

It shall be capable of:

- Enabling display-related or user control of viewing preferences, such as the degree of depth perception.

- Exploiting inter-view redundancy of two views suitable for direct plano-stereoscopic display in order to very substantially reduce the total bit-rate of the data required to enable the generation of the two views (e.g. using 2D video and a depth map).

- Exploiting inter-view redundancy of multiple views in order to very substantially reduce the total bit-rate required to enable the generation of many high-quality views in a manner suitable for multi-view display (e.g. on an autostereoscopic display).

- Enabling a legacy 2D HEVC decoder to extract and decode a single view from the encoded stereoscopic or multi-view bitstream.

# A. 1.3 Backward compatibility with Half Resolution (Frame Compatible) 3D system

- MPEG is studying if Full Resolution per eye 3D video needs to be compressed in a scalable fashion with Frame Compatible (Half Resolution per eye) 3D video as base layer. If that mode of scalability is needed, HEVC will be extended to support that mode of scalability.

A.1.4 Combinations of View Scalability with other Scalabilities

A.1.4.1 Combined Spatial Scalability and View Scalability

The scalability extension shall allow combination of spatial scalability with view scalability where

- the base view includes video layers with different resolutions but the enhancement layers, corresponding to other views, are of single resolution that is same as the highest resolution of the base view.

- the base view as well as enhancement layers include video layers with different resolutions. The spatial and temporal resolutions in the base view and enhancement layers are the same

A.1.4.2 Combined Coding Standard Scalability and View Scalability

The scalability extension should include a mode where it is able to operate with a base layer that is compliant with other MPEG standards (e.g. AVC).  The mechanism used to achieve compatibility with other MPEG standards shall not cause unacceptable increase in complexity and reduction in the performance of the HEVC extension when it is operating in a mode (Profile) with an HEVC-compliant base layer. The enhancement layer, corresponding to other views, may consist of pixels corresponding to those views (MVC style scalability) or HEVC based representation of 2D Depth and other data.

A.2 Combined Coding Standard Scalability and Spatial Scalability

The scalability extension should include one or more modes (Profiles) where it is able to operate with a base layer that is compliant with AVC Standard’s Constrained Baseline Profile, Main Profile and High Profile.  The Base layer, for now, is constrained to be progressively scanned video. The enhancement layer is also restricted to be progressively scanned video until single layer HEVC work concludes how to address interlaced scanned video (revisit this after the single layer HEVC work concludes how to address interlaced scanned video). The base layer resolution specifications are the same as that for HEVC base layer. The mechanism used to achieve compatibility with AVC standard shall not cause unacceptable increase in complexity and reduction in the performance of the HEVC extension when it is operating in a mode (Profile) with an HEVC-compliant base layer.

# A.3 Bit Depth (Revisit after HEVC develops single layer solution to support higher than 8 bits depth)

  - Provide support for bit-depth scalability with a change of at least 2 bits between layers.  For example, an 8-bit base layer and a 10-bit enhancement layer.

  - Open Issue: Where is the crossover point/region (in terms of quality vs bit rate) for HEVC between not using pre-processing for down conversion to 8 bits and directly using 10 (or higher) bits vs pre-processing to 8 bits, is not known at this stage.

  - Open Issue: Will the entertainment TV distribution channels support the bit rates required for a viewer to see the benefits of increased bit depth?

# A.4 Chroma Resolution (Revisit after HEVC develops single layer solution to support 4:4:4 resolution)

  - Provide support for scalability from 4:2:0 to 4:4:4 chroma resolution. The 4:4:4 chroma resolution extension may also be accompanied with the bit depth scalability.

  - Requirements to support wider color gamut are not clear at this stage.

  - Open Issue – Where is the crossover point/region (in terms of quality vs bit rate) for HEVC between not using pre-processing for down conversion to 4:2:0 and directly using 4:4:4 vs pre-processing to 4:2:0, is not known at this stage.

  - Open Issue – Will the entertainment TV distribution channels support the bit rates required for a viewer to see the benefits of increased chroma resolution?

A.5 Spatially Varying (non-linear) PSAR

  - The standard should be able to support spatially varying (non-linear) PSAR (e.g. the image may be stretched more on the edges than in the center). The information about how linear to non-linear mapping is done and non-linearly sub-sampled video may be generated outside the encoder and provided as inputs to the encoder. Open Issues: Lack of industry standard for the generation of non-linear mapping process and the data format. How to interface with the post-production system that generates the non-linear PSAR map?

A.6 Support (Signaling) of Interlaced Video

-	To be revisited after decision related to the support (signaling) of Interlaced video is made for the single layer case.

A.7 Shoot and Protect Area (revisit after HEVC single layer support is defined)

- The standard should be able to signal a “shoot and protect” or “safe” area within the video image, which contains the critical video content which is intended to be retained after a cropping operation (e.g. for aspect ratio conversion).   The shoot and protect aspect ratio will generally be different from the picture aspect ratio (e.g. there may be a 14:9 shoot and protect area within a 16:9 picture).

# A.8 Color space

-	The scalability extension shall support a base and enhancement layer with different color spaces

-	The scalability extension shall support a combination of color space scalability and spatial scalability.

-	The scalability extension shall support tristimulus values in the enhancement layer that can not be represented in the base layer color space.

  - Open Issue: Should we support different color space across the layers in Phase 1. If yes then should we include this in CfP for Phase 1?