INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 M22559

November 2011, Geneva, Switzerland

| Source: | WG11 AHG on Study of HEVC Extensions |
| --- | --- |
| Purpose: | Information and Discussion |
| Title: | Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC |
| Chair: | Ajay Luthra |
| Co-Chairs: | Jens-Rainer Ohm, Jörn Ostermann |

Abstract

This document provides the draft of the call for proposals associated with the scalable enhancement of HEVC as discussed by the WG11 adhoc group on Study of HEVC Extension (N12195).

# Introduction

A new generation of video compression technology known as High Efficiency Video Coding (HEVC) that has substantially higher compression capability than the existing AVC (ITU-T H.264 | ISO/IEC 14496-10) standard is being developed jointly by ISO/IEC MPEG and ITU-T/VCEG. This Call for Proposal is targeting at scalable extensions of that standard. More background information, as well as information about applications and requirements, is given in [XX].

Companies and organizations are invited to submit proposals in response to this Call.

# Results of these tests will be made public, taking into account that no direct identification of any of the proponents will be made (unless it is specifically requested or authorized by a proponent to be explicitly identified). Prior to having evaluated the results of the tests, no commitment to any course of action regarding the proposed technology can be made.

Descriptions of proposals shall be registered as input documents to the proposal evaluation meeting of ??? 2012 (see timeline in section 3). Proponents need to attend this meeting to present their proposals. Further information about logistical steps to attend the meeting can be obtained from the listed contact persons (see section xx).

# Purpose

# Timeline

2012/xx/xx	Draft Call for Proposals

2012/xx/xx	Availability of test materials

2012/xx/xx	Availability of single layer anchors

2012/xx/xx	Preliminary expression of interest

2012/xx/xx	Final Call for Proposals

2012/xx/xx	Formal Registration

2012/xx/xx	Submission of document to chairs

2012/xx/xx	Test results available

201x/xx/xx	Evaluation of proposals and start of collaborative design phase

# Test Conditions

# Category 1 (HEVC Base -> HEVC+ Enhancement, 2D video)

Base layer uses HEVC coding tools and enhancement layer uses HEVC and other coding tools (extensions).

# Test Material

Test material is progressively scanned in 4:2:0 color sampling and 8 bits per component. The class of video is known as Class B (others ??) in HEVC single layer project. Can we get 3840x2160 sequences? If not then consider using 2560x1600 – 1280x800.

# Parameters and Conditions

# Spatial Scalability

Number of layers: 2

Spatial resolution factor (enhancement/base) of 1.5 in each x and y direction. Same PAR and PSAR in the two layers. Enhancement layer: 1920x1080x60/50p and Base layer:1280x720x60/50p.

Spatial resolution factor (enhancement/base) of 2.0 in each x and y direction. Same PAR and PSAR in the two layers. Enhancement layer: 3840x2160x60/50p and base layer: 1920x1080x60/50p and/or Enhancement layer: 2560x1600 and base layer:1280x800.

Open item: Is base layer the same for everyone or considering that due to possible variations in the algorithms only the restriction to have similar visual quality at the base layer should be imposed. (Ed. Comment – mixed response so far)

Suggestion on the reflector (slightly edited): If there is an objection to requiring the same base layer, how about if a non-required, optional base layer anchor is provided?  Respondents who choose not to use the provided base layer anchor would have to match the total sequence bitrates, and would describe how their base layer encodings differ from the anchor. Open item in this case: how is that base layer created (filtering parameters etc).

Bit rates: To be discussed.

Anchors:

HEVC single layer at base resolution (Open Items: decimation process …), using HMx.n, Qp = 22, 27, 32, 37.

HEVC single layer at enhanced resolution at bit rates = x + s*s*x*n, where x is the base layer bit rate, s is the sale factor (1.5 or 2) and n is a variable with values TBD. ?? Should we use fix Qp for enhancement layer also ??

# Aspect Ratio scalability (??)
Number of layers: 2
Different aspect ratio in base and enhancement layers:
HD/VGA: 1280x720/640x480, HD/WVGA: 1280x720/853x480 (Cinemascope ??)

# PAR - HD: 16x9, VGA:4x3, WVGA: 16x9

# Case 1a: PSAR 1 in both enhancement and base layer. E.g. Enhancement Layer is 1280x720 and the base layer is 853x480 (WVGA).

# Case 1b: PSAR 1 in both enhancement and base layer but base layer is the scaled and cropped version of the enhancement layer, e.g. enhancement layer is 1280x720 and base layer is 640x480 (VGA) but PSAR of 1.

# Case 2: PSAR is 1 in enhancement layer and is 4/3 in the base layer, e.g. Enhancement layer is 1280x720 and the base layer is 640x480 (base layer is generated by linear filtering but unequal decimation factors in x and y directions).

# Case 3: same as Case 2 but with non-linear (content-adaptive) scaling.

# Open Item: Which cases, if any, to include in the phase 1 and CfP??

# SNR scalability

Number of layers: 2, each 1920x1080x60/50p

Bit rates: base layer: 0.75x, Enhancement layer: 0.25x (Note: bit rates at the both layers are tied to same x factor) ?? x = 6, 8 and10 Mbps for 1080p60/50 ??

Anchors:

HEVC single layers at lower (0.75x) and total (1x) bit rates.

HEVC single layer at lower rate to be used as the base layer for SNR scalability

# Combined Spatial and SNR scalability testing (??)

Number of layers: 3

Spatial resolution factor: 1.5 or 2 (Comment: pick one) in each direction (same as in 4.1.2.1)

SNR Scalable layer on top of higher resolution spatial layer with 25% (of combined 2 spatial layers) additional bits.

Anchors:

HEVC single layer at lower resolution, HEVC single layer at enhanced resolution and HEVC single layer at total rate of 3 layers.

Suggestion on the reflector: Either test Spatial and SNR scalability separately or combined but not both cases (i.e. do not test Spatial and SNR separately and combined).

# Combined Spatial and Temporal scalability testing (??)

# Test Procedure

Keep the base layer visual quality similar (same?) for everyone.

Objective testing

Spatial: PSNR vs total bit rate at the enhanced resolution for similar (same?) visual quality of the Base layer. Compare that to HEVC single layer Anchors (rates??).

SNR: Compare PSNR vs Bit rates at the enhanced layer against single layer.

Subjective Testing

Spatial: TBD

SNR: No  (??)

# Category 2: Coding standard scalability (AVC Base -> HEVC+, 2D video only) (if this is decided to be done in phase 1 in Geneva meeting in Nov/Dec 2011)

# Base layer uses AVC High Profile coding tools and enhancement layer uses HEVC and other coding tools.

# Test Material

Same as in 4.1.1

# Spatial Scalability

Number of layers: 2

Base layer: AVC High Profile

Enhancement layer: HEVC and other coding tools

# Procedure

Objective

Subjective

TBD

# Category 3: 3D Video with AVC Base -> HEVC+, HEVC Base -> HEVC+ (if this is decided to be done in phase 1 in Geneva meeting in Nov/Dec 2011)

# Base layer uses AVC High Profile coding tools and enhancement layer uses HEVC and other coding tools.

# Base layer uses HEVC coding tools and enhancement layer uses HEVC and other coding tools.

# Test Material

TBD

# View scalability

Number of layers: 2 (left eye, full resolution AVC coded, right eye full resolution coded with HEVC and other coding tools)

# View scalability

Number of layers: 2 (left eye, full resolution HEVC coded, right eye full resolution coded with HEVC and other coding tools)

# Fees (if any)

# Requirements on Submissions

# Requirements

Submit contributions in one or more categories of interest (Comment: expand).

The technical description shall contain information suitable to assess the complexity of the implementation of the technology, including the following:

      Encoding time (for each submitted bitstream) of the software implementation.  Proponents shall provide a description of the platform and methodology used to determine the time.  To help interpretation, a description of software and algorithm optimisations undertaken, if any, is welcome.

      Decoding time for each bitstream running the software implementation of the proposal, and for the corresponding constraint case anchor bitstream(s) run on the same platform.  Proponents shall provide a description of the platform and methodology used to determine the time.  To help interpretation, a description of software optimisations undertaken, if any, is encouraged.

      Expected memory usage of encoder and decoder.

      Complexity characteristics of Motion Estimation (ME) / Motion Compensation (MC): E.g. number of reference pictures, sizes of frame (and associated decoder data) memories, sample value wordlength, block size, and motion compensation interpolation filter(s).

      Description of transform(s): use of integer/floating point precision, transform characteristics (such as length of the filter/block size).

      Degree of capability for parallel processing.

# Source Code and IPR

# Contact

# References