| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G955 |
| --- | --- |

| Title: | Joint contribution on the integration of several chroma coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Tangi Poirier, Guillaume Laroche, Edouard François, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, FRANCE Yi-Jen Chiu, Yu Han, Lidong Xu, Wenhao Zhang, Hong Jiang Intel Corp. 2200 Mission College Blvd. Santa Clara CA USA 95054 Akira Minezawa, Kazuo Sugimoto, Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Atsuro Ichigaya, Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +33-299-87-96-14 christophe.gisquet@crf.canon.fr +1-408-765-9494 yi-jen.chiu@intel.com +81-467-41-2463 minezawa.akira@ds.MitsubishiElectric.co.jp +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | Canon Research Centre France, Intel Corp., Mitsubishi Electric Corporation, NHK(Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This joint contribution presents the results of combining several coding tools impacting chroma together. Namely those tools are “Cross-channel Intra chroma residual prediction” (JCTVC-G173), “Luma-based chroma prediction - Model correction” (JCTVC-G244), “On additional SAO Band Offset classifications” (JCTVC-G246) and “Improvement to chroma TU specification” (JCTVC-G442). Compared to the individual tools, it is reported that their combinations are showing near-additive results, with BD rate equal to -0.4%, -7.7% and -9.7% for respectively Y, U and V components for AIHE configuration and -0.5%, -8.2% and -9.9% for AILC configuration. For these Intra configurations AIHE and AILC, the encoding and decoding time variations are respectively 100% and 100%, compared to HM4.0 without bugfix on alpha bits. When the weight of distortion of the different color components is changed as proposed in JCTVC-G401, a BD rate gain of -1.1% for luma component is reported.

# Detailed results

## Results for combination results

The YUV column reports the BD rate obtained when using the value PSNRAV = (6*PSNRY + PSNRU + PSNRV)/8 as mentioned by several authors.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.5% | -17.4% | -22.2% | -3.3% | -0.6% | -17.5% | -20.9% | -3.0% |
| Class B | -0.2% | -4.8% | -7.0% | -1.2% | -0.4% | -4.9% | -7.0% | -1.2% |
| Class C | -0.6% | -4.0% | -5.5% | -1.4% | -0.7% | -5.0% | -6.9% | -1.6% |
| Class D | -0.6% | -3.7% | -5.0% | -1.2% | -0.6% | -4.3% | -6.2% | -1.4% |
| Class E | -0.2% | -9.8% | -9.3% | -1.9% | -0.5% | -10.5% | -8.7% | -1.9% |
| Overall | -0.4% | -7.7% | -9.7% | -1.8% | -0.5% | -8.1% | -9.9% | -1.8% |
|  | -0.4% | -7.6% | -9.6% | -1.8% | -0.5% | -8.1% | -9.7% | -1.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

The contribution JCTVC-G401 [1] provides an analysis of coding performance behavior between Luma and Chroma component. It is showed that modified the weight between Luma and Chroma in the distortion evaluation can convert Chroma gains in Luma gains.

The current RDO decision method of HM4.0 is given by the following formula:

min(SSEY+ω(SSEU+SSEV))+λR

We changed this formula by the following:

min(SSEY+0.65∗ω(SSEU+SSEV))∗64+2∗0.65+λR

The performance obtained is showed in the following table. In average, for both Intra configurations, the BDR equals to -1.1% for Luma component. Moreover, the average BDR for Chroma components vary between -1.4% to -3.7%. Eventually, the encoding time and decoding decrease compared to the current anchor.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -1.0% | -11.7% | -17.7% | -2.9% | -1.1% | -12.1% | -16.1% | -2.7% |
| Class B | -0.9% | 2.0% | 0.1% | -0.6% | -1.0% | 1.6% | -0.2% | -0.7% |
| Class C | -1.4% | 2.1% | 1.0% | -0.9% | -1.3% | 0.5% | -1.2% | -1.2% |
| Class D | -1.3% | 2.3% | 1.3% | -0.8% | -1.2% | 1.4% | -0.4% | -0.9% |
| Class E | -0.7% | -3.3% | -2.1% | -1.0% | -0.9% | -3.0% | -0.7% | -1.0% |
| Overall | -1.1% | -1.4% | -3.4% | -1.2% | -1.1% | -2.1% | -3.7% | -1.3% |
|  | -1.1% | -1.5% | -3.4% | -1.2% | -1.1% | -2.1% | -3.6% | -1.3% |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 98% |  |  |  |

## Individual results

### Cross-channel intra chroma residual prediction (JCTVC-G173 [2])

The performance of cross-channel intra chroma residual prediction on HM 4.0 reference software

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3% | -0.2% | -2.7% | -0.2% | 0.2% | -3.0% |
| Class B | -0.1% | -0.3% | -3.2% | -0.1% | 0.4% | -3.4% |
| Class C | -0.5% | -1.8% | -3.0% | -0.5% | -1.5% | -3.9% |
| Class D | -0.5% | -1.9% | -3.0% | -0.5% | -1.6% | -4.0% |
| Class E | -0.3% | -1.1% | -2.8% | -0.2% | -1.0% | -3.4% |
| Class F | -0.5% | -1.5% | -2.0% | -0.4% | -1.5% | -2.4% |
| Overall A-E | -0.3% | -1.0% | -3.0% | -0.3% | -0.6% | -3.5% |
| Overall A-F | -0.4% | -1.1% | -2.8% | -0.3% | -0.8% | -3.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

### Luma-based chroma prediction - Model correction (JCTVC-G244 [3])

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -14.1% | -16.8% | -0.1% | -14.2% | -16.0% |
| Class B | 0.0% | -1.8% | -1.0% | 0.0% | -2.0% | -1.0% |
| Class C | -0.1% | -1.3% | -1.7% | -0.1% | -1.6% | -1.5% |
| Class D | -0.1% | -1.4% | -1.4% | -0.1% | -1.5% | -1.5% |
| Class E | 0.0% | -2.6% | -1.6% | -0.1% | -2.9% | -1.7% |
| Overall | 0.0% | -4.2% | -4.5% | -0.1% | -4.4% | -4.3% |
|  | 0.0% | -4.1% | -4.4% | -0.1% | -4.3% | -4.2% |
| Enc Time[%] | 100.3% |  |  | 100.5% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.7% |  |  |

### On additional SAO Band Offset classifications (JCTVC-G246 [4])

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.9% | -1.4% | -0.1% | -0.3% | -0.5% |
| Class B | 0.1% | -1.5% | -2.0% | 0.0% | -1.0% | -1.4% |
| Class C | 0.1% | -0.9% | -1.2% | 0.1% | -0.7% | -0.9% |
| Class D | 0.0% | -0.6% | -0.9% | 0.0% | -0.6% | -0.7% |
| Class E | 0.2% | -2.9% | -3.0% | 0.0% | -1.0% | -1.0% |
| Overall | 0.0% | -1.3% | -1.6% | 0.0% | -0.7% | -0.9% |
|  | 0.0% | -1.3% | -1.6% | 0.0% | -0.8% | -0.9% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

### Improvement to chroma TU specification (JCTVC-G442 [5])

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -4.3% | -4.3% | -0.2% | -3.8% | -3.8% |
| Class B | -0.2% | -1.8% | -1.9% | -0.3% | -2.7% | -3.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -1.6% | -1.9% |
| Class D | -0.1% | -0.3% | -0.2% | -0.1% | -1.1% | -1.2% |
| Class E | -0.1% | -4.3% | -3.2% | -0.3% | -6.3% | -5.2% |
| Overall | -0.1% | -2.1% | -1.9% | -0.2% | -2.9% | -3.0% |
|  | -0.1% | -2.1% | -1.9% | -0.2% | -2.9% | -3.1% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This contribution reports the results of the combination of several tools, showing near-additive results, with BD rate improvements of 0.4%/7.7%/9.7% (Y/U/V) for AIHE and 0.5%/8.2%/9.9% (Y/U/V) for AILC. The average BDR gain computed by using the PSNRAV is 1.8% for both configurations without impact on encoding and decoding times. By modifying the RDO selection in order to report these significant Chroma gains to Luma, the BD rate improvements is 1.1%/1.4%/3.4% (Y/U/V) for AIHE and 1.1%/2.1%/3.7% (Y/U/V). Moreover this change gives encoding time reduction of 1% and a decoding time reduction from 2% to 3%.

Since proposed modifications achieve important coding gain without notable complexity increase, the companies represented in this document recommend considering the adoption of the four proposed technologies

# References

- B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft), “RDO with weighted distortion in HEVC”, JCTVC-G401, 7th JCT-VC meeting, Geneva, November 2011.

- Y. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel), “Cross-channel intra chroma residual prediction” JCTVC-G173, 7th JCT-VC meeting, Geneva, November 2011.

- C. Gisquet, E. François (Canon), “Non-CE6a: Luma-based chroma prediction - Model correction” JCTVC-G244, 7th JCT-VC meeting, Geneva, November 2011.

- G. Laroche, T. Poirier, P. Onno (Canon), “On additional SAO Band Offset classifications” JCTVC-G246, 7th JCT-VC meeting, Geneva, November 2011.

- A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi), A. Ichigaya, S. Sakaida (NHK), “Improvement to chroma TU specification” JCTVC-G442, 7th JCT-VC meeting, Geneva, November 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).