Working draft for Modification for JCTVC-G173

8.3.3.1.9 Specification of cross-channel intra chroma prediction

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring chroma samples pCb[ x, y ] and pCr[ x, y ], with x, y = -1..nS-1,

–	a variable nS specifying the prediction size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	the predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This process is invoked when decoding cIdx=1 and intraPredMode is not equal to 35 (LM mode).

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the follows:

predSamples[ x, y ] = Clip1C ( predSamplesCr[x, y] + α * resSamplesCb [ x, y ] ), with x, y = 0..nS-1

Where, predSamplesCr is the array of Cr samples predicted by intra directional prediction. α is default to be -1/2.

### 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| lm_chroma_parameters_flag | u(1) |
| if( lm_chroma_parameters_flag ) { |  |
| lm_chroma_param( ) |  |
| } |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

#### Alternate LM chroma prediction parameters decoding

| lm_chroma_param( ) { | Descriptor |
| --- | --- |
| quantization_alternate_threshold[0] | u(2) |
| if( quantization_alternate_threshold[0] ) |  |
| quantization_alternate_alpha_index[0] | u(4) |
| image_alternate_threshold[0] | u(3) |
| if( image_alternate_threshold[0] ) { |  |
| image_alternate_low_alpha_index[0] | u(4) |
| image_alternate_positive_alpha_index[0] | u(4) |
| image_alternate_negative_alpha_index[0] | u(4) |
| } |  |
| quantization_alternate_threshold[1] | u(2) |
| if( quantization_alternate_threshold[1] ) |  |
| quantization_alternate_alpha_index[1] | u(4) |
| image_alternate_threshold[1] | u(3) |
| if( image_alternate_threshold[1] ) { |  |
| image_alternate_low_alpha_index[1] | u(4) |
| image_alternate_positive_alpha_index[1] | u(4) |
| image_alternate_negative_alpha_index[1] | u(4) |
| } |  |
| } |  |

##### 8.3.3.1.8 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	the chroma component index uiChromaIdx

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = -1..2*nS	(8-2)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-3)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-4)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-5)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-6)

k2 = log2( (2*nS) >> k3 )		(8-7)

- Variables a and b are derived as:

a1 	 = ( LC << k2 ) – L*C		(8-8)
a2 	 = ( LL << k2 ) – L*L		(8-9)
ua1 = abs(a1) 		(8-10)
uiT1 = (quantization_alternate_threshold[uiChromaIdx]*QP)<<( BitDepthC + log2( nS ) – 10 ) 	(8-11)
if( ua1<uiT1 || a2<uiT1 ) {		(8-12)
    a            = lmAlternateAlpha[ quantization_alternate_alpha_index[uiChromaIdx] ]	(8-13)
    iShift     = 5		(8-14)
} else if( ua1 < image_alternate_threshold[uiChromaIdx] * (a2>>5) ) {	(8-15)
     if( ua1 < (a2>>6) ) {		(8-16)
        a        = lmAlternateAlpha[ alternate_low_alpha[uiChromaIdx] ]	(8-17)
        iShift = 7		(8-18)
     } else if ( a1 < 0 ) {		(8-19)
        a        = lmAlternateAlpha[ alternate_negative_alpha[uiChromaIdx] ]	(8-20)
        iShift = 6		(8-21)
     } else {		(8-22)
        a        = lmAlternateAlpha[ alternate_positive_alpha[uiChromaIdx] ]	(8-23)
        iShift = 6		(8-24)
    }			(8-25)
} else {		(8-26)
     k 	   = Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(8-27)
     a1s	   = a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(8-28)
     a2s	   = abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) + 1	(8-29)
     iShift = 13		(8-30)

[Ed. (WJ): bug in software – a2s should be abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) – without ‘+1’]

a	= Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(8-31)
}				(8-32)

b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(8-33)

where lmDiv is specified in Error: Reference source not found with the input a2s and lmAlternateAlpha below.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> iShift ) + b ), with x, y = 0..nS-1	(8-34)

lmAlternateAlpha[16] = [ -11,  -9,  -7,  -6,  -5, -4, -3, -2, -1, 1, 2,  3,  4,  5,  7,  9 ]

Working draft for Modification for JCTVC-G246

#### Sample adaptive offset parameter syntax

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) ¦ ae(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] > 5 && sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] !=12) { |  |
| sao_band_center[ cIdx ][ saoDepth ][ x0 ][ y0 ] + 15 | se(v) ¦ ae(v) |
| } |  |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) ¦ ae(v) |
| } |  |
| } |  |

#### 7.4.3.4 Sample adaptive offset parameter semantics

sample_adaptive_offset_flag specifies whether sample adaptive offset applies or not for the current picture.

sao_flag_cb equal to 1 denotes sample adaptive offset process for Cb shall be applied to the current picture.

sao_flag_cr equal to 1 denotes sample adaptive offset process for Cr shall be applied to the current picture.

sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] specifies whether a region is split into four sub regions with half horizontal and vertical number of LCU for the color component cIdx. The array indices rx and ry specify the region index and saoDepth specifies the split depth of the region. When sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] is not present, it shall be inferred to be equal to 0.

The maximum allowed depth for sample adaptive offset process SaoMaxDepth is derived as follows:

SaoMaxDepth = Min( 4, Min( Floor( Log2( PicWidthInLCUs ) ), Floor( Log2( PicHeightInLCUs ) ) ) )	(7-35)

where

PicWidthInLCUs = Ceil( PicWidthInSamplesL ÷ ( 1 << Log2MaxCUSize ) )	(7-36)
PicHeightInLCUs = Ceil( PicHeightInSamplesL ÷ ( 1 << Log2MaxCUSize ) )	(7-37)

[Ed.: (WJ) PicWidthInLCUs and PicHeightInLCUs could be defined elsewhere]

sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] indicates the offset type for the color component cIdx of the region specified by saoDepth, rx and ry.

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] indicates the offset value of i-th category for the color component cIdx of the region specified by saoDepth, rx and ry.

sao_band_center[ cIdx ][ saoDepth ][ rx ][ ry ] indicates the center of the interval when sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 6, 7, 8, 9, 10, 11, 13, 14.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

The offset value shall be in the range of [ -( 1 << ( SaoBitsRange – 1) ), ( 1<< ( SaoBitsRange – 1) ) – 1 ] where

SaoBitRange = Min( bitDepth, 10 ) – 4		(7-38)

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0		(7-39)

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i + 1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1		(7-40)

The number of categories, NumSaoClass, is specified in Table 7-1.

Table 7-1 – Specification of NumSaoClass

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4 | - 1D 0-degree edge |
| - 2 | - 4 | - 1D 90-degree edge |
| - 3 | - 4 | - 1D 135-degree edge |
| - 4 | - 4 | - 1D 45-degree edge |
| - 5 | - 16 | - Side band |
| - 6 | - 2 | - Band: 1/16th of range, 5 most significant bits |
| - 7 | - 4 | - Band: 1/8th of range, 5 most significant bits |
| - 8 | - 4 | - Band: 1/16th of range, 6 most significant bits |
| - 9 | - 8 | - Band: 1/4th of range, 5 most significant bits |
| - 10 | - 8 | - Band: 1/8th of range, 6 most significant bits |
| - 11 | - 8 | - Band: 1/4th of range, 7 most significant bits |
| - 612 | - 16 | - Central band |
| - 13 | - 16 | - Band: 1/4th of range, 6 most significant bits |
| - 14 | - 16 | - Band: 1/8th of range, 7 most significant bits |

##### 7.4.3.4 Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a variable NumSaoCategory specifying the number of categories,

–	a variable bitDepth specifying the bit deph of current component,

–	an array sao_band_center specifying center of band.

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-2.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-41)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-42)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 612, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- A variable bandType set equal to 0 if saoTypeIdx is equal to 12 otherwise bandType set equal  to 1.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 bandType][ bandIdx ] ]	(8-43)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-3.

–	Otherwise, if saoTypeIdx is equal to one of the values of 6, 7, 8, 9, 10, 11, 13 or 14, the following ordered steps applies:

- A variable bandShift is set equal to BitDepth – bandShiftTable[ saoTypeIdx – 6 ]. bandShiftTable is specified in Table 8.18.

- A variable SaoBandCenter is set equal to (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] << ( bandShift – 5 )).

- A variable MinSaoBand is set equal to ( SaoBandCenter ) – (NumSaoClass >> 1 ) + 1

- A variable MaxSaoBand is set equal to ( SaoBandCenter ) + ( NumSaoClass >> 1 )

- A variable bandTable is modified as

bandTable[i] = 0 with i =0..MinSaoBand-1

bandTable[i] = i- MinSaoBand with i = MinSaoBand..MaxSaoBand

bandTable[i] = 0 with i = MaxSaoBand+1..( 1 << bandShift ) - 1

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ bandTable[ bandIdx ] ]	(8-44)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift )

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-45)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table 8-2 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

Table 8-3 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Table 8-18 – Specification of array bandShiftTable according to saoTypeIdx – 6

| - saoTypeIdx – 6 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - bandShiftTable [saoTypeIdx – 6 ] | - 5 | - 5 | - 6 | - 5 | - 6 | - 7 | - 5 | - 6 | - 7 |

Working draft for Modification for JCTVC-G442

### 7.3.6  Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( entropy_coding_mode_flag && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) { |  |
| if( !(PartMode == PART_2Nx2N && merge_flag[ x0 ][ y0 ]) ) |  |
| no_residual_data_flag | ae(v) |
| } |  |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| chromaResidualDataPresentFlag = 0 |  |
| transform_tree_first( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_tree_second( x0, y0, log2CUSize, log2CUSize, 0, 0) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.8  Transform tree syntax

### 7.3.8.1  First transform tree syntax

| transform_tree_first( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 2 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] = ( ((PartMode != PART_2Nx2N) && (trafoDepth = = 0)) ¦¦ (log2TrafoSize > Log2MaxTrafoSize) ) |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 ) { |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if (cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1) |  |
| chromaResidualDataPresentFlag = 1 |  |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if (cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1) |  |
| chromaResidualDataPresentFlag = 1 |  |
| } |  |
| } |  |
| } |  |
| if( inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree_first ( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree_first ( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree_first ( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree_first ( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } |  |
| } |  |

### 7.3.8.2  Second transform tree syntax

| transform_tree_second( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( !entropy_coding_mode_flag && trafoDepth != 0 ) { |  |
| if( !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform_luma | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree_second ( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree_second ( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree_second ( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree_second ( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ chromaResidualDataPresentFlag ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && ((log2TrafoSize < Log2MaxTrafoSize) ¦¦ (chromaResidualDataPresentFlag && (log2CUSize <= Log2MaxTrafoSize+1) ) ) && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |

### 7.3.9  Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| split_flag = (cIdx = = 0) ? split_transform_flag[ x0 ][ y0 ][ trafoDepth ] : inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] |  |
| if( split_flag && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |

[Ed. (BB): the condition ( cIdx  = =  0  | |  log2TrafoSize > Log2MinTrafoSize + 1 ) assumes 4:2:0 color sampling and assures that the luma log2TrafoSize is greater than Log2MinTrafoSize for chroma. ]

### 7.4.8  Transform tree semantics

…

…

…

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a luma block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If log2TrafoSize is greater than Log2MaxTrafoSize or intraSplitFlag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- Otherwise (log2TrafoSize is less than or equal to Log2MaxTrafoSize and intraSplitFlag is equal to 0), the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

inferred_split_transform_flag [ x0 ][ y0 ][ trafoDepth ] specifies whether a chroma block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

…

…

…

cbp_and_split_transform contains a number of coded block flags and/or a split_transform_flag. Depending on a 4-bit value flagPattern defined as follows, the value of cbp_and_split_transform is defined according to Table 7-15.

codeLuma = trafoDepth  = =  0  | |  cbf_luma[ xB ][ yB ][ trafoDepth − 1 ]
codeCb = 		trafoDepth  = =  0  | | (cbf_cb[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize )
codeCr = 		trafoDepth  = =  0  | | (cbf_cr[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize ) 	(7-46)
codeSplitTrans  =  log2TrafoSize  <=  Log2MaxTrafoSize  &&  
					log2TrafoSize  >  Log2MinTrafoSize  &&  
					trafoDepth  <  maxDepth  &&  
					!intraSplitFlag
flagPattern = (codeLuma << 3) + (codeCb << 2) + (codeCr << 1) + codeSplitTrans

The values of codeLuma, codeCb, codeCr and codeSplitTrans specify respectivley if an associated flag of cbf_luma, cbf_cb, cbf_cr, and split_transform_flag is included in the syntax element cbp_and_split_transform.

In Table 7-15, Luma, Cb, Cr and Split denote the values of cbf_luma[ x0 ][ y0 ][ trafoDepth ], cbf_cb[ x0 ][ y0 ][ trafoDepth ], cbf_cr[ x0 ][ y0 ][ trafoDepth ] and split_transform_flag[ x0 ][ y0 ][ trafoDepth ] respectively. Luma1, Luma2 and Luma3 denote respectively the values of cbf_luma[ x1 ][ y1 ][ trafoDepth ], cbf_luma[ x2 ][ y2 ][ trafoDepth ] and cbf_luma[ x3 ][ y3 ][ trafoDepth ]. The values of x1, y1, x2, y2, x3 and y3 are derived as follows.

If InterTUSplitDirection is equal to 2,

x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 )		(7-47)

y1 = y0			(7-48)

x2 = x0			(7-49)

y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 )		(7-50)

x3 = x1			(7-51)

y3 = y2			(7-52)

Otherwise (InterTUSplitDirection is equal to 0 or 1),

x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-53)

y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-54)

x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-55)

y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-56)

x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-57)

y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-58)

Table 7-15 - Specification of cbp_and_split_transform

| - flagPattern | - cbp_and_split_transform |
| --- | --- |
| - 8 | - (Luma<<3) + (Luma1<<2) + (Luma2<<1) + Luma3 - 1 |
| - 9 | - (Luma<<1) + Split |
| - 10 | - (Luma<<1) + Cr |
| - 11 | - (Luma<<2) + (Cr<<1) + Split |
| - 12 | - (Luma<<1) + Cb |
| - 13 | - (Luma<<2) + (Cb<<1) + Split |
| - 14 | - (Luma<<2) + (Cb<<1) + Cr |
| - 15 | - (Luma<<2) + ((Cb¦Cr)<<1) + Split |

cbp_and_split_transform_luma contains a number of coded block flags and/or a split_transform_flag. Depending on a 4-bit value flagPattern defined as follows, the value of cbp_and_split_transform is defined according to Table 7-15.

codeLuma = trafoDepth  = =  0  | |  cbf_luma[ xB ][ yB ][ trafoDepth − 1 ]	(7-59)
codeSplitTrans  =  log2TrafoSize  <=  Log2MaxTrafoSize  &&  
					log2TrafoSize  >  Log2MinTrafoSize  &&  
					trafoDepth  <  maxDepth  &&  
					!intraSplitFlag
flagPattern = (codeLuma << 3) + codeSplitTrans

The values of codeLuma and codeSplitTrans specify respectivley if an associated flag of cbf_luma and split_transform_flag is included in the syntax element cbp_and_split_transform.

### 8.4.3  Decoding process for the residual signal of coding units coded in inter prediction mode

…

…

…

### 8.4.3.2  Decoding process for chroma residual blocks

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nCS specifying the size, in chroma samples, of the current coding unit,

–	a (nCS)x(nCS) array resSamples of chroma residual samples.

Output of this process is:

–	a modified version of the (nCS)x(nCS) array of chroma residual samples.

The variable splitChromaFlag is derived as follows:

–	If inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than Log2MinTrafoSize + 1, splitChromaFlag is set equal to 1.

–	Otherwise (inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to Log2MinTrafoSize + 1), splitChromaFlag is set equal to 0.

Depending inferred_splitChromaFlag, the following applies:

–	If inferred_splitChromaFlag is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB2 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB3, yB3 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

–	Otherwise (inferred_splitChromaFlag is equal to 0), the following ordered steps apply:

- The variable nW is set equal to ( 1 << log2TrafoWidth ) >> 1.

- The variable nH is set equal to (1 << log2TrafoHeight ) >> 1.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xC + xB, yC +yB ), the variable trafoDepth, the variable cIdx, the transform width trafoWidth set equal to nW, and the transform height trafoHeight set equal to nH as the inputs and the output is a (nW)x(nH) array resSamplesBlock.

The array construction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), the variable cIdx, the variable inputArrayWidth set equal to nW, the variable inputArrayHeight set equal to nH, the variable outputArraySize set equal to nCS, the (nW)x(nH) array resSamplesBlock, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

### 8.6.1  Deblocking filter process

…

…

…

### 8.6.1.1  Derivation process of luma transform unit boundary

…

…

…

### 8.6.1.2  Derivation process of chroma transform unit boundary

Inputs of this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlagsChroma and verEdgeFlagsChroma.

Depending on inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB1 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB3, yB3 ), the variable log2TrafoWidth1 set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

–	Otherwise (inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following applies:

- If yB is equal to zero, horEdgeFlagsChroma[ xB + k ][ yB ] is set equal to FilterTopCuEdgeFlag, otherwise horEdgeFlagsChroma[ xB + k ][ yB ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoWidth ) – 1.

- If xB is equal to zero, verEdgeFlagsChroma[ xB ][ yB + k ] is set equal to FilterLeftCuEdgeFlag, otherwise verEdgeFlagsChroma[ xB ][ yB + k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoHeight ) – 1.

### 8.6.1.3  Derivation process of prediction unit boundary

…

…

…

#### 8.6.1.4  Derivation process of luma boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the luma boundary filtering strength.

…

…

…

#### 8.6.1.5  Derivation process of chroma boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlagsChroma and verEdgeFlagsChroma.

Output of this process is an array of size (2)x(nS)x(nS), bSChroma specifying the chroma boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlagsChroma[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlagsChroma[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### 8.6.1.6  Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

…

…

…

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bSChroma[ 0 ][ xDk*2 ][ yDm*2 ] 	(8-60)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bSChroma[ 1 ][ xDk*2 ][ yDm*2 ] 	(8-61)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.