| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G956 |
| --- | --- |

| Title: | Non-CE4: Cross-verification of Sony’s proposal (JCTVC-G362) by Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jewon Kang Antti Hallapuro Jani Lainema | Tel: Email: | +358 50 4860857 ext-jewon.kang@nokia.com antti.hallapuro@nokia.com Jani.Lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports a cross verification result of Sony’s proposal on “efficient binarization of delta QP in CABAC” proposed in JCTVC-G362. In the proposal, the binarization of delta QP is modified for the coding efficiency on top of [1]. The experimental results are the same as one provided by the proponent. The proposed scheme further achieves total BD-rate savings about -0.1% in Y, U, and V, and about -2.0 %, -2.7 %, and -2.9 % in delta QP bits, respectively in AI HE, RA HE, and LB HE, over [1].

# Introduction

This contribution is to report the results of JCTVC-G362 by Sony. The proposal removes redundancy in the binarization of [1] using the given MAX, MIN, and the predicted QP. It is noted that the sign bits and some bits signaling the delta QP can be saved using the proposed binarization scheme. The proposed scheme further achieves BD-rate savings in delta QP bits about -2.0 %, -2.7 %, and -2.9 %, respectively in AI HE, RA HE, and LB HE, over [1].

# Software Implementation

There was no problem in compiling or running the software. The changes can be found under “SONY_DQP_BINARY” flag. In implementation, the proposal employs “JCTVC-F174 and JCTVC-F745 harmonization” scheme which has been issued in CE4 reflector [2], even though the relevant contents are skipped in the proposal. The table I summarize the related software condition used in this proposal and its reference [1].

|  | JCTVC-F174 [1] | The proposal |
| --- | --- | --- |
| MAX_MIN_QP_F174 [1] | O | O |
| USE_MIN0_MAX51_F174 [2] | X | X |
| F174_AND_F745_HARMONIZATION[2] | X | O |
| SONY_DQP_BINARY | - | O |

# Experimental results

The experimental conditions are the same as CE4 subset 1 [3]. The proposal is implemented in JCTVC-F174 software and the software is provided by the proponent for the cross checking. We confirm that the results are the same with what proponent reported in the contribution.

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -2.2% |
| Class B | -0.1% | -0.1% | -0.1% | -1.9% |
| Class C | -0.1% | -0.1% | -0.1% | -1.8% |
| Class D | -0.1% | -0.1% | -0.1% | -2.2% |
| Class E | -0.1% | -0.1% | -0.1% | -1.8% |
| Overall | -0.1% | -0.1% | -0.1% | -2.0% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Random Access HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -2.6% |
| Class B | -0.1% | -0.1% | -0.1% | -2.3% |
| Class C | -0.1% | -0.1% | -0.1% | -2.6% |
| Class D | -0.1% | -0.1% | -0.1% | -3.2% |
| Class E |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -2.7% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Low delay B HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -2.6% |
| Class C | -0.1% | -0.1% | -0.1% | -2.8% |
| Class D | -0.1% | -0.1% | -0.1% | -3.6% |
| Class E | -0.1% | -0.1% | -0.1% | -2.5% |
| Overall | -0.1% | -0.1% | -0.1% | -2.9% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 1: Summary results of JCTVC-G362 as compared to JCTVC-F174.

# Comments

The proposal enhances the binarization scheme of [1], achieving BD-rate saving about 0.1% for Y, U, and V, and about -2.0 %, -2.7 %, and -2.9 % of the delta QP bits, respectively in AI HE, RA HE, and LB HE, as compared to [1]. There can be possible coding gain or loss from the harmonization scheme above, and thus it is worth to see how much coding gain/loss actually comes from the proposal. It is also suggested to compare the results with the CE4 anchor.

# Reference

- K. Sugimoto, S. Sekiguchi, “Signaling of Max and Min QP in slice”, JCTVC-F174, Torino, July 2011.

- Keiichi Chono and Hirofumi Aoki, “CE4 subtest 1.2.a: study and cross-verification of Mitsubishi’s signaling of Max and Min QP in slice,” JCTVC-G054, Geneva, November, 2011.

- “Description of Core Experiment 4: Quantization”, JCTVC-F904r3, Torino, July 2011.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).