| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G958 |
| --- | --- |

| Title: | Non-CE11: Modified context selection for Significant Coefficient Flags with Diagonal sub-block scan |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method for context selection, modified from the approach presented in contribution JCTVC-G323 by Qualcomm. Similarly to JCTVC-G323, the context selection is modified along the top-left and above the lower-right position in each sub-block within the significance map of a 16x16 and 32x32 TU. In these locations, a value for a significant coefficient flag, immediately preceding a current significant coefficient flag along the scan pattern, is predicted. This modification was implemented in HM-4.0 on top of JCTVC-G323 and resulted in degradations of 0.0% for IA_HE, 0.0% for RA_HE and 0.0% for LD_HE configurations in the Luma channel (relative to HM-4.0). Use of the predicted value instead of the previous significant coefficient flag enables context index determination to occur before the previous significant coefficient flag has been decoded by the arithmetic decoder, increasing the throughput of a decoder implementation.

# Introduction

To improve bin throughput while processing the significant_coefficient_flag syntax element, this contribution proposes a modification to the context selection process for the top-left and above the lower-right position in each sub-block for 16x16 and 32x32 TUs.

This contribution provides an alternative solution to the parallelism issue solution in JCTVC-G323. JCTVC-G323 introduces a sub-block scan to the significance map. This resulted in two locations in each sub-block where the context derivation for a significant coefficient flag depends on the decoded value of the immediately preceding significant coefficient flag. This slows down hardware implementations because the context selection cannot commence until the preceding significant coefficient flag has been decoded by the arithmetic decoder.

The solution presented in JCTVC-G323 is to exclude the preceding significant coefficient flag from the context derivation in the two identified locations per sub-block.

This contribution presents an alternative method of context derivation for these two locations. This method also excludes the preceding significant coefficient flag value, but uses three significant coefficient flag values to ‘predict’ a value to use in place of the excluded significant coefficient flag.

# Modified context selection for Significant Coefficient Flags

## Background

Figure 1 shows the backward diagonal scan with sub-blocks for an 8x8 TU (however this contribution is concerned only when the sub-block scan is applied tr 16x16 and 32x32 TUs).

Figure 1. Backward diagonal scan with sub-blocks (Illustrated with 8x8 TU).

The dependency map for context derivation for each significant coefficient flag is shown in Figure 2.

NOTE: ‘Xctx’ values clipped at 4, i.e.: range is [0, 4]. Clipping present in HM-4.0, G323 and G958.

Figure 2. Dependency map for selection a context for significant coefficient flag X using previous significant coefficient flags.

JCTVC-G323 by Qualcomm includes a ‘parallel-friendly’ implementation where significant coefficient flag C is excluded from context derivation in two locations in each sub-block, as shown in Figure 3. By excluding C, the context derivation can complete before the arithmetic decode operation of C has completed, increasing throughput.

Figure 3. Parallel-friendly dependency map for specific locations in one 4x4 sub-block.

## Modified context selection

The proposed approach, presented in Figure 4, is to replace the significant coefficient flag C (from Figure 2) with a value P, derived from significant coefficient flags A, D and E. This approach retains parallelism-friendly implementation (‘modified parallel friendly neighborhood’) of significance map context derivation.

The intent of this approach is to harmonise the context derivation for the significant coefficient flags X in Figure 3 with the remaining significant coefficient flags, while retaining the parallel-friendly implementation.

This approach is applied to locations in each sub-block in accordance with Figure 3.

Figure 4. Modified context selection.

# Experiment Results

Results are presented in Table 1 and Table 2 below for the 4x4 sub-block case. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Modified context selection for Significant Coefficient Flags with Diagonal sub-block scan results in HM-4.0.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.3% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% |
| Class E | -0.1% | -0.2% | -0.2% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall (A-F) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Overall (A-E) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1% | 0.3% | 0.5% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | 0.0% | -0.3% |
| Class E |  |  |  |
| Class F | 0.0% | 0.1% | -0.2% |
| Overall (A-F) | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Overall (A-E) | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | -0.3% |
| Class E | 0.1% | 0.0% | 0.4% |
| Class F | -0.2% | -0.5% | 0.1% |
| Overall (A-F) | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |
| Overall (A-E) | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.0% | -0.1% |
| Class C | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | -0.4% | -0.6% |
| Class E | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | 0.0% | -0.3% |
| Overall (A-F) | 0.0% | -0.1% | -0.2% |
|  | 0.0% | 0.0% | -0.3% |
| Overall (A-E) | 0.0% | -0.1% | -0.2% |
|  | 0.0% | 0.0% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2.  Modified context selection for Significant Coefficient Flags with Diagonal sub-block scan results relative to G323.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.06% | -0.07% | -0.10% |
| Class B | -0.05% | -0.09% | -0.08% |
| Class C | -0.03% | -0.01% | -0.03% |
| Class D | -0.01% | -0.06% | -0.09% |
| Class E | -0.06% | -0.07% | -0.11% |
| Class F | -0.02% | -0.01% | -0.09% |
| Overall (A-F) | -0.04% | -0.05% | -0.08% |
|  | -0.04% | -0.06% | -0.09% |
| Overall (A-E) | -0.04% | -0.06% | -0.08% |
|  | -0.04% | -0.07% | -0.09% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.03% | 0.16% | 0.42% |
| Class B | -0.03% | -0.03% | -0.05% |
| Class C | -0.03% | -0.09% | 0.11% |
| Class D | -0.06% | 0.05% | -0.18% |
| Class E |  |  |  |
| Class F | 0.03% | 0.00% | -0.02% |
| Overall (A-F) | -0.02% | 0.02% | 0.05% |
|  | -0.02% | 0.00% | 0.03% |
| Overall (A-E) | -0.04% | 0.02% | 0.07% |
|  | -0.04% | 0.01% | 0.03% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.00% | -0.16% | 0.21% |
| Class C | -0.05% | -0.15% | 0.03% |
| Class D | 0.02% | 0.48% | -0.08% |
| Class E | -0.06% | 0.10% | -0.22% |
| Class F | -0.15% | -0.38% | 0.22% |
| Overall (A-F) | -0.04% | -0.04% | 0.05% |
|  | -0.04% | -0.03% | 0.05% |
| Overall (A-E) | -0.02% | 0.05% | 0.01% |
|  | -0.02% | 0.06% | 0.00% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.01% | -0.15% | 0.02% |
| Class C | 0.02% | 0.28% | -0.03% |
| Class D | -0.01% | -0.58% | -0.36% |
| Class E | -0.05% | -0.17% | -0.44% |
| Class F | -0.01% | -0.04% | -0.22% |
| Overall (A-F) | -0.01% | -0.13% | -0.18% |
|  | -0.01% | -0.08% | -0.19% |
| Overall (A-E) | -0.01% | -0.15% | -0.17% |
|  | -0.01% | -0.12% | -0.20% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Text for Working Draft

Note: Provided WD text is on top of WD text in [2].

###### 9.3.3.1.1.4  Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
- If (xC, yC) is not the top-left corner of a sub-block or (xC, yC+1) is not the bottom-right corner of a sub-block, sigCtx is derived as follows.

sigCtx  =  34  +  tempMin(4, temp + (significant_coeff_flag[ xC + 1 ][ yC ] + significant_coeff_flag[ xC + 1 ][ yC + 1 ] + significant_coeff_flag[ xC ][ yC + 2 ] >> 1)

- Otherwise,

sigCtx  =  34  +  Min(4, temp + significant_coeff_flag[ xC ][ yC + 1 ])

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) – 1 and (xC, yC) is not the top-left corner of a sub-block or (xC, yC+1) is not the bottom-right corner of a sub-block, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1<< log2TrafoSize ) – 1 and (xC, yC) is not the top-left corner of a sub-block or (xC, yC+1) is the bottom-right corner of a sub-block, the following applies.

sigCtxTmp  =  0		(9-9)

  - When xC is less than ( 1<< log2TrafoSize ) – 1.

sigCtxTmp =  sigCtxTmp  +  significant_coeff_flag[ xC + 1 ][ yC ] + significant_coeff_flag[ xC + 1 ][ yC + 1]	 	(9-10)

  - When yC is less than ( 1<< log2TrafoSize ) – 2.

sigCtxTmp  =  sigCtxTmp  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-11)

sigCtx  =  sigCtx  +  sigCtxTmp >> 1		(9-12)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-13)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-14)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-15)

# Conclusion

Presented is an approach to alter the context derivation for the ‘modified parallel friendly neighborhood’. We assert that this approach harmonises the context derivation for significant coefficient flags using the ‘modified parallel friendly neighborhood’ with other significant coefficient flags. Results were presented using HM-4.0.

# References

- C. Rosewarne, M. Maeda, “Predicted neighbour for context selection of significant_coeff_flag for parallel processing”, JCTVC-F186, 6th JCT-VC Meeting, Torino, IT, July 2011.

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding”, JCTVC-G323, 7th JCT-VC Meeting, Geneva, CH, Nov 2011.

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).