INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2011/m22588

18 November 2011, Geneva, CH

Title: 	Interlace profiling in HEVC

Status:	Contribution

Authors: 	David Singer

Organization: 	Apple Inc.

Group: 	Video/Requirements (or Requirements/Video)

# Introduction

The support of interlace display in systems is becoming increasingly difficult.  People used to watch video content on analog CRT displays, with natural ‘softness’ and the phosphor decay, and interlace worked acceptably well.  CRTs are long gone; we watch on crisp LCD displays often with higher resolution than the content, and at short viewing distances.  Under these circumstances simply painting field-by-field yields unacceptable combing artifacts, and motion-estimating, post-filtering, de-interlace must be employed. Even with the best de-interlacers, artifacts remain, and de-interlacing is expensive.

We have before us a proposal to introduce the signaling that the content is interlaced, into HEVC. It has been argued that since no coding tools are involved – no ‘feature’ of the codec – profiling is inappropriate. But of course the expense of interlace lies not in its decoding, but in its presentation. If every implementation can legitimately be exposed to interlaced content – which would be the case if interlace is not profiled – then every implementation (well, except those using CRTs) will need a de-interlacer of some sort.

I therefore strongly believe that:

- the indicators that say “the video in this sequence is composed of fields, not full frames” and the matching indicator(s) “this picture is a top/bottom field”, or their moral equivalents, must have mandatory ‘progressive’ values except in a few, selected interlace profiles.

- Each interlace profile thus defined must have a matching ‘progressive-only’ profile, and that those pairs of profiles are identical in every other respect.  This avoids the situation where some other choice is only in the interlaced profile, and someone is forced into supporting interlace because of that other choice.

We must start guiding interlace to its resting place in history; as a compression technique for analog signals in systems that essentially ran synchronously from camera through to display, it was brilliant.  But we should be able to retire it, and celebrate that, a mere century after its invention in 1925. HEVC deployment should be strong in 2025, and either interlace-free, or mostly so.

I therefore believe that there are only two reasonable outcomes:

- we retain HEVC as progressive only with no mention of interlace or fields anywhere, or

- or we profile the interlace/field indicators

as having interlace ‘bleed’ all over the rest of the world is no longer acceptable.