| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G967 |
| --- | --- |

| Title: | 4:2:2 support in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes support of 4:2:2 format in HEVC standard, which could expand opportunities of its market adoption especially in digital broadcasting area. The main point of this proposal is to introduce modifications as small as possible in order to accommodate 4:2:2 format. Modifications of the current specification to support 4:2:2 format is proposed.

# Introduction

The HEVC standard has been focusing on its application to new visual services in near-future [1]. Although such target should be important, it is still clear that 4:2:2 format contents would be used in broadcasting and contents production area. In order to apply HEVC standard in the future broadcasting or package business, an option of handling 4:2:2 format in HEVC will significantly contribute to expansion of its market adoption opportunities. Considering this motivation, we propose to start discussion on how to support 4:2:2 format in HEVC in a reasonable way. The proposed approach is to introduce modifications to block-based coding tools as small as possible. In this contribution, we provide a starting point of the specification to support 4:2:2 format.

# 4:2:2 support in HEVC

The proposed modifications to the current specification are listed:

- When chroma_format_idc is equal to 2 which indicates 4:2:2 format, the following modifications are applied.

  - TU height is always 2 times of TU for chroma signals, and other shape for TU chroma is prohibited.

  - Intra prediction is simply extended to support Nx2N size for chroma signals.

  - Coefficient scanning is the same way as in HM-4.0, but the length of scanning is 2 times longer in height.

  - For the vertical MC interpolation filter for chroma, 8 tap filter used for luma is applied.

  - No syntax change except the number of coefficients within chroma TU.

Proposed modification of WD text is attached.

# Conclusion

Modification of the current specification to support 4:2:2 format in HEVC is presented. We propose to begin studying 4:2:2 format support in HEVC upon the proposed modifications so that 4:2:2 format support can be supported by the first version of HEVC standard.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).