| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G970 |
| --- | --- |

| Title: | Cross verification of Some possible motion vector coding related simplifications (JCTVC-G690) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Some possible motion vector coding related simplifications (JCTVC-G690) proposal. The results of the crosscheck matched those reported by I2R.

# Introduction

Combination of part 4 on limiting the number of ‘combined’ merge candidates and part 5 on simplified pruning during addition of ‘combined’ merge candidates of JCTVC-G690 is cross verified [1]. In this part 4 the proposal, different limits are placed on the maximum number of combinations constructed.  In part 5, limitation on combination of merge candidates is imposed where it is allowed only if both L0 and L1 components of the two potential combination candidates are different.

# Results

Results for RA-HE, LB-HE and LP-HE as well as RA-LC, LB-LC and LP-LC were crosschecked, as specified in common test conditions [2]. Table 1 shows summary of the BD-rate results for combination of part 4 and part 5. Encoding as well as decoding was done on a LINUX cluster.

Table 1: BD-rate results for the proposed algorithm.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.1% | 0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.0% | 0.5% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class E | 0.1% | 0.4% | 0.7% | 0.0% | 0.5% | -0.3% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

We have verified results reported in JCTVC-G690  proposal. The results of the crosscheck matched those reported by the proponents. Encoding and decoding times match closely.

# References

[1] Y. H. Tan, C. Yeo, Z. Li, “Some possible motion vector coding related simplifications,” JCTVC-G690, JCT-VC 7th Meeting, Geneva, 2011.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.