| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G974 WG11 Number: m22610 |
| --- | --- |

| Title: | Cross-check for JCTVC-G326: On the exponential memory decay probability update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gergely Korodi, Jinwen Zan, Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 gkorodi@rim.com jzan@rim.com dhe@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This document is a crosscheck report of JCTVC-G326 entitled “On the exponential memory decay probability update”, which modifies the one-parameter version of probability update of proposal JCTVC-F254.  A modified probability update mechanism introducing an offset parameter, together with a reduction in the LPS range table size culminates in a BD-rate gain of 0.47%, 0.57%, 0.53%, 0.48% for modes AI, RA, LB, LP, respectively, compared to HM-4.0.

# Introduction

The G326 source code based on HM-4.0 was provided by Qualcomm. Modifications include:

- TypeDef.h: control macros, including QC_BAC_TABLES, DELTA_V.

- ContextModel.cpp: changes the initialization using parameter DELTA_V.

- TComCABACTables.h and TComCABACTables.cpp: new LPS range definition reduces the table size from 512x64x9 bits (F254) to 64x4x4 bits.

- TEncBinCoderCABAC.cpp: changing the encodeBin procedure to use the new range table

- TDecBinCoderCABAC.cpp: changing the decodeBin procedure to use the new range table

- TEncSbac.cpp: changing the coding bits estimation accordingly

# Results

The tests were run on a 64-bit Linux cluster, on High-Efficiency settings with the common test conditions. The results are summarized in the tables below.

All test points matched that of Qualcomm’s. We did not collect encoding and decoding time. Otherwise, the cross check verifies the results reported in G326.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.58% | -0.56% | -0.58% |
| Class B | -0.49% | -0.82% | -0.81% |
| Class C | -0.47% | -0.94% | -1.01% |
| Class D | -0.49% | -1.53% | -1.61% |
| Class E | -0.46% | -1.17% | -1.19% |
| Class F | -0.33% | -0.43% | -0.30% |
| Overall | -0.47% | -0.89% | -0.90% |
|  | -0.47% | -0.89% | -0.89% |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.71% | -0.18% | 0.33% |
| Class B | -0.57% | -0.70% | -0.76% |
| Class C | -0.52% | -0.80% | -0.79% |
| Class D | -0.63% | -1.18% | -1.38% |
| Class E |  |  |  |
| Class F | -0.40% | -0.33% | -0.45% |
| Overall | -0.57% | -0.64% | -0.62% |
|  | -0.55% | -0.63% | -0.60% |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.49% | -0.69% | -1.15% |
| Class C | -0.49% | -0.90% | -1.03% |
| Class D | -0.72% | -0.95% | -1.33% |
| Class E | -0.52% | -1.62% | -1.69% |
| Class F | -0.43% | -0.71% | -0.01% |
| Overall | -0.53% | -0.93% | -1.01% |
|  | -0.52% | -0.91% | -0.96% |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.49% | -0.39% | -0.62% |
| Class C | -0.46% | -0.58% | -0.77% |
| Class D | -0.55% | -0.77% | -0.96% |
| Class E | -0.62% | -1.88% | -1.94% |
| Class F | -0.32% | -0.19% | -0.17% |
| Overall | -0.48% | -0.69% | -0.83% |
|  | -0.48% | -0.62% | -0.82% |

# Conclusions

This cross-check report confirms the performance of proposal G326.