| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G976 |
| --- | --- |

| Title: | Cross-check of Canon’s modified context selection for significant coefficient flags with diagonal sub-block scan (JCTVC-G958) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution confirms the results presented in JCTVC-G958 on a modified context derivation process for the significance map when the sub-block based scan of JCTVC-G323 is used. The proposed context is parallel friendly and achieves slightly better performance.

- Introduction

Software implementing JCTVC-G958 [1] on top of HM4.0 + JCTVC-G323 [2] has been provided by Canon. The code has been run under common test conditions and results match those provided by Canon.

JCTVC-G323 proposes a scanning process based on a diagonal scan within sub-blocks and a diagonal scan across sub-blocks, like figure 1 shows. This sub-block based scan replaces the large diagonal scans in blocks of sizes 16×16 and 32×32.

Figure 1. Diagonal sub-block scan (sub-block size 4×4)

Along with these scans, a parallel friendly neighborhood is used for the positions that would otherwise use the previous coefficient value to derive the context. These coefficients are the ones at the top-left corner of a sub-block and the ones right above the right-bottom corner of a sub-block.

JCTVC-G958 proposes to compensate for not using the coefficient right below when computing the context of the significance map in the two cases described above. The compensation is done by estimating the value of the unused coefficient. The estimation is obtained by adding the significance of three coefficients in the neighborhood (below, right and diagonal top-right) and a bit right-shift by 1. JCTVC-G323 assumes that the value of the unused coefficient is zero.

- Results

Coding results using common conditions (High-Efficiency settings) are summarized in the table below. HM4.0 is the anchor. JCTVC-G958 uses the case of sub-block size of 4×4. Note that encoding/decoding times are not reliable since different types of machines were used.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.31% | -0.11% | 0.09% | 0.29% | 0.47% |  |  |  |
| Class B | 0.03% | 0.02% | 0.04% | 0.04% | -0.03% | 0.07% | 0.06% | -0.05% | -0.11% |
| Class C | -0.02% | 0.00% | 0.00% | -0.04% | -0.11% | 0.09% | 0.01% | -0.05% | 0.05% |
| Class D | -0.01% | -0.04% | -0.07% | 0.00% | 0.02% | -0.32% | 0.00% | 0.07% | -0.32% |
| Class E | -0.06% | -0.15% | -0.17% |  |  |  | 0.07% | 0.04% | 0.40% |
| Class F | -0.03% | -0.02% | -0.04% | 0.01% | 0.07% | -0.16% | -0.16% | -0.47% | 0.12% |
| Overall | -0.02% | -0.08% | -0.05% | 0.02% | 0.04% | 0.03% | -0.01% | -0.10% | 0.00% |
|  | -0.02% | -0.09% | -0.08% | 0.02% | 0.02% | 0.01% | -0.01% | -0.10% | -0.01% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 105% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  | 94% |  |  |

- References

- C. Rosewarne, M. Maeda, “Non-CE11: Modified context selection for Significant Coefficient Flags with Diagonal sub-block scan”, JCTVC-G958, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for residual coding”, JCTVC-G323, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.