| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November, 2011 | Document: JCTVC-G977 |
| --- | --- |

| Title: | nonCE10: new 8-bits core spatial transform with fast algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Email: | hurricanezhb@hotmail.com |
| Source: | None. |  |  |

_____________________________

# Abstract

New 8x8, 16x16 and 32x32 core spatial transforms denoted by 8-bits signed integer are proposed in this document. The odd part of the current HM 8-bits transform use matrix multiplication, it is complex. The proposal modifies the transform matrix of the odd part to new 8-bits matrix so that it is fast algorithm friendly.

# New 8-bits Core Spatial Transform

The HM 8-bits core spatial transform has the following properties:

- 16-bits depth for all intermediate buffer

- Uniform quantization design for all size transform

- 16-bits multipliers for all multiplications

- two implementation methods: matrix multiplication directly or partial butterfly

- 8 bit representation of transform coefficients

- small transform is part of bigger transforms

In addition, the 8 bit representation could give another advantage over high bit-depth transform matrix: apply the entire 16-bits computation for the transform process when the block difference or de-quantized DCT coefficients are small. But as the restriction of the 8-bits representation, it is very difficult to give a full butterfly factorization for the 8-bits transform matrix. The odd parts of the current design only have one implementation: matrix multiplication.

The proposed new transform and its corresponding inv-transform are shown in figure 1 and 2. The transform matrixes are listed in figure 3, 4 and 5.

Fig. 1: the proposed forward transform

Fig. 2: The proposed inverse transform

64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 6490, 89, 88, 86, 81, 77, 73, 68, 60, 53, 48, 40, 29, 20, 15,  8, -8,-15,-20,-29,-40,-48,-53,-60,-68,-73,-77,-81,-86,-88,-89,-9090, 86, 80, 69, 58, 42, 27,  8, -8,-27,-42,-58,-69,-80,-86,-90,-90,-86,-80,-69,-58,-42,-27, -8,  8, 27, 42, 58, 69, 80, 86, 9090, 82, 68, 47, 22, -6,-31,-52,-74,-85,-90,-88,-77,-60,-38,-13, 13, 38, 60, 77, 88, 90, 85, 74, 52, 31,  6,-22,-47,-68,-82,-9088, 77, 48, 20,-20,-48,-77,-88,-88,-77,-48,-20, 20, 48, 77, 88, 88, 77, 48, 20,-20,-48,-77,-88,-88,-77,-48,-20, 20, 48, 77, 8888, 66, 29,-13,-54,-81,-90,-77,-48, -4, 40, 72, 90, 86, 61, 22,-22,-61,-86,-90,-72,-40,  4, 48, 77, 90, 81, 54, 13,-29,-66,-8886, 58,  8,-42,-80,-90,-70,-27, 27, 70, 90, 80, 42, -8,-58,-86,-86,-58, -8, 42, 80, 90, 70, 27,-27,-70,-90,-80,-42,  8, 58, 8685, 47,-14,-66,-90,-73,-22, 37, 83, 88, 54, -3,-62,-89,-78,-30, 30, 78, 89, 62,  3,-54,-88,-83,-37, 22, 73, 90, 66, 14,-47,-8583, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 8381, 25,-56,-90,-60, 11, 79, 85, 30,-44,-90,-68,  5, 71, 87, 41,-41,-87,-71, -5, 68, 90, 44,-30,-85,-79,-11, 60, 90, 56,-25,-8179, 11,-71,-87,-24, 55, 90, 45,-45,-90,-55, 24, 87, 71,-11,-79,-79,-11, 71, 87, 24,-55,-90,-45, 45, 90, 55,-24,-87,-71, 11, 7978, -3,-82,-73, 13, 85, 66,-20,-88,-62, 30, 90, 53,-39,-90,-47, 47, 90, 39,-53,-90,-30, 62, 88, 20,-66,-85,-13, 73, 82,  3,-7874,-16,-89,-52, 52, 89, 16,-74,-74, 16, 89, 52,-52,-89,-16, 74, 74,-16,-89,-52, 52, 89, 16,-74,-74, 16, 89, 52,-52,-89,-16, 7472,-30,-90,-23, 78, 69,-39,-89,-14, 82, 59,-45,-88, -7, 85, 55,-55,-85,  7, 88, 45,-59,-82, 14, 89, 39,-69,-78, 23, 90, 30,-7270,-43,-87,  9, 90, 26,-80,-57, 57, 80,-26,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-26, 80, 57,-57,-80, 26, 90,  9,-87,-43, 7064,-47,-72, 33, 83,-13,-88, -1, 90, 21,-90,-36, 84, 54,-78,-64, 64, 78,-54,-84, 36, 90,-21,-90,  1, 88, 13,-83,-33, 72, 47,-6464,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 6464,-78,-54, 84, 36,-90,-21, 90,  1,-88, 13, 83,-33,-72, 47, 64,-64,-47, 72, 33,-83,-13, 88, -1,-90, 21, 90,-36,-84, 54, 78,-6457,-79,-26, 90, -9,-87, 43, 70,-70,-43, 87,  9,-90, 26, 79,-57,-57, 79, 26,-90,  9, 87,-43,-70, 70, 43,-87, -9, 90,-26,-79, 5755,-85, -7, 88,-45,-59, 82, 14,-89, 39, 69,-78,-23, 90,-30,-72, 72, 30,-90, 23, 78,-69,-39, 89,-14,-82, 59, 45,-88,  7, 85,-5552,-89, 16, 74,-74,-16, 89,-52,-52, 89,-16,-74, 74, 16,-89, 52, 52,-89, 16, 74,-74,-16, 89,-52,-52, 89,-16,-74, 74, 16,-89, 5247,-90, 39, 53,-90, 30, 62,-88, 20, 66,-85, 13, 73,-82,  3, 78,-78, -3, 82,-73,-13, 85,-66,-20, 88,-62,-30, 90,-53,-39, 90,-4745,-90, 55, 24,-87, 71, 11,-79, 79,-11,-71, 87,-24,-55, 90,-45,-45, 90,-55,-24, 87,-71,-11, 79,-79, 11, 71,-87, 24, 55,-90, 4541,-87, 71, -5,-68, 90,-44,-30, 85,-79, 11, 60,-90, 56, 25,-81, 81,-25,-56, 90,-60,-11, 79,-85, 30, 44,-90, 68,  5,-71, 87,-4136,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 3630,-78, 89,-62,  3, 54,-88, 83,-37,-22, 73,-90, 66,-14,-47, 85,-85, 47, 14,-66, 90,-73, 22, 37,-83, 88,-54, -3, 62,-89, 78,-3027,-70, 90,-80, 42,  8,-58, 86,-86, 58, -8,-42, 80,-90, 70,-27,-27, 70,-90, 80,-42, -8, 58,-86, 86,-58,  8, 42,-80, 90,-70, 2722,-61, 86,-90, 72,-40, -4, 48,-77, 90,-81, 54,-13,-29, 66,-88, 88,-66, 29, 13,-54, 81,-90, 77,-48,  4, 40,-72, 90,-86, 61,-2220,-48, 77,-88, 88,-77, 48,-20,-20, 48,-77, 88,-88, 77,-48, 20, 20,-48, 77,-88, 88,-77, 48,-20,-20, 48,-77, 88,-88, 77,-48, 2013,-38, 60,-77, 88,-90, 85,-74, 52,-31,  6, 22,-47, 68,-82, 90,-90, 82,-68, 47,-22, -6, 31,-52, 74,-85, 90,-88, 77,-60, 38,-13 8,-27, 42,-58, 70,-80, 86,-90, 90,-86, 80,-70, 58,-42, 27, -8, -8, 27,-42, 58,-70, 80,-86, 90,-90, 86,-80, 70,-58, 42,-27,  8

Fig. 5 the proposed 32x32 transform

A00A01A02

The matrix multiplication in fig. 6 could be computed as follows using 26 additions and some shift operations (multiplication is treated as addition and shift):

B00 = -128*x00 + 255*x02;

B01 = -68*x00 +   0*x02;

B02 =  10*x00 -  25*x02;

B03 =  16*x00 -  16*x02;

B02 =   4*B03 + B02;

B01 =   1*B03 + B01;

C00 =  16*x01 -  16*x03;

C01 =  -9*x01 -   6*x03;

C02 =   0*x01 +  68*x03;

C03 = 255*x01 - 128*x03;

C02 =   1*C00 + C02;

C01 =  -5*C00 + C01;

A00 = B00 + C00;

A01 = B01 + C01;

A02 = B02 + C02;

A03 = B03 + C03;

A03 =   2*A01 + A03;

A00 =   2*A02 + A00;

The matrix multiplication in fig. 7 could be computed as follows using 103 additions and some shifts:

C00 =   4*C04 + C00;

C06 =  -2*C00 + C06;

D00 =  -2*x03 +   0*x06;

D01 =   8*x03 -  24*x06;

D02 =   0*x03 -  32*x06;

D03 =  33*x03 + 128*x06;

D04 =  -1*x03 -  64*x06;

D05 = -36*x03 -   4*x06;

D06 = -16*x03 +   2*x06;

D07 =  32*x03 +   1*x06;

D05 =  -1*D01 + D05;

D02 =  -2*D07 + D02;

D00 =  -2*D06 + D00;

D07 =  -1*D02 + D07;

B00 = C00 + D00;

B01 = C01 + D01;

B02 = C02 + D02;

B03 = C03 + D03;

B04 = C04 + D04;

B05 = C05 + D05;

B06 = C06 + D06;

B07 = C07 + D07;

B06 =   1*B04 + B06;

B03 =  -1*B00 + B03;

B01 =   4*B04 + B01;

B05 =  -2*B00 + B05;

F00 =  -1*x01 +   2*x02;

F01 =  -2*x01 +   0*x02;

F02 = -16*x01 -   4*x02;

F03 =   0*x01 -  80*x02;

F04 =   4*x01 - 256*x02;

F05 =  -8*x01 -   1*x02;

F06 =   0*x01 +   2*x02;

F07 =   2*x01 - 122*x02;

F06 =  32*F00 + F06;

F05 =   1*F02 + F05;

F01 =  -1*F04 + F01;

F04 =  -2*F02 + F04;

F03 =   3*F05 + F03;

F00 =   2*F02 + F00;

F02 =   2*F00 + F02;

G00 =  18*x05 +   0*x07;

G01 =  -8*x05 +   0*x07;

G02 =  -2*x05 -  34*x07;

G03 = -16*x05 +   0*x07;

G04 =  -4*x05 +  28*x07;

G05 =   0*x05 -  65*x07;

G06 =  -1*x05 - 107*x07;

G07 = 128*x05 -   4*x07;

G06 =  -1*G00 + G06;

G05 =   2*G00 + G05;

G03 =   4*G04 + G03;

G04 =   2*G05 + G04;

G01 =   2*G02 + G01;

G02 =   1*G07 + G02;

G00 =   2*G01 + G00;

E00 = F00 + G00;

E01 = F01 + G01;

E02 = F02 + G02;

E03 = F03 + G03;

E04 = F04 + G04;

E05 = F05 + G05;

E06 = F06 + G06;

E07 = F07 + G07;

E02 =  -1*E03 + E02;

E00 =  -1*E07 + E00;

E07 =   1*E01 + E07;

E04 =  -1*E03 + E04;

A00 = B00 + E00;

A01 = B01 + E01;

A02 = B02 + E02;

A03 = B03 + E03;

A04 = B04 + E04;

A05 = B05 + E05;

A06 = B06 + E06;

A07 = B07 + E07;

A01 =   2*A03 + A01;

A05 =  -1*A06 + A05;

A04 =   1*A01 + A04;

A03 =  -1*A05 + A03;

A02 =   1*A03 + A02;

A07 =  -1*A02 + A07;

A00 =   1*A05 + A00;

A01 =  -1*A03 + A01;

A06 =  -1*A02 + A06;

The matrix multiplication in fig. 8 could be computed as shown in 16x16.txt using 406 additions and some shifts.

A00A01A02A03A04A05A06

A00A01A02A03A04A05A06A07A08A09A10A11A12A13A14

# Simulation Results

The HM4.0 r1354 is used as the platform to test the proposed transform. The simulation experiments were conducted following the testing conditions in [1]. The encoding/decoding times were estimated using two PC listed in table 1. The results are listed in table 2.

| CPU | AMD Phenom(tm) II x6 1075T@3.0GHz (6 cores) | AMD bulldozer FX-8150 @3.6GHz (8 cores) |
| --- | --- | --- |
| DRAM | 6G 1333MHz | 8G 1333MHz |
| BUS | 1333MHz | 4.8GT/s |
| HardDisk | WD 1TB | WD 512GB |
| OS | Win7x64 | Win7x64 |
| EXE | X64 version (VC2008) | X64 version (VC2008) |

Table 1: the computers to generate the results

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono1 | 0.7% | 0.1% | 0.0% |
| ParkScene | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Kimono1 | 0.3% | 0.0% | 0.0% |
| ParkScene | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.1% | 0.2% |
| Class E |  |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Kimono1 | 0.1% | 0.2% | -0.2% |
| ParkScene | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.5% | 0.0% |
| Class E | 0.0% | -0.4% | -0.4% |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Kimono1 | 0.1% | 0.2% | -0.3% |
| ParkScene | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% |
| Class E | 0.1% | -0.9% | -0.9% |

Table 2: Results of the proposed transform

# Conclusions

The algorithm is on its early development stage, it is expected that the algorithm is improved greatly in the continuing research. It is suggested to do further study of the algorithm in the CE 10.

# Reference

[1] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).