| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G979 |
| --- | --- |

| Title: | non-CE4: Crosscheck of Fine granularity QP offset (JCTVC-G850) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a cross-check report for fine granularity QP offset (JCTVC-G850). It is reported that the proposed fine granularity brings about 0.9%, 1.6%, 1.3%, and 2.0% in luma bit rate savings for RA_HE, RA_LC, LB_HE, and LB_LC respectively, and that these measurements match the results reported by the proponent. In addition to the tests performed according to the suggested by the proponent, additional testing results and code study are also provided to analyze the properties of the fine granularity and its use by the encoder. It is reported that most of the measured gain is actually from non-normative modifications rather than from the effect of the normative aspects of the proposal.

# Introduction

This document reports cross-check result for fine granularity QP offset Error: Reference source not found. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

- Anchor: HM-4.0, Test: AQO2, QPR3

Table 1 Summary results of AQO2 QPR3 (BD-Rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | -6.4% | -6.9% | -2.2% | -7.7% | -8.2% |
| Class B | -0.8% | -7.2% | -7.7% | -1.5% | -7.5% | -7.9% |
| Class C | -1.2% | -5.6% | -5.3% | -1.9% | -6.2% | -5.8% |
| Class D | -0.4% | -5.5% | -5.6% | -1.1% | -5.8% | -5.7% |
| Class E |  |  |  |  |  |  |
| Overall | -0.9% | -6.2% | -6.4% | -1.6% | -6.8% | -6.9% |
|  | -0.6% | -6.0% | -6.2% | -1.6% | -6.7% | -6.7% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.8% | -4.6% | -4.9% | -2.5% | -5.6% | -5.6% |
| Class C | -1.7% | -4.5% | -4.4% | -2.6% | -5.1% | -5.2% |
| Class D | -1.1% | -3.7% | -3.6% | -1.6% | -4.9% | -4.8% |
| Class E | -0.3% | -2.3% | -3.1% | -0.8% | -5.0% | -5.8% |
| Overall | -1.3% | -3.9% | -4.1% | -2.0% | -5.2% | -5.3% |
|  | -1.3% | -3.9% | -4.1% | -2.0% | -5.1% | -5.2% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |

- Anchor: HM-4.0, Test: AQO2, QPR1

Table 2 Summary results of AQO2 QPR1 (BD-Rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5% | -2.0% | -2.1% | -2.4% | -2.8% | -3.1% |
| Class B | -1.1% | -2.3% | -2.7% | -1.9% | -2.3% | -2.5% |
| Class C | -1.4% | -1.9% | -1.9% | -2.2% | -2.0% | -1.7% |
| Class D | -0.7% | -1.1% | -1.8% | -1.4% | -0.8% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | -1.2% | -1.9% | -2.2% | -2.0% | -2.0% | -2.1% |
|  | -0.9% | -1.7% | -2.0% | -1.9% | -1.8% | -1.9% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9% | -2.4% | -2.3% | -2.8% | -1.1% | -1.0% |
| Class C | -1.9% | -2.2% | -2.1% | -3.0% | -1.1% | -1.2% |
| Class D | -1.3% | -1.5% | -1.6% | -1.9% | 0.6% | 0.6% |
| Class E | -0.3% | -0.1% | -0.3% | -1.1% | 0.9% | 0.5% |
| Overall | -1.4% | -1.7% | -1.7% | -2.3% | -0.3% | -0.3% |
|  | -1.5% | -1.6% | -1.6% | -2.3% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |

Note: large loss on UV is observed for LB_LC, BQSquare

- Anchor: HM-4.0 with low QP (2~17), Test: AQO2, QPR0 with low QP (2~17)

Table 3 Summary results of AQO2 QPR0, Low QP (2~17) (BD-Rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.4% | -1.6% | -1.2% | -1.4% | -1.6% |
| Class B | -0.4% | -2.4% | -3.9% | -1.3% | -3.1% | -3.9% |
| Class C | -0.5% | -2.6% | -2.8% | -1.5% | -2.7% | -2.8% |
| Class D | -0.5% | -2.8% | -2.7% | -1.5% | -3.2% | -3.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -2.3% | -2.8% | -1.3% | -2.6% | -2.9% |
|  | -0.5% | -1.9% | -2.5% | -1.3% | -2.5% | -2.8% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | -1.9% | -0.7% | -0.2% | -1.1% |
| Class C | -0.1% | -0.4% | -1.2% | -0.8% | -0.4% | -1.0% |
| Class D | -0.2% | -0.8% | -1.1% | -1.0% | -1.3% | -1.6% |
| Class E | -0.4% | -1.2% | -1.4% | -0.6% | -2.1% | -2.2% |
| Overall | -0.1% | -0.6% | -1.4% | -0.8% | -0.9% | -1.4% |
|  | -0.1% | -0.4% | -1.3% | -0.9% | -0.3% | -0.7% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

The above three sets of results match those from the proponent.

# Code study and some other tests

There are many modifications in the software provided by the proponent.

- SYNTAX_CHROMA_QPIDX_OFFSET

If this macro is defined, the syntax element of chroma_qp_idx_offset is sent at the slice header. We also think there should be some similar syntax element to signal the chroma QP offset as what H.264/MPEG-4 AVC does. (Maybe they should be signaled at some other places rather than slice header.) We planned to test the case when this macro is defined as 0, but we met many many compiling errors when it is defined as 0, and we had to give up this attempt.

In H.264/MPEG-4 AVC, there are some similar syntax elements chroma_qp_index_offset and second_chroma_qp_index_offset at PPS level having similar functionalities.

As there should be some similar kinds of syntax element to signal the chroma QP offset (although missing in both HM-4.0 and WD4 currently), the code relating to this macro is not treated as normative change in our crosschecking report.

- QPROUNDING

If this macro is defined, the QP of each slice is determined by the coefficients before and after quantization in the slices belonging to the same temporal level. These modifications are done without normative changes.

But as described in [1], the default QP rounding parameter for luma in inter frames is set to be 19 (in the range of QP+1), while for chroma in inter frames it is set to −10 (in the range of QP). It means that for luma, it tends to use larger QP but for chroma, it tends to use equal or smaller QPs. That is also part of the reason that the bits saving of chroma is much larger than that of luma. It relates to the bits allocation between luma and chroma.

- MTK_AQO

This macro related to the adaptive dequantization offset. In our test, we modify the code to disable the modifications of the quantization and dequantization. It means that all the normative changes brought by MTK_AQO are disabled.

dij = ( cij * LevelScale(nS)x(nS)[ qP%6 ][ i ][ j ] * (128+delta) << ( qP/6 + trafoPrecisionExt-7)

The highlighted parts were modified back to the same with HM-4.0.

The summary results are shown in the table below.

Anchor: HM-4.0, Test: disabling the normative changes brought by MTK_AQO

Table 4 Summary results of AQO2 QPR3 without normative changes by MTK_AQO (BD-Rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -5.5% | -6.1% | -1.3% | -7.3% | -7.6% |
| Class B | -0.3% | -6.4% | -6.9% | -0.9% | -6.8% | -7.2% |
| Class C | -0.7% | -5.0% | -4.8% | -1.3% | -5.6% | -5.4% |
| Class D | -0.5% | -5.0% | -5.5% | -0.9% | -5.5% | -5.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -5.5% | -5.9% | -1.1% | -6.4% | -6.4% |
|  | -0.2% | -5.3% | -5.6% | -1.1% | -6.2% | -6.2% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -4.2% | -4.6% | -1.9% | -4.8% | -5.0% |
| Class C | -1.4% | -3.8% | -4.0% | -2.2% | -4.4% | -4.4% |
| Class D | -1.1% | -3.6% | -4.1% | -1.4% | -3.9% | -4.2% |
| Class E | -0.4% | -3.3% | -3.1% | -0.9% | -5.2% | -5.4% |
| Overall | -1.1% | -3.8% | -4.0% | -1.7% | -4.5% | -4.7% |
|  | -1.2% | -3.8% | -4.0% | -1.7% | -4.5% | -4.7% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |

According to our understanding, for the proposed fine granularity QP offset, the “fine granularity” is mainly because of MTK_AQO, as the dequantization offset is provided by MTK_AQO. With the help of the dequantization offset, the dequantization offset is “fine granularity”. But as the results in Table 4, most of the gain does not come from the “fine granularity” (The dequanzation offset was disabled in the results in Table 4).

It seems that most gain of the proposed method comes from QPROUNDING (non-normative changes) and SYNTAX_CHROMA_QPIDX_OFFSET (we also treated those code as non-normative changes as the corresponding syntax elements (maybe at some other places) should be in HEVC but currently missing). The benefit brought by MTK_AQO is relatively small.

Besides the above code study, there are some other aspects need to be explained or clarified in more details.

- The increasing of the encoding and decoding time.

- The gain brought by the normative changes.

- The behavior without SYNTAX_CHROMA_QPIDX_OFFSET

# References

- JCT-VC, “Non CE4: Fine granularity QP offset”, JCTVC-G850, JCT-VC Meeting, Geneva, CH, Nov. 2011.