| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G982 WG11 Number: m22639 |
| --- | --- |

| Title: | Non-CE12 Subtest 5: Cross-check of Qualcomm’s transform dependent deblocking filter parameter adjustment in slice level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Saori Asaka | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-check results of JCTVC-G291 (Non-CE12 Subtest 5: Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level) are reported.  Coding efficiency results are exactly matched for the common test conditions.  Relative encoding and decoding time are similar to those of proponent’s.  Source code was reviewed and confirmed the code was consistent with the description in JCTVC-G291.

# Introduction

This contribution is a report of cross-check results of JCTVC-G291.

Cross-check is performed under the CE12 test conditions and additional higher Qp range conditions that refer to JCTVC-F900 and JCTVC-F912.  Coding efficiency results are matched with the proponent’s results for the common test conditions.  For higher Qp range, the simulation is still running.  Relative encoding and decoding time are similar to those of proponent’s.

Source code was reviewed and confirmed that it was consistent with the description in JCTVC-G291.

# Experimental Results

Detailed results in Excel sheet are attached.  The following tables are the summary of experimental results:

Table 1  Common test conditions

| Test case | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| Class A | -0.3% | -0.2% |  |  | 0.0% | 0.0% |  |  |
| Class B | -0.3% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class C | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class D | -0.2% | 0.1% | 0.2% | 0.2% | -0.1% | 0.1% | 0.1% | 0.2% |
| Class E | -0.2% |  | 0.1% | 0.2% | 0.0% |  | 0.0% | 0.3% |
| Average | -0.3% | -0.1% | 0.1% | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Enc time | 101% | 104% | 105% | 104% | 100% | 105% | 105% | 104% |
| Dec time | 101% | 100% | 100% | 100% | 100% | 101% | 101% | 101% |

Table 2  Higher Qp test conditions (Qp = 32, 37, 42, 47)

| Test case |  | High Efficiency |  |  |  |  |  |  |  | Low Complexity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO |  | RA |  | LDB |  | LDP |  | IO |  | RA |  | LDB |  | LDP |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

There is no mismatch between encoder and both decoders.  Relative encoding and decoding time are similar to those of proponent’s.

# Conclusion

This contribution shows cross-check results of JCTVC-G291. Coding efficiency results are confirmed for the common test conditions.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.  Source code was reviewed.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).