| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G988 |
| --- | --- |

| Title: | HEVC software player Demonstration on Mobile devices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ken McCann (Samsung/ZetaCast) Jeong-Yong Choi Kulbhushan Pachauri Kwang Pyo Choi Chanyul Kim Youngo Park Young Jin Kwak Sunyoung Jun Minseok Choi Heechul Yang JeongHoon Park | Tel: Email: | ken@zetacast.com eldragon.choi@samsung.com kb.pachauri@samsung.com kp5.choi@samsung.com dionism@samsung.com youngo.park@samsung.com yj2005.kwak@samsung.com sunyoung.jeon@samsung.com minseok48.choi@samsung.com h_c.yang@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Real-time decoding is the key feature for most applications. This document describes an HEVC software decoder running on commercial devices such as mobile phones and x86. The software decoder is implemented based on Samsung’s commercial media player framework, which is already loaded on various PC and mobile devices. This HEVC software decoder on mobile devices, together with a file container and android media player, is reported to be capable of running on current commercial devices (the Galaxy Note and Galaxy S made by Samsung).  The results are broadly consistent with the complexity and profiling measurement using a low complexity decoder that is reported in JCTVC-G757, providing confidence that the current HEVC design using CABAC could be commercially implemented at an early date.

# Introduction

The software decoder contains tools supported by the HM4.0 except for NSQT, ALF and CAVLC as shown in Table 1.

Table 1. Including tools in software decoder compared to HM-4.0

| Tools | HM-4.0* | Real time decoder |
| --- | --- | --- |
| CABAC | O | O |
| NSQT/AMP | O/O | X/O |
| Deblock filter | O | O |
| ALF | O | X |
| SAO | O | O |

The following decoding rates are achieved with Single Instruction Multiple Data (SIMD), MMX/SSE on x86 to improve decoding time, which are the similar result of [1].

- 1920x1080@60fps on a single core of Core i5 750 (Nehalem) clocked at 2.66GHz.

# Profiling

Profiling of software decoder on x86 was performed using Intel Parallel Amplifier 2011. The configuration is RA and QP is 32.

# Playback with Software Decoder on Mobile devices

The HEVC software player can be demonstrated running on a mobile device. Pre-encoded bitstreams are used for playback on this device.

- Samsung Galaxy Note, (OS: Android 2.3.6, a.k.a. Gingerbread, Processor: ARM Cortex-A9)

- File container : Handling of fps, running time, codec type

- Media player including renderer

- Running on single core of ARM Cotex-A9

HEVC playback demonstration can be made during the JCT-VC meeting.

# Conclusion

Samsung implemented an HEVC software decoder on x86 and ARM Cortex-A9. The results are broadly consistent with those of [1], confirming that the current HEVC design using CABAC could be commercially implemented at an early date.

# References

- Frank Bossen, “On software complexity”, JCTVC-G757, JCT-VC 7rd Meeting, Geneva, November, 2010.