| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G990 |
| --- | --- |

| Title: | Cross check of inverse transform complexity reported in JCTVC-G757 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution presents a cross-check of the HM4.0 inverse transform complexity reported in JCTVC-G757.

# Introduction

In document JCTVC-G757, the cycle counts for a SIMD-optimized implementation of the HM4.0 inverse transforms (using partial butterfly) are reported. The measurements are done an Intel Core i5 M540 processor (Nehalem microarchitecture). The cycle counts reported in JCTVC-G757 are as follows:

- 8x8 inverse transform:		158 cycles

- 16x16 inverse transform:	861 cycles

- 32x32 inverse transform:	4696 cycles

# Results

The inverse 32x32 transform implementation of JCTVC-G757 was compiled and run on an Intel Core i7 (Sandy Bridge) computer. Using two different compiler options, the results are as follows:

- 32x32 inverse transform, SSE compiler option:	4732 cycles

- 32x32 inverse transform, AVX compiler option:	3974 cycles

We believe the differences are reasonable considering differences in CPU, instruction set, and compiler.