| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G997 |
| --- | --- |

| Title: | Non-CE11: Cross-check of JCTVC-G448 on context reduction for coding transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin Joel Sole 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-3481 vseregin@qualcomm.com joels@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a cross-check report of JCTVC-G448 related to context reduction for coding transform coefficients.

# Introduction

Software of JCTVC-G448 [1], implemented on top of HM4.0, was released by a proponent.

The code is matching the description in the document.

The code change is very localized and found only in one function. The significance map context derivation for 4x4 and 8x8 blocks is changed so that the high frequencies share more contexts than currently are doing, so the number of total contexts is reduced. There is a little increase in the logic to derive the context to attain this reduction.

The initial probabilities for the contexts are kept the same. The context reduction is not reflected in the ContextTable file, where these context have to be removed as well.

# Results

Simulation results under common test conditions with HM4.0 as an anchor are summarized in the below tables 1-4. Encoding and decoding time may have inaccuracy since software was executed on non-uniform cluster. PSNR and bitrate are matched with proponent’s data. Additionally to common test configuration Qp, low Qp 12, 17, 22 and 27 set is tested as well.

Table 1. BR6 case

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.07% | -0.02% |
| Class B | 0.04% | 0.04% | 0.02% |
| Class C | 0.04% | 0.05% | 0.06% |
| Class D | 0.03% | 0.01% | 0.08% |
| Class E | 0.01% | -0.03% | 0.06% |
| Overall | 0.03% | 0.01% | 0.04% |
|  | 0.03% | 0.01% | 0.03% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.02% | -0.09% | -0.14% |
| Class B | 0.02% | 0.20% | -0.03% |
| Class C | 0.05% | 0.09% | 0.04% |
| Class D | 0.07% | 0.13% | -0.07% |
| Class E |  |  |  |
| Overall | 0.04% | 0.09% | -0.05% |
|  | 0.04% | 0.08% | -0.03% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.02% | -0.15% | -0.34% |
| Class C | 0.06% | -0.19% | -0.09% |
| Class D | 0.09% | 0.33% | 0.23% |
| Class E | 0.10% | -0.01% | 0.05% |
| Overall | 0.06% | -0.01% | -0.06% |
|  | 0.06% | -0.01% | -0.14% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2. BR6 case low qp

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.01% | -0.01% |
| Class B | 0.08% | 0.07% | 0.06% |
| Class C | 0.07% | 0.06% | 0.07% |
| Class D | 0.01% | 0.00% | 0.02% |
| Class E | 0.02% | -0.05% | 0.03% |
| Overall | 0.04% | 0.02% | 0.04% |
|  | 0.04% | 0.02% | 0.03% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | -0.03% | 0.10% |
| Class B | 0.07% | 0.09% | -0.06% |
| Class C | 0.08% | 0.09% | 0.10% |
| Class D | 0.07% | 0.13% | 0.01% |
| Class E |  |  |  |
| Overall | 0.06% | 0.07% | 0.03% |
|  | 0.06% | 0.08% | 0.04% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.06% | 0.06% | 0.02% |
| Class C | 0.08% | -0.01% | 0.07% |
| Class D | 0.05% | -0.04% | 0.00% |
| Class E | 0.03% | -0.28% | -0.03% |
| Overall | 0.06% | -0.04% | 0.02% |
|  | 0.07% | -0.04% | 0.00% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3. BR4 case

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.03% | 0.01% |
| Class B | 0.02% | 0.01% | -0.01% |
| Class C | 0.01% | 0.05% | -0.03% |
| Class D | 0.01% | 0.00% | -0.01% |
| Class E | 0.00% | 0.01% | 0.00% |
| Overall | 0.01% | 0.01% | -0.01% |
|  | 0.01% | 0.01% | -0.01% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | -0.17% | 0.02% |
| Class B | -0.03% | 0.10% | -0.04% |
| Class C | 0.02% | 0.09% | -0.06% |
| Class D | 0.04% | 0.07% | 0.01% |
| Class E |  |  |  |
| Overall | 0.01% | 0.03% | -0.02% |
|  | 0.01% | 0.00% | -0.03% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | -0.10% | -0.21% |
| Class C | 0.04% | 0.08% | -0.02% |
| Class D | 0.01% | 0.12% | -0.15% |
| Class E | 0.04% | 0.16% | 1.06% |
| Overall | 0.02% | 0.05% | 0.09% |
|  | 0.02% | 0.02% | 0.06% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |

Table 4. BR4 case low qp

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | 0.00% | 0.01% |
| Class B | 0.05% | 0.03% | 0.01% |
| Class C | 0.01% | 0.02% | 0.03% |
| Class D | 0.00% | -0.01% | -0.02% |
| Class E | 0.01% | 0.00% | 0.01% |
| Overall | 0.02% | 0.01% | 0.01% |
|  | 0.02% | 0.01% | 0.00% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | 0.05% | 0.04% |
| Class B | 0.03% | 0.00% | -0.06% |
| Class C | 0.03% | 0.03% | 0.04% |
| Class D | 0.02% | 0.15% | -0.07% |
| Class E |  |  |  |
| Overall | 0.02% | 0.05% | -0.02% |
|  | 0.02% | 0.05% | -0.01% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.03% | 0.00% | -0.03% |
| Class C | 0.03% | -0.01% | -0.04% |
| Class D | 0.01% | -0.03% | 0.01% |
| Class E | 0.01% | -0.56% | 0.13% |
| Overall | 0.02% | -0.12% | 0.01% |
|  | 0.02% | -0.14% | -0.03% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 104% |  |  |

- References

- S.-T. Hsiang and S. Lei, “Non-CE11: Context reduction for coding transform coefficients”, JCTVC-G448, 7th JCT-VC Meeting, Geneva, CH, November 2011.