| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0002 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC Draft and Test Model editing (AHG 2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Ken McCann (Samsung/ZetaCast) Woo-Jin Han (Gachon University) Il-Koo Kim (Samsung) Jens-Rainer Ohm (RWTH Aachen) Shun-ichi Sekiguchi (Mitsubishi) Gary J. Sullivan (Microsoft) Thomas Wiegand (Fraunhofer HHI) | Email: | benjamin.bross@hhi.fraunhofer.de ken@zetacast.com hurumi@gmail.com ilkoo.kim@samsung.com ohm@ient.rwth-aachen.de Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Draft and Test Model editing (AHG2) between the 7th JCT-VC meeting in Geneva (20-30 November, 2011) and the 8th JCT-VC meeting in San José (1-10 February, 2012).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-G1102 HEVC Test Model 5 (HM 5) Encoder Description - Produce and finalize JCTVC-G1103 HEVC text specification Working Draft 5 - Gather and address comments for refinement of these documents - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text | B. Bross K. McCann (co-chairs) W.-J. Han I.-K. Kim J.-R. Ohm S. Sekiguchi G. J. Sullivan T. Wiegand (vice chairs) | No |

# Background

The fifth High Efficiency Video Coding (HEVC) test model (HM5) was developed from the fourth HEVC test model (HM4), following the decisions taken at the 7th JCT-VC meeting in Geneva (21-30 November, 2011).

# Method of Working

Two editorial teams were formed to work on the two documents that were to be produced:

- JCTVC-G1102 HEVC Test Model 5 (HM 5) Encoder Description [1]

  - Il-Koo Kim

  - Shun-ichi Sekiguchi

  - Benjamin Bross

  - Woo-Jin Han

  - Ken McCann

- JCTVC-G1103 WD5: Working Draft 5 of High Efficiency Video Coding [2]

  - Benjamin Bross

  - Woo-Jin Han

  - Jens-Rainer Ohm

  - Gary J. Sullivan

  - Thomas Wiegand

Editing JCTVC-G1103 was assigned a higher priority than editing JCTVC-G1102.

An issue tracker (http://hevc.kw.bbc.co.uk/trac) was created in order to facilitate the reporting issues on the text of both documents.

# Status

One draft of JCTVC-G1102 and eight drafts of JCTVC-G1103 were published by the Editing AHG between the 7th JCT-VC Meeting in Geneva (21-30 November, 2011) and the 8th JCT-VC Meeting in San José (1-10 February, 2012).  JCTVC-G1102 still needs further improvement, whilst the final draft of JCTVC-G1103 is reasonably complete.

The main changes in JCTVC-G1103, relative to the previous JCTVC-F803, were:

- Removed CAVLC

- Incorporated weighted prediction (JCTVC-F265)

- Incorporated wavefront parallel processing (JCTVC-F274)

- Incorporated wavefront CABAC flush (JCTVC-F275)

- Incorporated tiles (JCTVC-F335)

- Removed ClipMv (JCTVC-G134)

- Removed merge partition redundancy check (JCTVC-G681)

- Incorporated simplified merge pruning (JCTVC-G1006)

- Incorporated extend scaling factor clipping to 16 (JCTVC-G223)

- Incorporated amvp position dependency removal (JCTVC-G542)

- Incorporated simplified TMVP refidx derivation (JCTVC-G163)

- Incorporated MaxNumMergeCand signaling in slice header (JCTVC-G091)

- Incorporated modified H and center TMVP positions (JCTVC-G082)

- Incorporated intra smoothing for horizontal and vertical directions (G457)

- Incorporated removal of ALF DC offset (JCTVC-G445)

- Incorporated line buffer elimination (JCTVC-G145)

- Incorporated simplified intra mode mapping (JCTVC-G418/G109/G144)

- Incorporated modified intra mode coding (JCTVC-G119)

- Incorporated luma interpolation filter (JCTVC-G778)

- Incorporated chroma interpolation filter (JCTVC-G778)

- Incorporated simplified intra padding (JCTVC-G812)

- Incorporated modified cRiceParam update (JCTVC-G700)

- Incorporated 8bit init values for CABAC (JCTVC-G633)

- Incorporated harmonized pred and part mode binarization (JCTVC-G1042)

- Incorporated significant map context reduction (JCTVC-G1015)

- Incorporated level chroma context reduction (JCTVC-G783)

- Incorporated diagonal sub-block scan for residual coding (JCTVC-G323)

- Removed NSQT remapping and transform reordering (JCTVC-G1038)

- Incorporated modified last_significant_coeff_x/y coding (JCTVC-G201/G704)

- Incorporated shared chroma CBF contexts (JCTVC-G718)

- Incorporated luma intra mode bypass coding (JCTVC-G767)

- Incorporated multi-level significant map (JCTVC-G644)

- Incorporated WD and HM mismatch for LM prediction (JCTVC-G1034)

- Revert the slice boundary padding for adaptive loop filter to WD4

- Incorporated 4x4 BA classification (JCTVC-G609)

- Incorporated virtual boundary processing (JCTVC-G212)

- Incorporated fixed K-table for ALF (JCTVC-G610)

- Incorporated removing 15th merge flag for BA mode in ALF (JCTVC-G216)

- Incorporated prediction of ALF coefficients (JCTVC-G665)

- Incorporated deblocking clean-up (JCTVC-G1035/G620)

- Revert the deblocking decision to HM3 (JCTVC-G088)

- Incorporated support of varying QP in deblocking (JCTVC-G1031)

- Incorporated reducing motion data line buffers (JCTVC-G229)

- Incorporated reference picture set (RPS) (JCTVC-G1002)

- Incorporated reference picture set prediction (JCTVC-G198)

- Incorporated separate decisions for each half (4 lines) of a length 8 block boundaries (JCTVC-G590)

- Incorporated BoG on deblocking fix (JCTVC-G1035)

- Incorporated core transform (JCTVC-G495)

- Incorporated clipping at the output of the first inverse transform (JCTVC-G782)

- Incorporated forbidding level values outside of 16b (JCTVC-G719)

- Incorporated reducing cbf flag signalling redundancy (JCTVC-G444)

- Incorporated changing luma/chroma coefficient interleaving from CU to TU level (JCTVC-G381)

- Incorporated defining MaxIPCMCUSize, MinChromaTrafoSize (JCTVC-G112)

- Incorporated harmonization of implicit TU, AMP and NSQT (JCTVC-G519)

- Incorporated improved weighted prediction (JCTVC-G065)

- Incorporated redundancy removal of explicit weighted prediction syntax (JCTVC-G441)

- Incorporated non-cross-tiles loop filtering for independent tiles (JCTVC-G194)

- Incorporated low latency CABAC initialization for dependent tiles (JCTVC-G197)

- Incorporated AVC-based quantization matrices syntax (JCTVC-G434)

- Incorporated HVS-based quantization matrices (JCTVC-G880)

Some open issues remain for JCTVC-G1103:

- Should support for monochrome, 4:2:2 and 4:4:4 (with and w/o separate color planes) be included from the start? Currently, it has been left in the text as it doesn't seem to affect much text.

- Handling of the term "frame". We are currently leaning towards changing all occurrences of "frame" to "picture" (removed or marked all occurrences of "field")

- Large size table (de-scaling matrices) is not inserted yet.

- Slice-header syntaxes and their semantics are not completed yet. Also possible modifications that maybe necessary because of larger treeblocks (64x64) compared to macroblocks (16x16) are not yet considered.

- High-level syntax items are significantly different from software in terms of both meaning and position. Needs to be unified.

- Use of bin string and bit string should be consistent.

- MD5 hash (JCTVC-E490)

- ALF coefficient derivation from syntax elements

- Improve text quality by considering: strict use of terms “unit” and “block” – block = a rectangular 2D array (one component), unit = collective term for specifying information for both luma and chroma. Don’t use ther term ‘unit’ by itself – always use the term ‘unit’ with prefix – coding unit, prediction unit or transform unit.

- Syntax functions cuAddress and granularity_block_boundary (fine granularity slices, JCTVC-E483) are missing

- Do we need SPS flag for disabling PCM mode? Currently PCM mode is turned on for all cases.

- Software seems to have SPS syntax to turn on/off combined reference list (JCTVC-D421), but currently text does not.

- Both variables IntraPredMode and IntraPredModeC are used in the syntax table but the actual derivations are specified in the decoding process. Maybe it’s better to move them to the semantics section.

- Binarization of intra_chroma_pred_mode reflecting codeword switching and luma-based chroma intra prediction (JCTVC-E266) is missing.

- Clarification on the use of intra_chroma_pred_mode and IntraPredModeC is needed. The former specifies the syntax item to indicate how to determine the chroma intra prediction mode (IntraPredModeC) as 0 (to Intra_FromLuma), 1 (to Intra_Vertical), 2 (to Intra_Horizontal), 3 (Intra_DC or Intra_Planar) or 4 (re-use luma mode). The latter specifies chroma intra prediction mode, which is actually mapped to the specific prediction process.

- Chroma QP derivation process is missing. Can be taken from H.264/AVC and modified accordingly.

- The variable qPL used in deblocking filter process has to be defined by, e.g. setting it to QPY.

- The Chroma QP should be considered in chroma edge filtering instad of luma QP.

- The tc and beta values of the deblocking filter should be explicitly scaled in the text since their values are provided in the table for the 8-bit case.

- There is no CABAC parsing described for the following syntax elements (ae(v)): sao_flag_cb, sao_flag_cr, sao_split_flag, sao_type_idx and sao_offset.

- Deblocking filter text seems incorrect. Some horizontal edges are not filtered mainly due to the parallel deblocking process. Usage of indices xDk and XPOS also has to be cross-verified with the software behaviour.

- Software-text mismatch of JCTVC-F252: SW applies DC filtering to 64x64 intra block but WD does not.

- There are input/output parameter inconsistencies between function call and actual functions of intra prediction. Function call uses sample position, block, size and chroma index while some functions do not use some parameters.

- PPS flag disabling the temporal motion vector predictor missing in both WD and HM (JCTVC-F744).

- No semantics for deblocking filter control syntaxes (inherit_dbl_params_from_APS_flag, beta_offset_div2, and tc_offset_div2). Maybe high-level text already has them.

- Absolute operation |·| is not defined. It could be replaced by Abs(·) or defined separately in arithmetic operators subclause.

The general list of key HEVC issues that still need to be addressed was identified to be:

- In-loop filtering cleanup

- Profiles & levels

- Picture buffering and high-level syntax

- Parallel processing cleanup

- NSQT & SDIP harmonization and cleanup

# Recommendations

The recommendations of the HEVC Draft and Test Model Editing AHG are to:

- Approve the edited JCTVC-G1102 and JCTVC-G1103 documents as JCT-VC outputs

- Continue to edit both documents to ensure that all agreed elements of HEVC are fully described

- Encourage the use of the issue tracker (http://hevc.kw.bbc.co.uk/trac) to facilitate the reporting of issues with the text of either document

- Compare the HEVC documents with the HEVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG

- Continue to improve the overall editorial quality of the HEVC WD, to allow it to proceed to CD ballot shortly after the San José meeting

- Ensure that, when considering the addition of new tools to HEVC, properly drafted text for addition to both the HEVC Draft and the HM Test Model (if appropriate) is made available in a timely manner

# References

- JCT-VC, “HM5: High Efficiency Video Coding (HEVC) Test Model 5 Encoder Description”, JCTVC-G1102, JCT-VC Meeting, Geneva, November 2011

- JCT-VC, “WD5: Working Draft 5 of High-Efficiency Video Coding”, JCTVC-G1103, JCT-VC Meeting, Geneva, November 2011