| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, 1-10 February, 2012 | Document: JCTVC-H0005 MPEG Doc. m23846 |
| --- | --- |

| Title: | Spatial Transforms Ad Hoc Group Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala, FastVDO LLC Madhukar Budagavi, Texas Instruments Robert Cohen, Mitsubishi Electric Research Labs Rajan Joshi, Qualcomm, Inc. | Email: | pankaj@fastvdo.com. madhukar@ti.com cohen@merl.com rajanj@qualcomm.com |
| Source: | FastVDO LLC, Texas Instruments, Mitsubishi Electric, Qualcomm |  |  |

_____________________________

# Introduction

The purpose of AhG is to investigate the design and application of transforms in HEVC.  This includes the architectural improvements of the conventional block integer DCT-like transforms, as well as alternative transforms proposed for use in intra coding. This AhG addresses the design of transforms that achieve high performance, but with reasonable complexity.  Complexity is measured not only in terms of operations count, but memory access impact and software and hardware implementation convenience. The objectives of these experiments are to:

- Evaluate compression efficiency vs. complexity of proposed designs according to test conditions.

- Analyze hardware implementation complexity in terms of area, memory (e.g., transpose buffer size and bit-width), and throughput (e.g. number of cascaded multipliers).

- Analyze software complexity in terms of operations (number of operations, SIMD cycles, 16 bit v/s 32 bit etc.), memory accesses, and on-chip memory requirements (e.g. 16-bit or 32-bit transpose buffer size).

4.  Analyze memory requirements of additional quantization matrices used for compensating fixed-		     point effects of transform and study impact of transforms on quantization complexity.

# Experiment tasks

The following tasks were performed in this AhG:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency vs. complexity of proposed designs according to test conditions;

- Analyze complexity in terms of operations, memory access, and hardware implementability ease;

- Report the results and conclusions of these experiments.

# Description of tools under study

The following coding tools and analysis methods were used for study in this AhG.

1. Spatial integer block transforms of size 4, 8, 16, 32, as either core or secondary transforms.

2. Methods to evaluate computational complexity of proposed transforms, and its impact on software or hardware implementations.

Examples of proposals at the San Jose meeting that demonstrate such tools include:

JCTVC-H0125, JCTVC-H0126, (JCTVC-H0305), JCTVC-H0309, JCTVC-H0559, JCTVC-H0456

Cross-checks for the above proposals include:

JCTVC-H0080, JCTVC-H0124, JCTVC-H0127, JCTVC-H0324, JCTVC-H0393, JCTVC-H0589, JCTVC-H0603, (JCTVC-H0672), JCTVC-H0689, (JCTVC-H0692)

Document numbers in parentheses indicate that the document was not available at the time of writing of this report.

New proposals are welcome along these or similar lines, and are especially sought in the domain of hardware implementation complexity analysis.  Regarding specific transform proposals, it is intended to conduct experiments similar to core experiments, with cross-checks, etc; see below.

# Activities This Cycle

The activity in this AhG was mainly focused on the CE7 work on additional primary and secondary transforms. Only one document, not yet uploaded, was on the topic of core transforms. The additional transform work is covered in detail in the CE7 Summary Report. Several discussions took place via email among CE7 participants regarding how these transforms worked, along with updates and corrections to documents being submitted for this JCT-VC meeting. An overview of transform-related documents for CE7 is provided in the tables below.

Summary of documents for CE7 on Additional Transforms

| Tool # | Participants | Document | Tool Description and cross-checkers |
| --- | --- | --- | --- |
| 1 | A. Saxena, Y. Shibahara, F. Fernandes, T. Nishi | JCTVC-H0125 | Mode-dependent secondary transform for intra luma: Depending on the intra prediction mode, a secondary transform may operate on the low-frequency coefficients of the 8x8 and larger HM core transform, for luma. When used, this transform is applied in addition to the HM core transform. Secondary transform sizes of 4x4 and 8x8 will be tested. |
|  | Cross-checks: | JCTVC-H0124 | 2. A. Ichigaya, Y. Sugito |
| 3 | J. An, X. Guo, X. Zhao, S. Lei | JCTVC-H0309 | Boundary-dependent transform for inter: For 4x4, 8x8, and 16x16 inter TUs, different transforms (DST-VII/DCT-IV or DCT-II, using the existing HM transform cores) are selected for luma depending on whether the TU boundary is a PU boundary. When used, this transform is applied in place of the HM core transform. |
|  | Cross-checks: | JCTVC-H0324 JCTVC-H0080 JCTVC-H0127 | 1. R. Cohen 2. Y. Sugito, A. Ichigaya 3. Ankur Saxena, Felix Fernandes |
| 4. | A. Saxena, F. Fernandes | JCTVC-H0126 | Unified boundary-dependent secondary transform for inter luma: For 8x8 and larger TUs, the HM core transform is followed by a secondary transform. For 4x4 TUs, tests will include a secondary transform and a boundary-dependent transform in the framework of JCTVC-G281. Unifying this tool with JCTVC-G108 for Intra blocks will also be investigated. Secondary transform sizes of 4x4 and 8x8 will be tested. |
|  | Cross-checks: | JCTVC-H0589 JCTVC-H0393 | 1.M. Mrak, D. Flynn 2. R. Cohen |

Summary of “Non-CE7” documents related to CE7 on additional transforms

| Document No. | Title | Authors |
| --- | --- | --- |
| JCTVC-H0559 | Non-CE 7 : Recent results for secondary transforms for intra/inter prediction residual | A. Saxena (Samsung), Y. Shibahara (Panasonic), E. Alshina (Samsung), F. Fernandes (Samsung), T Nishi (Panasonic) |
| JCTVC-H0456 | Non CE 7: Experimental Results for the ROT | Z. Ma, F. Fernandes, E. Alshina, A. Alshin (Samsung) |

# Appendix.  AhG Mandate

| Spatial Transforms (jct-vc@lists.rwth-aachen.de) - Study the transforms in the HM design, including compression performance, computational complexity, dynamic range, clipping, storage requirements, etc. - Perform analysis of block transform design and architecture, including software and hardware considerations. - Discuss transform-related Core Experiments, and identify potential synergies or incompatibilities related to the tools being tested in the CEs. - Report the results and conclusions of these discussions and experiments to the JCT-VC. | P. Topiwala (chair), M. Budagavi, R. Cohen, R. Joshi (vice chairs) |
| --- | --- |

# Recommendations

- Review CE7 and its outputs/recommendations

- Review related documents in summary form, especially as they relate to the CE

- Make selection suggestions to the JCTVC as appropriate