| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0008 |
| --- | --- |

| Title: | JCT-VC AHG report: Profile and level definitions (AHG 8) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | M. Horowitz (eBrisk Video) K. McCann (Samsung/ZetaCast) T. Suzuki (Sony) T. K. Tan (NTT DOCOMO) W. Wan (Broadcom) Y. -K. Wang (Qualcomm) | Email: | michael@ebriskvideo.com ken@zetacast.com teruhikos@jp.sony.com ttk@pacific.net.sg wwan@broadcom.com yekuiw@qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Profile and level definitions (AHG8) between the 7th JCT-VC meeting in Geneva (20-30 November, 2011) and the 8th JCT-VC meeting in San Jose (1–10 February, 2012).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| Profile and level definitions (AHG8) (jct-vc@lists.rwth-aachen.de) - Study potential architectures of profiles that could be used to define operation points with trade-off between complexity and compression efficiency, facilitating maximum interoperability. - Study a potential set of levels to span the target range of applications with a reasonable degree of granularity - Investigate the feasibility of meeting the vast majority of industry requirements for complexity / compression efficiency trade-off in the target 8-bit 4:2:0 video applications with a single profile. - Identify potential clustering of coding tools and constraints in terms of coding efficiency vs. complexity trade-off for the majority of applications. - Evaluate the coding tools identified in the previous mandate to help assist profiling decisions by JCT-VC. | M. Horowitz, K. McCann (co-chairs), T. Suzuki, T. K. Tan, W. Wan, Y. -K. Wang (vice chairs) | No |

# Email Activity

AHG8 used the main JCT-VC reflector: jct-vc@lists.rwth-aachen.de.

A kick-off message was sent out by the chairs on 9 December 2011.  Only one other email relating to the work of AHG8 was sent to the reflector, although there are six related input documents.

# List of Related Input Documents

The following input documents have been identified as relevant to this AHG:

- JCTVC-H0116 [TK Tan, A Fujibayashi, J Takiue (NTT DOCOMO)] “[AHG8] Objective and subjective evaluation of HM5.0”

- JCTVC-H0168 [T. Suzuki (Sony), H. Sasai, T. Nishi (Panasonic)] “AHG8: Proposal of HEVC profile/level definitions”

- JCTVC-H0353 [M. Zhou, V. Sze, H. Tamama (TI)] “AHG8: Comments on HEVC version 1 Profile and Level definition”

- JCTVC-H0463 [W. Wan, T. Hellman (Broadcom)] “Comments on specifying tile support in HEVC Profiles and Levels”

- JCTVC-H0637 [Ajay Luthra (Motorola Mobility)] “Profiles and Levels for HEVC”

- JCTVC-H0668 [A. Wells, C. Fogg (Ambarella)] “Profiles and Level considerations”

# Architecture of Profiles and Levels

Profiles and Levels have been used in video coding standards since MPEG-2 to facilitate interoperability by defining specific operating points.  Profiles and Levels are defined as follows in the MPEG HEVC Requirements [1]:

“A Profile is a set of algorithmic tools, representing a particular trade-off of performance and resource consumption, supporting the requirements of a particular set of applications.  Applications which require similar trade-offs between these parameters should generally use the same Profile.”

“A Level describes performance parameters within each Profile, which describe lower limits on decoder capability.  Performance parameters typically include maximum picture size, macroblocks per second, bitrate, frame rate, buffer sizes and similar parameters.”

A profile architecture that uses an “onion ring” approach, where each profile is a strict subset or superset of other profiles, helps maximize interoperability between profiles. Alternative approaches, such as the “garlic” structure suggested in [7], may bring other benefits.

# “Main” Profile

The Main Profile is a provisional name for a profile that is capable of meeting the vast majority of industry requirements for complexity / compression efficiency trade-off in the target 8-bit 4:2:0 video applications with a single profile.

Contribution JCTVC-H0168 [4] suggested the following constraints for the Main Profile:

- LCU size: Max 64x64, Min 16x16

- Slice constraints: Min slice granularity of LCU size and each slice starts from LCU boundary

- Max number of reference frames: max value of 4 (maybe 5, if necessary)

- Number of PS: Max SPS of 8, Max PPS of 16, Max APS of 8

- Parallel processing: Constraints on tiles

An alternative approach would be to define two profiles to cover mainstream requirements, such as the Baseline Profile and High Profile proposed in JCTVC-H0353 [5].

# Clustering of Tools

The AHG chairs suggest the following strategy to identify potential clustering of coding tools and constraints in terms of coding efficiency vs. complexity trade-off for the majority of applications:

- Identify the set of “mainstream” applications that we seek to cover by the "Main" profile (or whatever it will be called)

- Characterise the requirements of these applications

- Identify a set of tools that could form a profile that would fulfil all these requirements

- Within this set, identify those tools whose performance / complexity trade-off may be borderline for this profile

- Further evaluate those tools identified during the above step

# Evaluation of Tools

Contribution JCTVC-H0116 [3] reported objective and subjective performance measures of the HM5.0 for high efficiency (HE) and low complexity (LC) common conditions as specified in JCTVC-G1200 [2].  Combinations of LC with one or more of the following tools enabled in HE but not in LC were also investigated: RDOQ, ALF, SAO, LMChroma, AMP, and NSQT. It described a step-by-step assessment of the performance of HM5.0 and its tools combinations starting from objective measurements and followed by informal subjective assessments.

Graphs of the average BD-rate vs. encoding / decoding time reportedly showed that the objective gains of HE compared to LC were mainly contributed by RDOQ, ALF and SAO.  It also reported that RDOQ does not incur any additional decoding time for the decoder, but SAO and ALF contributes most if not all of the additional decoding time needed by HE compared to LC. Between SAO and ALF, ALF contributes the larger portion of the decoding time.

A summary of the objective evaluation (i.e., BD-rate % gains relative to JM18.0, encoding and decoding times) reported in contribution JCTVC-H0116 for the coding tools RDOQ, ALF, SAO, LMChroma, AMP, and NSQT is as follows:

- The gains are mostly contributed by RDOQ, ALF and to a lesser extent SAO.

- Random access:		RDOQ (3.5%), ALF (2.6%) and SAO (0.8%)

- Low delay B: 		RDOQ (2.3%), ALF (2.0%) and SAO (1.3%)

- Low delay P: 		ALF (6.4%), SAO (5.4%) and RDOQ (1.4%)

- Intra only:			RDOQ (4.2%) and ALF (1.7%)

- Together ALF and SAO account for most of the difference in decoding time between HE and LC.

- AMP increases encoding time, but contributes coding gains as follows.

- Random access: 	Dec Time: -4%	 	Enc Time: +8%		BD-Rate gains: 0.6%

- Low delay B: 	Dec Time: +1%		Enc Time: +14%	BD-Rate gains: 0.7%

- Low delay P: 	Dec Time: +1%		Enc Time: +10%	BD-Rate gains: 0.6%

- NSQT has negligible impact on encoding and decoding time, but contributes coding gains as follows

- Random access: 	BD-Rate gains: 0.2%

- Low delay B: 	BD-Rate gains: 0.6%

- Low delay P: 	BD-Rate gains: 0.5%

The contribution asserted that visual comparison showed that no tool except for SAO gave any significant subjective quality improvement. Based on these results, the contribution selected LC with RDOQ and SAO enabled (LC+RDOQ+SAO) as the tool combination that gave the best trade off in terms of coding gains and decoding time.

# Levels and constraints

Initial ideas on a potential set of levels were proposed by JCTVC-H0168 [4] JCTVC-H637 [7] and JCTVC-H0668 [8]. The level definitions for HEVC should be discussed after reviewing these contributions.

Level-specific constraints on coding tools were suggested in JCTVC-H0168 [4], JCTVC-H0353 [5] and JCTVC-H0463 [6]. The following types of constraints were discussed:

- Parallel processing tools (restrictions to Tile and/or Wavefronts)

- LCU size, Slice size

- Worst case complexity reduction

- Entropy coding

- Memory bandwidth restrictions

This issue should be discussed during the JCT-VC meeting.

# Recommendations

The recommendations of the AHG on Profile and level definitions are:

- The profile architecture for HEVC should follow an “onion ring” approach of strict sub-sets and super-sets, unless there are good reasons to do otherwise

- JCT-VC should seek to define some form of “Main” profile that meets the majority of industry requirements for complexity/compression efficiency trade-off in a single profile

- The inclusion or exclusion of specific coding tools from the “Main” profile should be based on the result of an evaluation in terms of coding efficiency vs. complexity trade-off for the majority of applications

- Such an evaluation process should be based on subjective assessment

- JCT-VC should review all relevant contributions and should seek to define levels of HEVC with a reasonable degree of granularity

# References

- ISO/IEC JTC1/SC29/WG11/N11872, “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)”, January 2011

- JCTVC-G1200 [F. Bossen], “Common conditions and software reference configurations”, November 2011

- JCTVC-H0116 [TK Tan, A Fujibayashi, J Takiue (NTT DOCOMO)] “[AHG8] Objective and subjective evaluation of HM5.0”, February 2012

- JCTVC-H0168 [T. Suzuki (Sony), H. Sasai, T. Nishi (Panasonic)] “AHG8: Proposal of HEVC profile/level definitions”, February 2012

- JCTVC-H0353 [M. Zhou, V. Sze, H. Tamama (TI)] “AHG8: Comments on HEVC version 1 Profile and Level definition”, February 2012

- JCTVC-H0463 [W. Wan, T. Hellman (Broadcom)] “Comments on specifying tile support in HEVC Profiles and Levels”, February 2012

- JCTVC-H0637 [Ajay Luthra (Motorola Mobility)] “Profiles and Levels for HEVC”, February 2012

- JCTVC-H0668 [A. Wells, C. Fogg (Ambarella)] “Profiles and Level considerations”, February 2012