| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0012 |
| --- | --- |

| Title: | JCT-VC AHG Report: Objective quality metric and alternative methods for measuring coding efficiency (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan (co-chair) Koohyar Minoo(co-chair) | Tel: Email: | garysull@microsoft.com kminoo@motorola.com |
| Source: | AHG12 |  |  |

_____________________________

# Abstract

This report summarizes the activities, related to the mandates of AHG12 on Objective Quality Metric and Alternative methods for measuring the coding efficiency.

# Mandates

- Study and report suggested methods for objective quality assessment of reconstructed video sequences (for purposes of HEVC coding experiment evaluations).

- Identify problems and possible solutions for extending any proposed objective quality metrics to color spaces other than 4:2:0 YCbCr. (e.g. 4:4:4 RGB)

- Study methods to compare and report the overall coding efficiency of coding tools as a single number which captures overall rate and overall quality (over all color components) of a sequence.

- Identify associated changes for HM software and Excel template (used for reporting simulation results) to support the suggested methods.

# Reflector activity

For the first two mandates, on objective quality metrics there were some discussions on the reflector. These discussions mainly considered the suitability of different color spaces for the purpose of objective quality assessment and how a mean squared error metric should be modified by effective sub-sampling in different color components.

On the last two mandates (i.e. error/quality pooling for the purpose of reporting a single coding efficiency value) there was an email message sent to the reflector by the AHG12 chairs. The message (quoted below) described computing a multi-component (weighted or unweighted) average MSE in the YUV domain and converting this to the PSNR domain, rather than using our historical practice of computing signle-component PSNR values and then averaging these together in the PSNR domain.

In response, various issues were discussed on the e-mail thread. These included the following:

- Adjustments for 4:2:0 versus 4:4:4 or 4:2:2 sampling

- Potentially measuring error in the RGB domain rather than the YUV domain (and the extent to which these may be equivalent, and clipping effects).

- How to select MSE weighting values for individual components (e.g., weighting G more in an RGB measurement, picking weights based on perceptual importance, or picking weights based on slopes of rate-distortion curves)

The starting message of the conversation is quoted below (including a small correction discussed on the reflector):

The objective metric data that we typically use (for 8 bit per sample video) is to measure this on a per-frame basis:
 PSNR_Y = 10 * log10( 255^2 / MSE_Y )
 PSNR_U = 10 * log10( 255^2 / MSE_U )
 PSNR_V = 10 * log10( 255^2 / MSE_V )

We then typically create a sequence average by averaging the per-frame measurements. This effectively becomes a geometric mean of the per-frame individual component MSE values.

To combine these three numbers into a single number, the following has sometimes been used:

 PSNRwavg = wy * PSNR_Y + wu * PSNR_U + wv * PSNR_v

 where wy + wu + wv = 1

And typically this has used the weighting of wy = 3/4, wu = wv = 1/8.

A simple candidate alternative metric is to measure this on a per-frame basis:

 wMSE = wy * MSE_Y + wu * MSE_U + wv * MSE_V

 PSNR_wYUV = 10 * log10( 255^2 / wMSE )

 where wy + wu + wv = 1

And then to create a sequence average by averaging the per-frame measurements.

An especially simple form of this metric for 4:2:0 video is with wy = 2/3, wu = wv = 1/6, which corresponds to an unweighted MSE per sample.
For 4:2:2, this would become wy = 1/2, wu = wv = 1/4. For 4:4:4, this would simply be wy = wu = wv = 1/3.
That version can simply be called PSNR_YUV.

For the sake of simplicity, this may be the best to focus on as a starting point (at least for 4:2:0).

One advantage of using this type of metric is that having a very low MSE in a single component does not have undue influence on the combined measure, as the component-specific MSEs get averaged together before transforming them to the log domain.

Another advantage is that the minimization of wMSE can be easily incorporated directly into the RDO decision-making process of the encoder.

A more complicated metric, e.g. something from ITU-T J.246, J.247, or J.341, would probably be better-performing in terms of correspondence to true subjective quality. However, it would also be harder to understand, more difficult to include in our HM reference software, and more difficult to optimize for in the encoder decision-making process.

We suggest to incorporate the computation of PSNR_YUV into a version of the reference software and the results spreadsheet template for testing purposes, and to generate some modified BD measures based on it.

We also would like to align the work in this AHG with the work on subjective testing by generating PSNRwavg and PSNR_wYUV scores for the same test cases that are used in the subjective testing and see how well the results seem to correspond to those from the subjective evaluation.

# Related Contributions

The following informational contribution is noted to be related to objective quality metric:

- JCTVC-H0063 Coding efficiency comparison between HM5.0 and JM16.2 based on PQI, PSNR and SSIM [Yin Zhao (Zhejiang University)]