| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0013 |
| --- | --- |

| Title: | JCT-VC AHG report: Interlace indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono NEC Corporation Chad Fogg Ambarella Inc. Kazushi Sato Sony Corporation Shun-ichi Sekiguchi Mitsubishi Electric Corporation Wade Wan Broadcom Corporation | Email: | chono@ct.jp.nec.com chadfogg@gmail.com Kazushi.Sato@jp.sony.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp wwan@broadcom.com |
| Source: | AHG chair and vice-chairs |  |  |

_____________________________

# Abstract

This report summarizes the Interlace indication Ad hoc activities between the 7th JCT-VC meeting in Geneva, CH (21 to 30 November, 2011) and the current 8th JCT-VC meeting in San Jose, US (1 to 10 February, 2012).

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Interlace indication (jct-vc@lists.rwth-aachen.de) - Develop candidate text for an interlaced format indicator, starting from a VUI flag signaling the presence of an SEI message. | K. Chono (chair), C. Fogg, K. Sato, S. Sekiguchi, W. Wan (vice chairs) | N |

# Activities

A SEI message based on H.263 Annex W was defined according to the last meeting decisions. The staring text specifying the VUI flag and SEI message was distributed to the JCTVC main reflector on December 22.

The following opinions were expressed on the starting text (or interlace indication):

- Move the VUI flag to SPS in order to simplify the signaling of the type of a coded sequence;

- Reserve more bits of the field of the SEI message to indicate other ways of interpreting the decoded arrays in addition to interlace;

- Perhaps we need to define some stronger means than an SEI message to convey this kind of information to avoid "external hacks". The file format, for example, needs to specify the presence of the SEI message;

- AVC style indication (conditional signaling of bottom_field_flag in slice headers) can be such a stronger means but it uses normative bits.

Furthermore, the following were suggested beyond interlace indication in order to bundle and signal all of display processing information in the final HEVC design:

- Define an unified sequence level SEI message that contains information at a sequence level about characteristics of the sequence (e.g., color space, progressive/interlace, etc);

- Define an unified picture level SEI message that contains information at a picture level about characteristics of the picture (e.g., top/bottom indication for interlaced video, stereoscopic side-by-side/mono indication for frame-compatible stereo video, etc);

- Define a picture-level signaling means other than SEI message to indicate dynamically changing source material properties.

# Related contributions

[Interlace indication] 3 contributions

JCTVC-H0056 [NEC] AHG13: On interlace indication

JCTVC-H0588 [Mitsubishi] AHG13: Proposed interlace format indication for HEVC

JCTVC-H0666 [Ambarella] Interlace Indicators

[Interlace indication and coding] 1 contribution

JCTVC-H0222 [Fujitsu] Introduction of AVC's derivation process of chroma motion vector for interlace format support

[Bundling of display processing information] No contribution

# Summary of contributions

[JCTVC-H0056]

- Move the VUI flag in the AHG13 staring text to SPS and modify its definition to specify the type of the associated coded sequence in addition to the presence of the SEI message in each of access units.

- One-bit interlace field indication in SEI message to indicate the parity of each field.

- Modified access unit delimiter RBSP to indicate dynamically changing source material properties.

Note: The third aspect is beyond interlace indication and out of the AHG13 focus.

[JCTVC-H0588]

- Define a constraint that a coded sequence shall contain coded pictures consisting of frames only (progressive or interlace) or fields only.

- One-bit interlace format indication in SPS or VUI

- One-bit interlace field indication in SEI message to indicate the parity of each field.

[JCTVC-H0666]

- A flag to indicate the scanning method of the original source of an coded sequence.

- A flag to indicate whether a coded sequence contains frames only or fields only.

- A flag to indicate the parity of each field.

- A flag to indicate whether the coded frame or field is a "dummy frame" resulting from a repetition process, such as 3:2 pulldown.

Note: The second and third aspects are similar to the VUI flag and the SEI message, respectively, in the AHG13 starting text. The remaining aspects are similar to pic_struct in AVC for display processing.

[JCTVC-H0222]

- SPS flag to specify the scanning method of an input source.

- One-bit interlace field indication in slice header when SPS flag indicates that the input source is interlaced scan.

- AVC-style chroma MVphase adjustment.

Note: The third aspect is interlace coding and out of the AHG13 focus.

# Recommendation

- Discuss and conclude SEI/VUI proposals to complete the WD/CD text (VUI flag and SEI message) first

- Then review other related proposals as an enhancement