| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0016 |
| --- | --- |

| Title: | JCT-VC AHG report: Unification of NSQT and SDIP (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCTVC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | X. Zheng Hisilicon Technologies J. Xu Microsoft X. Wang Qualcom A. Tabatabai Sony J. Lim LGE | Email: | zhengxiaozhen@huawei.com jzxu@microsoft.com xianglin@qualcomm.com ali.tabatabai@am.sony.com jaehyun.lim@lge.com |
| Source: | JCT-VC AHG on Unification of NSQT and SDIP (AHG 16) |  |  |

_____________________________

# Abstract

This document summarizes the activities of the JCT-VC ad-hoc group on Unification of NSQT and SDIP (AHG16) between the 7th JCT-VC meeting in Geneva, CH (21-30 November 2011) and the 8th JCT-VC meeting in San José, USA (1-10 February 2012).

# Mandates

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Unification of NSQT and SDIP (AHG16) (jct-vc@lists.rwth-aachen.de) - Implement and harmonize the non-square partitioning techniques in SDIP (JCTVC-G558 and JCTVC-G754) with the NSQT in HM5. - Study and investigate combinations of PU and TU sizes that lead to the best coding efficiency and complexity tradeoff - Provide the WD text that is entirely consistent with the unified software (HM5 + JCTVC-G558 + JCTVC-G754). | X. Zheng (chair), J. Xu, X. Wang, A. Tabatabai, J. Lim (vice chairs) | N |

# Related activities and input contributions

A kick-off email was sent on 13th January 2011. There were about 30 emails exchanged on the reflector concerning AHG16. The AHG16 software is distributed via email. There are two technical proposals and four cross-checks related to the mandates of AHG16.

There were discussions on intra PU type, intra PU signalling method, and TU sizes at intra- and inter- block coding, which have been studied and evaluated at JCTVC-H0347 and JCTVC-H0455.

Input Documents:

- JCTVC-H0347 [Tsinghua, USTC, HiSilicon, Microsoft, Huawei, LGE, Qualcomm, Sony, MediaTek] AHG16: SDIP unified with NSQT

      - Cross-check: JCTVC-H0106 [Sharp] AHG16:Crosscheck of Unification of SDIP and NSQT (JCTVC-H0347 case1)

      - Cross-check: JCTVC-H0300 [Intel] AHG16: Crosscheck of SDIP and NSQT unification (JCTVC-H0347)

      - Cross-check: JCTVC-H0558 [Samsung] AhG16: Cross-Check of H0347 on unification of SDIP and NSQT (tests 1 and 2)

- JCTVC-H0455 [MediaTek] AHG16: Harmonization of 2NxN/Nx2N Intra PU with SDIP and NSQT

      - Cross-check: JCTVC-H0302 [Intel] AHG16: Crosscheck of MediaTek’s harmonization of 2NxN/Nx2N Intra PU with SDIP and NSQT (JCTVC-H0455)

# Analysis

Table 1: PU types used at NSQT at WD5, and SDIP at H0347 and H0455

| CU size | NSQT | SDIP case1 (H0347) | SDIP case3.a (H0347) | SDIP case3.b (H0347), H0455 | SDIP case2 (H0347) | SDIP case4 (H0347) |
| --- | --- | --- | --- | --- | --- | --- |
| 64 x 64 | 2Nx2N 2NxN, Nx2N 2NxnU, 2NxnD, nLx2N, nRx2N | 2Nx2N |  |  |  |  |
| 32 x 32 | 2Nx2N 2NxN, Nx2N 2NxnU, 2NxnD, nLx2N, nRx2N | 2Nx2N, 2NxN, Nx2N |  |  | 2Nx2N, 2NxhN, hNx2N |  |
| 16 x 16 | 2Nx2N 2NxN, Nx2N 2NxnU, 2NxnD, nLx2N, nRx2N | 2Nx2N, 2NxN, Nx2N |  |  | 2Nx2N, 2NxhN, hNx2N |  |
| 8 x 8 | 2Nx2N 2NxN, Nx2N | 2Nx2N, NxN |  | 2Nx2N, 2NxN, Nx2N, NxN | 2Nx2N, 2NxhN, hNx2N, NxN | 2Nx2N, NxN |

Table 2: TU sizes used at NSQT at WD5, and SDIP at H0347 and H0455

| CU size | PU types | NSQT (TU level 0 ~ 2) | SDIP case1 (H0347, TU level 0 ~ 2) | SDIP case3.a (H0347, TU level 0 ~ 2) | SDIP case3.b (H0347), H0455 (TU level 0 ~ 2) | SDIP case2 (H0347, TU level 0 ~ 2) | SDIP case4 (H0347, TU level 0 ~ 2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64 x 64 | 2Nx2N PU | 64x64 (skip) 32x32 16x16 |  |  |  |  |  |
|  | 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, nRx2N PU | 64x64 (skip) 32x32 32x8 / 8x32 | Null |  |  |  |  |
| 32 x 32 | 2Nx2N PU | 32x32 16x16 8x8 |  |  |  |  |  |
|  | 2NxN, Nx2N PU | 32x32 32x8 / 8x32 16x4 / 4x16 | 32x 32 (skip) 32x8 / 8x32 |  |  | Null |  |
|  | 2NxnU, 2NxnD, nLx2N, nRx2N PU |  | Null |  |  |  |  |
|  | 2NxhN, hNx2N PU | Null |  |  |  | 32x32 (skip) 32x8 / 8x32 |  |
| 16 x 16 | 2Nx2N PU | 16x16 8x8 4x4 |  |  |  |  |  |
|  | 2NxN, Nx2N PU | 16x16 16x4 / 4x16 4x4 | 16x16 (skip) 16x4 / 4x16 |  |  | Null |  |
|  | 2NxnU, 2NxnD, nLx2N, nRx2N PU |  | Null |  |  |  |  |
|  | 2NxhN, hNx2N PU | Null |  |  |  | 16x16 (skip) 16x4 / 4x16 |  |
| 8 x 8 | 2Nx2N PU | 8x8 4x4 |  |  |  |  |  |
|  | 2NxN, Nx2N PU | 8x8 4x4 | 8x8 (skip) 8x2 / 2x8 | Null | skip 4x4 | Null |  |
|  | NxN PU | 8x8 (skip) 4x4 |  |  |  | 8x8 (skip) 4x4 |  |
|  | 2NxhN, hNx2N | Null |  |  |  | 8x8 (skip) 8x2 / 2x8 | Null |

Table 3: Transform coefficient scanning used at NSQT at WD5, and SDIP at H0347 and H0455

| TU size | NSQT | SDIP case1 (H0347) | SDIP case2 (H0347) | SDIP case3.a (H0347) | SDIP case3.b (H0347), H0455 | SDIP case4 (H0347) |
| --- | --- | --- | --- | --- | --- | --- |
| 32x8 / 8x32 | subblock scan + HM5 diag. scan |  |  |  |  |  |
| 16x4 / 4x16 | subblock scan + HM5 diag. scan | subblock scan + MDCS |  |  |  |  |
| 8x2 / 2x8 | Null | 8x2 / 2x8 MDCS scan |  | Null |  |  |

Table 4: Syntax elements used at NSQT at WD5, and SDIP at H0347 and H0455

| CU size | NSQT | SDIP case1 (H0347) | SDIP case3.a (H0347) | SDIP case3.b (H0347), H0455 | SDIP case2 (H0347) | SDIP case4 (H0347) |
| --- | --- | --- | --- | --- | --- | --- |
| 64 x 64 | no new syntax | no new syntax |  |  |  |  |
| 32 x 32 |  | introduce inter PartMode (2NxN, Nx2N) to intra |  |  | add PartMode "2NxhN" and "hNx2N" to intra |  |
| 16 x 16 |  |  |  |  |  |  |
| 8 x 8 |  | introduce inter PartMode (2NxN, Nx2N) to intra | no new syntax | introduce inter PartMode (2NxN, Nx2N) to intra | add PartMode "2NxhN" and "hNx2N" to intra | no new syntax |

Regarding the coding of significance map and last significant coefficient, the methods used at 16x4, 4x16, 32x8 and 8x32 transform blocks at H0347 and H0455 are the same as NSQT. For the cases of 8x2 / 2x8 transform blocks at H0347 and H0455, a new CABAC context is used to indicate the last position of the length of 2; the contexts of 8x2 and 2x8 block’s significant map share the same context of 8x8 block.

# Results

|  | All Intra HE case1 (H0347) |  |  | All Intra HE case2 (H0347) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -1.3% | -3.6% | -4.1% | -1.4% | -3.8% | -4.3% |
| Class B | -1.5% | -3.6% | -3.7% | -1.4% | -3.6% | -3.8% |
| Class C | -1.9% | -2.7% | -2.9% | -1.8% | -2.5% | -2.7% |
| Class D | -1.8% | -2.5% | -2.5% | -1.7% | -2.3% | -2.2% |
| Class E | -2.3% | -6.7% | -6.4% | -2.2% | -6.8% | -6.4% |
| Overall | -1.8% | -3.6% | -3.7% | -1.7% | -3.6% | -3.7% |
|  | -1.8% | -3.7% | -3.7% | -1.7% | -3.6% | -3.7% |
| Class F | -4.8% | -5.2% | -5.2% | -5.9% | -6.0% | -6.2% |
| Enc Time[%] | 128% |  |  | 130% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

|  | All Intra HE case3a (H0347) |  |  | All Intra HE Case4 (H0347) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -3.3% | -4.1% | -1.0% | -3.6% | -4.3% |
| Class B | -1.0% | -3.2% | -3.4% | -1.0% | -3.3% | -3.5% |
| Class C | -0.7% | -2.0% | -2.2% | -0.8% | -2.1% | -2.4% |
| Class D | -0.6% | -1.5% | -1.4% | -0.6% | -1.6% | -1.5% |
| Class E | -1.8% | -7.1% | -6.5% | -1.9% | -7.2% | -6.6% |
| Overall | -1.0% | -3.2% | -3.3% | -1.0% | -3.3% | -3.4% |
|  | -1.0% | -3.2% | -3.3% | -1.0% | -3.4% | -3.4% |
| Class F | -0.7% | -1.4% | -1.5% | -0.9% | -1.4% | -1.7% |
| Enc Time[%] | 117% |  |  | 119% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  | All Intra HE case3b (H0455)+ |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -1.1% | -3.4% | -4.0% |
| Class B | -1.1% | -3.3% | -3.4% |
| Class C | -1.0% | -2.2% | -2.3% |
| Class D | -0.9% | -1.8% | -1.7% |
| Class E | -2.0% | -6.9% | -6.4% |
| Overall | -1.2% | -3.3% | -3.4% |
|  | -1.2% | -3.3% | -3.4% |
| Class F | -1.0% | -1.6% | -1.7% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 100% |  |  |

+ Use AhG16+H0455 combined encoder optimization.

# Recommendations

The AHG recommends that the JCT-VC review the related input contributions.