| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 |  |  |  | Document: JCTVC-H0022r1 |  |
| --- | --- | --- | --- | --- | --- |
| Title: | JCT-VC AHG report: HM subjective quality investigation (AHG22) |  |  |  |  |
| Status: | AHG report input to JCT-VC |  |  |  |  |
| Purpose: | AHG report |  |  |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm (RWTH Aachen) Gary Sullivan (Microsoft Corp.) Frank Bossen (Docomo Innovations) Thomas Wiegand (Fraunhofer HHI) Vittorio Baroncini (FUB) Mathias Wien (RWTH Aachen) Jizheng Xu (MSRA) | Email: | ohm@ient.rwth-aachen.de garysull@microsoft.com bossen@docomoinnovations.com thomas.wiegand@hhi.fraunhofer.de vittorio@fub.it wien@ient.rwth-aachen.de jzxu@microsoft.com |  |  |
| Source: | AHG |  |  |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on HM subjective quality investigation (AHG22) between the 7th JCT-VC meeting in Geneva (21-30 November, 2011) and the 8th Meeting in San Jose (1-10 February, 2012).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HM subjective quality investigation (AHG22) (jct-vc@lists.rwth-aachen.de) - Establish a testing scenario for comparison of HM versus JM together with the test coordinator, Vittorio Baroncini - Perform encoding of HM and JM test cases. - Prepare the evaluation of test results as input to the February 2012 meeting. | G. Sullivan, J.-R. Ohm (co-chairs), F. Bossen, T. Wiegand (vice chairs) | No |

# Background

The goal of the AHG work is to quantify, to the extent feasible, the rate savings for which similar subjective quality is achieved for the current HM5 compared to a similarly-configured JM encoder/decoder.

# Method of Working

It was assessed to be important to conduct the subjective tests with non-expert viewers. Due to the limited resources that were available for this purpose, tests were restricted to the RA-HE settings of the HM (ref JCTVC-G1200) and to classes B and C. A modified version of the JM encoder was used in the test, which includes some non-normative improvements as described previously in JCTVC-G399 and updated in JCTVC-H0360. The release of a new version of the JM is planned to support these non-normative modifications.

By negotiation with the test coordinator, Vittorio Baroncini, the maximum number of cases that could be tested was set to 72. With 9 sequences in classes B and C and two codecs, it was possible to test four rate points for each. Instead of testing pre-defined rate points, it was considered to try finding points of similar subjective quality, and run both codecs with fixed QP. In terms of subjective quality that would provide reasonable results when assessed by non-expert viewers, QP settings 27, 30, 33 and 36 were selected for the JM codec, with confirmation by the test coordinator. Several experts involved in the preparation performed pre-assessment in comparing the JM versus the HM codec, and the overall opinion unveiled that with same QP settings, the HM would provide substantially better visual quality than the JM, which in fact can also be confirmed by PSNR numbers. Finally, it was decided to use QPHM= QPJM+4 settings for the HM encoded cases. Though PSNR numbers are slightly worse for this case, the experts involved in pre-viewing came to the opinion that this gave a quality which is approximately equal to the JM quality in most cases. It was therefore decided to run the tests with QP settings of 31, 34, 37 and 40 for the HM (and QP settings of 27, 30, 33 and 36 for the JM).

Subjective tests were performed using a Double Stimulus Impairment Scale (DSIS) method equivalent to the approach described in [ref JCTVC-A204].

# HM Configuration used for Bitstream Generation

The HM bitstreams were generated using the HM reference software from

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/.

The simulations were run using the configuration file cfg/encoder_randomaccess.cfg from that version of the reference software. The HM encoder was run with this configuration file and command line parameters adapted for each sequence. An example for encoding the sequence BQMall_832x480_60.yuv is shown below:

bin/TAppEncoderStatic -c cfg/encoder_randomaccess.cfg --FrameRate=60 \\
--FramesToBeEncoded=600 --SourceHeight=480 --SourceWidth=832 \\
--InputFile=orig/BQMall_832x480_60.yuv --IntraPeriod=64 --QP=40 --SEIpictureDigest=0 \\
--BitstreamFile=str/BQMall_832x480_60_40.hvc --ReconFile=tmp/BQMall_832x480_60_40.yuv

The sequences of Class B and Class C were encoded. The command line parameters used in the simulations are listed in the table below. The simulations were run for QP={31,34,37,40}.

| InputFile | SourceWidth | SourceHeight | FrameRate | FramesToBeEncoded | IntraPeriod |
| --- | --- | --- | --- | --- | --- |
| Class B |  |  |  |  |  |
| BasketballDrive_1920x1080_50.yuv | 1920 | 1080 | 50 | 500 | 48 |
| BQTerrace_1920x1080_60.yuv | 1920 | 1080 | 60 | 600 | 64 |
| Cactus_1920x1080_50.yuv | 1920 | 1080 | 50 | 500 | 48 |
| Kimono_1920x1080_24.yuv | 1920 | 1080 | 24 | 240 | 24 |
| ParkScene_1920x1080_24.yuv | 1920 | 1080 | 24 | 240 | 24 |
| Class C |  |  |  |  |  |
| BasketballDrill_832x480_50.yuv | 832 | 480 | 50 | 500 | 48 |
| BQMall_832x480_60.yuv | 832 | 480 | 60 | 600 | 64 |
| PartyScene_832x480_50.yuv | 832 | 480 | 50 | 500 | 48 |
| RaceHorses_832x480_30.yuv | 832 | 480 | 30 | 300 | 32 |

- Results and Evaluation

Results of the test (MOS, standard deviation and confidence intervals) are available from the attached Excel sheet. As the subjective tests did not unveil exactly same quality of HM and JM for the different test cases, linear interpolation was performed on RD graphs (MOS over rate). Such an approach appears appropriate since the QP differences between the various test points are relatively close, such that a fairly precise estimate of the rate difference between the two codecs at same quality could be achieved. Rate differences have been computed for all cases where an MOS point for one of the codecs has a matching point (i.e. in the range where the two plots overlap on the MOS axis). By that, a gross average rate reduction of 67% for class B sequences, 49% for class C sequences and 58% overall could be deduced.

# Recommendations

The recommendations of the HM subjective quality investigation AHG are to:

- Evaluate the results of the subjective viewing tests by finding rate points where the HM and JM codecs provide most similar quality, and find a more precise rate saving for each test case by interpolation.

- To conclude on average rate savings, conduct the assessment by breakout work during the 8th JCT-VC meeting.

- Summarize the results in a report.