| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0031 |
| --- | --- |

| Title: | CE1: Entropy Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core experiment report |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Qualcomm Elena Alshina Samsung Electronics Co., Ltd. Hisao Sasai Panasonic Corporation Heiner Kirchhoffer Fraunhofer HHI Jani Lainema Nokia | Email: | rajanj@qualcomm.com elena_a.alshina@samsung.com sasai.hisao@jp.panasonic.com heiner.kirchhoffer@hhi.fraunhofer.de jani.lainema@nokia.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of Core Experiment 1 (CE1) is to evaluate changes to the existing entropy coding. In this experiment, changes to the arithmetic coding engine (CABAC), different methods of 8-bit initialization of the CABAC context models and techniques to achieve higher throughput were evaluated. This document, JCTVC-H0031, provides a summary of activities and results for this core experiment.

# Introduction

The purpose of Core Experiment 1 (CE1) is to evaluate changes to the existing entropy coding. The core experiment was divided into 4 subtests. In subtest A, proposal for modification of the existing arithmetic coding engine was investigated. In subtest B, different methods for 8-bit initialization of the CABAC context models were evaluated. In subtest C, alternatives to the arithmetic coding probability model update were explored. In subtest D, methods for increasing the throughput of CABAC were studied. This report provides a summary of activities and results for this core experiment.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Elena Alshina (coordinator) | Samsung | elena_a.alshina@samsung.com |
| Hisao Sasai (coordinator) | Panasonic | sasai.hisao@jp.panasonic.com |
| Heiner Kirchhoffer (coordinator) | Fraunhofer HHI | Heiner.Kirchhoffer@hhi.fraunhofer.de |
| Jani Lainema (coordinator) | Nokia | Jani.Lainema@nokia.com |
| Gaelle Martin-Cocher | Research In Motion | gmartincocher@rim.com |
| Dake He | Research In Motion | dhe@rim.com |
| Alexander Alshin | Samsung | alexander_b.alshin@samsung.com |
| Joel Sole Rojals | Qualcomm | joels@qualcomm.com |
| Liwei Guo | Qualcomm | liweig@qualcomm.com |
| Yeo Chuo Hao | I2R | chyeo@i2r.a-star.edu.sg |
| Andrew Segall | SHARP | asegall@sharplabs.com |

# Summary of experimental results

## Subtest A

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included):

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm JCTVC-H0444 | 64x4 range rescaling table | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.1% | 0.1% |
|  | 32x4 range rescaling table | -0.2% | -0.6% | -0.6% | -0.2% | -0.4% | -0.5% | -0.3% | -0.1% | -0.3% |

Related documents:

JCTVC-H0444: "CE1: Modified LPS range and state transition tables for BAC", J. Sole, L. Guo and M. Karczewicz (Qualcomm)

Based on: JCTVC-G324: "Modified LPS range and state transition tables for BAC", J. Sole, M. Karczewicz (Qualcomm)

Test description: JCTVC-G324 modifies the binary arithmetic coder state machine to allow a slower probability adaptation process and the coding of more skewed binary distributions than the current state machine. Various reduction ratios of the range LPS table are analyzed.

Crosschecker: Cisco (JCTVC-H0210) (detailed comments)

Related non-CE documents: JCTVC-H0265

Summary of tests results:

Table 1: 64x4 range rescaling table

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | -0.6% | -0.5% | -0.4% | -0.5% | -0.5% |  |  |  |
| Class B | -0.3% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class C | -0.3% | -0.7% | -0.5% | -0.3% | -0.4% | -0.4% |  |  |  |
| Class D | -0.4% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class E | -0.3% | -0.3% | -0.2% | -0.3% | -0.4% | -0.4% |  |  |  |
| Class F | -0.2% | -0.4% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Overall | -0.3% | -0.5% | -0.5% | -0.3% | -0.4% | -0.4% |  |  |  |
|  | -0.3% | -0.5% | -0.4% | -0.3% | -0.4% | -0.4% |  |  |  |
| Enc Time[%] | 92% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | -0.4% | -0.5% | -0.4% | -0.4% | -0.5% | -0.5% | 0.1% | 0.4% |
| Class B | -0.2% | -0.4% | -0.3% | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |
| Class C | -0.2% | -0.5% | -0.2% | -0.2% | -0.4% | -0.3% |  |  |  |
| Class D | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.2% | -0.2% | 0.0% | -0.1% | -0.3% | -0.2% |  |  |  |
| Overall | -0.2% | -0.4% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.1% | 0.1% |
|  | -0.2% | -0.4% | -0.2% | -0.2% | -0.3% | -0.4% | -0.3% | -0.1% | 0.2% |
| Enc Time[%] | 92% |  |  | 97% |  |  | 105% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | 0.0% | -0.4% | -0.1% | 0.5% |  |  |  |
| Class C | -0.1% | -0.6% | -0.3% | -0.2% | -0.2% | -0.1% |  |  |  |
| Class D | -0.1% | -0.1% | -0.5% | -0.1% | 0.3% | -0.8% |  |  |  |
| Class E | -0.1% | 0.8% | -0.4% | -0.1% | -0.2% | 0.3% |  |  |  |
| Class F | -0.1% | -0.1% | -0.3% | 0.0% | -0.6% | -0.4% |  |  |  |
| Overall | -0.2% | 0.0% | -0.3% | -0.2% | 0.0% | 0.0% |  |  |  |
|  | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 96% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |  |  |  |

Table 2: 32x4 range rescaling table

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.9% | -0.8% | -0.4% | -0.6% | -0.6% |  |  |  |
| Class B | -0.2% | -0.8% | -0.8% | -0.3% | -0.6% | -0.6% |  |  |  |
| Class C | -0.2% | -0.8% | -0.7% | -0.2% | -0.5% | -0.5% |  |  |  |
| Class D | -0.3% | -0.7% | -0.7% | -0.3% | -0.5% | -0.6% |  |  |  |
| Class E | -0.2% | -0.7% | -0.6% | -0.3% | -0.6% | -0.6% |  |  |  |
| Class F | -0.1% | -0.4% | -0.2% | -0.1% | -0.2% | -0.3% |  |  |  |
| Overall | -0.3% | -0.8% | -0.7% | -0.3% | -0.5% | -0.6% |  |  |  |
|  | -0.3% | -0.8% | -0.7% | -0.3% | -0.5% | -0.6% |  |  |  |
| Enc Time[%] | 88% |  |  | 91% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 88% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | -0.8% | -0.7% | -0.4% | -0.5% | -0.6% | -0.4% | 0.6% | 0.2% |
| Class B | -0.2% | -0.6% | -0.6% | -0.3% | -0.5% | -0.5% | -0.2% | -0.6% | -0.7% |
| Class C | -0.1% | -0.6% | -0.4% | -0.1% | -0.6% | -0.4% |  |  |  |
| Class D | -0.1% | -0.5% | -0.4% | -0.1% | -0.4% | -0.7% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | -0.1% | -0.4% | -0.3% |  |  |  |
| Overall | -0.2% | -0.6% | -0.5% | -0.2% | -0.5% | -0.6% | -0.3% | -0.1% | -0.3% |
|  | -0.2% | -0.6% | -0.5% | -0.2% | -0.5% | -0.6% | -0.2% | 0.0% | -0.2% |
| Enc Time[%] | 88% |  |  | 96% |  |  | 101% |  |  |
| Dec Time[%] | 92% |  |  | 92% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.5% | -0.3% | -0.5% | -0.1% |  |  |  |
| Class C | -0.1% | -0.5% | -0.4% | -0.1% | -0.5% | -0.4% |  |  |  |
| Class D | 0.0% | -0.3% | -0.6% | 0.0% | 0.8% | -0.3% |  |  |  |
| Class E | 0.0% | 0.2% | -0.7% | 0.1% | -0.5% | -0.3% |  |  |  |
| Class F | 0.0% | -0.2% | -0.2% | 0.3% | -0.4% | -0.2% |  |  |  |
| Overall | -0.1% | -0.3% | -0.5% | -0.1% | -0.2% | -0.3% |  |  |  |
|  | -0.1% | -0.3% | -0.5% | -0.1% | -0.2% | -0.3% |  |  |  |
| Enc Time[%] | 96% |  |  | 96% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |  |  |  |

## Subtest B

### B1 & B3

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
|  | Formula 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for low QPs = {17, 22, 27, 32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | Formula 2 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for high QPs = {27, 32, 37, 42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% |
|  | Formula 2 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |

Related documents:

JCTVC-H0535: "CE1 B3: 8-bit Linear Initialization for CABAC", L. Guo, J. Sole, R. Joshi, M. Karczewicz (Qualcomm), C. Yeo, Y. H. Tan and Z. Li (I2R)

Based on: JCTVC-G155, "Non-CE1: On CABAC context initialization", C. Yeo, Y. H. Tan, Z. Li (I2R), JCTVC-G837, "Non-CE1: 8-bit Initialization for CABAC", L. Guo, R. Joshi, J. Sole, X. Wang and m. Karczewicz (Qualcomm)

Test description: JCTVC-G837 proposes an 8-bit initialization method for CABAC. Each context is assigned an 8-bit initialization value InitIdx which is composed of 4-bit SlopeIdx and 4-bit IntersecIdx. SlopeTable (16 elements) and IntersecTable (16 elements) are introduced to convert the SlopeIdx and IntersecIdx  to m, and n respectively,  and the initial CABAC state is calculated using a linear equation. In this CE, different tables will be tested, some of which can be replaced by a formula.   Combination of this method with JCTVC-G155 will also be tested.

Crosschecker: HHI (JCTVC-H0395)

Summary of tests results:

Table 3: BD-rate results for common QPs (formula 1)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.2% | -0.2% | 0.0% | 0.2% | 0.3% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.3% | -0.1% | 0.0% | 0.3% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.3% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | 0.3% | -0.2% |  |  |  |
| Class E | 0.0% | -0.2% | -1.5% | 0.1% | -0.6% | -0.2% |  |  |  |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
|  | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class F | -0.1% | -0.3% | -0.6% | -0.1% | -1.3% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 4: BD-rate results for low QPs (formula 1)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | 0.2% | -0.2% |  |  |  |
| Class E | 0.0% | -0.4% | -0.8% | 0.1% | -0.8% | 0.3% |  |  |  |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | 0.1% |  |  |  |
|  | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | 0.1% |  |  |  |
| Class F | -0.2% | -0.2% | -0.3% | 0.1% | -0.3% | -0.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 5: BD-rate results for high QPs (formula 1)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.4% | 0.0% | 0.3% | -0.4% | 0.1% | 0.3% | -0.2% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.4% | -0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.3% | -0.4% | -0.1% | 0.3% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class F | -0.1% | 0.1% | -0.2% | 0.0% | -0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | -0.4% | 0.0% |  |  |  |
| Class C | 0.0% | 0.2% | 0.2% | -0.1% | 0.4% | 0.1% |  |  |  |
| Class D | 0.0% | -0.3% | -0.1% | 0.0% | 0.3% | -0.3% |  |  |  |
| Class E | 0.0% | -0.4% | -2.2% | 0.2% | -1.2% | -0.9% |  |  |  |
| Overall | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.2% |  |  |  |
|  | 0.0% | 0.0% | -0.3% | 0.0% | -0.2% | -0.2% |  |  |  |
| Class F | 0.1% | -0.7% | -0.6% | -0.3% | -2.6% | -1.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 6: BD-rate results for common QPs (formula 2)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.1% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.4% | 0.4% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | 0.3% | -0.1% | 0.4% | 0.1% |  |  |  |
| Class E | 0.0% | 0.8% | -0.9% | 0.1% | -0.6% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -1.0% | -0.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 7: BD-rate results for low QPs (formula 2)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.1% |  |  |  |
| Class E | 0.0% | 0.4% | 0.3% | 0.0% | -0.3% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class F | -0.1% | 0.0% | -0.1% | 0.1% | 0.1% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 8: BD-rate results for high QPs (formula 2)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.4% | 0.1% | 0.2% | 0.0% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.3% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.3% | -0.2% | 0.0% | 0.2% | -0.3% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | -0.1% | 0.2% | 0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.2% | 0.0% | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.9% | 0.7% |  |  |  |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.3% | -0.5% |  |  |  |
| Class D | 0.0% | -1.0% | 0.0% | -0.1% | 0.2% | -0.3% |  |  |  |
| Class E | 0.0% | 1.4% | -1.9% | 0.1% | -1.5% | -0.1% |  |  |  |
| Overall | 0.0% | -0.1% | -0.4% | 0.0% | -0.4% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | 0.0% |  |  |  |
| Class F | -0.1% | -0.6% | -0.2% | -0.2% | -2.7% | -1.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

### Subtest B2

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included).

For low QPs = {17, 22, 27, 32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,1% | 0,1% | 0,0% | 0,2% | 0,1% | 0,0% | -0,1% | 0,0% |
|  | Test3: ‘disabled’ mapping | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,1% | 0,0% | 0,0% | 0,0% |

For common QPs = {22, 27, 32, 37}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,1% | 0,1% | 0,0% | 0,2% | 0,1% | 0,0% | -0,1% | 0,1% |
|  | Test3: ‘disabled’ mapping | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,1% | 0,0% | 0,0% | 0,1% |

For high QPs = {27,32,37,42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% |
|  | Test3: ‘disabled’ mapping | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% |

Related documents:

JCTVC-H0266: " CE1: Report of test results related to subtest B2 (8-bit-init) and C3 (Alt. PMU)", Heiner Kirchhoffer, Benjamin Bross, Philipp Helle, Detlev Marpe, Tung Nguyen, Mischa Siekmann, Jan Stegemann, Thomas Wiegand (Fraunhofer HHI)

Based on: JCTVC-G633: "CE1: Report of test results related to PIPE-based Unified Entropy Coding ", H. Kirchhoffer, B. Bross, A. Henkel, D. Marpe, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand (Fraunhofer HHI)

Test description: 8-bit CABAC context initialization was adopted into HM5.0. It uses a piece-wise linear interpolation when deriving an initial context model state from an initialization value and the QP. In this sub-test two alternative sets of parameters are tested for the piece-wise linear interpolation, a ‘linear’ mapping and a ‘disabled’ mapping. For each of the two mappings, a new set of context model initialization values was computed and used for the simulations.

Crosschecker - I2R & Qualcomm (JCTVC-H0076): “We have cross-checked CE1 Subtest B2 Test 1 and Test 3, studied the algorithm, and verified the coding simulation results. We also note that the current initialization method requires the use of 3 look-up tables”.

Summary of tests results:

Table 9: BD-rate results for test 1 (QP = 17, 22, 27, 32)

Table 10: BD-rate results for test 1 (QP = 22, 27, 32, 37)

Table 11: BD-rate results for test 1 (QP = 27, 32, 37, 42)

Table 12: BD-rate results for test 3 (QP = 17, 22, 27, 32)

Table 13: BD-rate results for test 3 (QP = 22, 27, 32, 37)

Table 14: BD-rate results for test 3 (QP = 27, 32, 37, 42)

### Subtest B4

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for low QPs = {17,22,27,32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for high QPs = {27,32,37,42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

Related documents:

JCTVC-H0540: "CE1: Subtest B4 - On cabac_init_idc", K. Misra, A. Segall (Sharp)

Based on: JCTVC-G716, "On CABAC Init IDC ", K. Misra and A. Segall (Sharp)

Test description: This contribution proposes modification of the cabac_init_idc indicator that is associated with CABAC initialization. In HEVC, cabac_init_idc is carried over from the H.264/AVC design but not well defined.  Specifically, the indicator appears in the HEVC WD but not in the HM software. This document proposes to define a meaning for this indicator. Compared to H.264/AVC, the proposed approach results in fewer tables available for CABAC initialization (reduced from seven to three). Additionally, the proposed approach allows different slice types to use the same table. Combination of this method with other technologies in Subtest B will be tested.

Cross-checker: Panasonic

Summary of tests results:

Table 15: Low QP Settings (QP = 17, 22, 27, 32)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class D | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class F | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |  |  |  |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.2% | 0.0% | -0.2% | -0.2% | 0.2% | 0.0% |  |  |  |
| Class E | -0.1% | 0.1% | -0.5% | -0.3% | -0.2% | -0.1% |  |  |  |
| Overall | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% |  |  |  |
|  | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% |  |  |  |
| Class F | -0.3% | -0.4% | -0.4% | -0.1% | -0.2% | -0.4% |  |  |  |

Table 16: Common conditions QP Settings (QP = 22, 27, 32, 37)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class D | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class F | -0.3% | -0.3% | -0.3% | -0.1% | -0.2% | -0.1% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | 0.0% |  |  |  |
| Class C | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% |  |  |  |
| Class D | -0.2% | 0.1% | -0.1% | -0.2% | 0.3% | 0.1% |  |  |  |
| Class E | -0.2% | 0.6% | -0.5% | -0.3% | -0.6% | 0.0% |  |  |  |
| Overall | -0.2% | 0.1% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
|  | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
| Class F | -0.3% | -0.5% | -0.7% | -0.3% | -1.1% | -0.8% |  |  |  |

Table 17: High QP Settings (QP = 27, 32, 37, 42)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class B | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class C | -0.1% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |  |  |  |
| Class D | -0.1% | -0.5% | -0.6% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
| Class F | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% | -0.2% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -0.2% | -0.4% | -0.5% |  |  |  |
| Class C | -0.2% | -0.5% | -0.3% | -0.2% | 0.2% | -0.6% |  |  |  |
| Class D | -0.2% | -0.1% | -0.1% | -0.2% | 0.2% | 0.4% |  |  |  |
| Class E | -0.2% | 0.6% | -0.4% | -0.2% | -1.2% | 0.0% |  |  |  |
| Overall | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% | -0.2% |  |  |  |
|  | -0.2% | -0.1% | -0.2% | -0.2% | -0.3% | -0.2% |  |  |  |

## Subtest C

### Subtest C1

Withdrawn.

### Subtest C2

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Canon JCTVC-H0140 | Probability update mod | -0.7% | -0.5% | -0.4% | -0.7% | -0.7% | -0.8% | -0.8% | -0.1% | -0.1% |

Related documents:

JCTVC-H0140: "CE1: Modified probability update and table removal for multi-parameter CABAC update", C. Rosewarne, M. Maeda (Canon)

Based on: JCTVC-G413 "Modified probability update and table removal for multi-parameter CABAC update", C. Rosewarne, M. Maeda (Canon)

Test description: The contribution JCTVC-G413 by Canon is built on the multi-parameter CABAC contribution JCTVC-G764 from Samsung. JCTVC-G413 replaces a 288Kb look-up table with a 6-bit by 9-bit multiplier, with the product setting a threshold for subinterval division in the arithmetic coder. Restriction of the threshold is necessary to preserve a minimum sub-interval size at extreme probabilities. This is achieved by clipping the threshold at the lower end of the threshold range and restricting the probability model update (PMU) to restrict the probability and hence the upper end of the threshold range.

Cross-checker: Samsung (JCTVC-0165) (detailed comments)

Summary of tests results:

Table 18: BD-rate results for subtest C2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -0.9% | -0.8% | -0.9% | -0.9% | -0.9% |
| Class B | -0.8% | -0.8% | -0.7% | -0.8% | -0.9% | -0.9% |
| Class C | -0.8% | -0.8% | -0.7% | -0.7% | -0.8% | -0.8% |
| Class D | -0.8% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% |
| Class E | -0.7% | -0.8% | -0.6% | -0.8% | -0.9% | -0.9% |
| Overall | -0.8% | -0.7% | -0.6% | -0.8% | -0.9% | -0.9% |
|  | -0.8% | -0.7% | -0.6% | -0.8% | -0.8% | -0.9% |
| Class F | -0.6% | -0.6% | -0.3% | -0.6% | -0.4% | -0.6% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  |  | Random Access HE |  |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U |  | V |  | Y |  | U | V |
| Class A (8bit) |  | -0.9% | -0.6% |  | -0.9% |  | -0.9% |  | -1.0% | -1.2% |
| Class B |  | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  | -0.9% | -1.0% |
| Class C |  | -0.6% | -0.6% |  | -0.4% |  | -0.6% |  | -1.0% | -1.0% |
| Class D |  | -0.5% | -0.4% |  | -0.3% |  | -0.6% |  | -0.7% | -0.9% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  | -0.9% | -1.0% |
|  |  | -0.7% | -0.6% |  | -0.4% |  | -0.7% |  | -0.9% | -1.0% |
| Class F |  | -0.5% | -0.4% |  | -0.3% |  | -0.6% |  | -0.5% | -0.7% |
| Enc Time[%] |  | 102% |  |  |  |  | 100% |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  | 100% |  |  |  |
|  |  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |  |
|  |  | Y | U |  | V |  | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.7% | -0.1% |  | -0.2% |  | -0.8% |  | -0.9% | -0.5% |
| Class C |  | -0.5% | -0.2% |  | -0.2% |  | -0.6% |  | -0.5% | -0.7% |
| Class D |  | -0.4% | -0.4% |  | 0.3% |  | -0.4% |  | 0.2% | -0.2% |
| Class E |  | -0.3% | 0.3% |  | -0.3% |  | -0.6% |  | -0.4% | -0.3% |
| Overall |  | -0.5% | -0.1% |  | -0.1% |  | -0.6% |  | -0.4% | -0.4% |
|  |  | -0.5% | -0.2% |  | -0.1% |  | -0.6% |  | -0.4% | -0.4% |
| Class F |  | -0.5% | -0.5% |  | -0.6% |  | -0.5% |  | -0.8% | -0.5% |
| Enc Time[%] |  | 103% |  |  |  |  | 102% |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  | 102% |  |  |  |
|  | Random Access HE-10 |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | U |  | V |  |  |  |  |
| Class A (8bit) | -0.9% |  |  | 0.6% |  | 0.4% |  |  |  |  |
| Class B | -0.7% |  |  | -0.6% |  | -0.5% |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.8% |  |  | -0.1% |  | -0.1% |  |  |  |  |
|  | -0.8% |  |  | 0.0% |  | -0.1% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 103% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |

### Subtest C3

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition. The size of rangeTabLPS is 16×8 in all cases.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVH0266 | Test 4 | -0.2% | -0.4% | -0.2% | -0.3% | -0.5% | -0.4% | -0.4% | -0.1% | 0.1% |
|  | new init (test 8) | -0.3% | -0.4% | -0.4% | -0.3% | -0.5% | -0.5% | -0.4% | 0.0% | 0.1% |
|  | Window size 32 (test 9) | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% | -0.3% | 0.2% | 0.2% |
|  | Window size 32 new init (test 10) | -0.3% | -0.2% | -0.1% | -0.3% | -0.1% | -0.2% | -0.3% | 0.2% | 0.1% |

Related documents:

JCTVC-H0266: "CE1: Report of test results related to subtests B2 (8-bit-init) and C3 (Alt. PMU)", H. Kirchhoffer, B. Bross, P. Helle, D. Marpe, T. Nguyen, M. Siekmann, J. Stegemann, T. Wiegand (Fraunhofer HHI)

Based on: JCTVC-G547, "Non-CE1: Counter-based probability model update with adapted arithmetic coding engine", Jan Stegemann, Heiner Kirchhoffer, Detlev Marpe, Thomas Wiegand (Fraunhofer HHI)

Test description: This contribution presents a counter based probability model update algorithm for the usage with an adapted version of the M Coder of HEVC. The proposed method is closely related to JCTVC-G764 in conjunction with a concise design of the arithmetic coding engine but retaining the R-D performance improvements of employing sophisticated probability estimation.

Cross-checker: Samsung and Qualcomm (JCTVC-0161) (detailed comments)

Summary of tests results:

Table 19: BD-rate results for subtest C3, test 8, window size 64, rangTabLPS 16×8

Table 20: BD-rate results for subtest C3, test 8, window size 64, rangTabLPS 16×8, new initialization

Table 21: BD-rate results for subtest C3, test 9, window size 32, rangTabLPS 16×8

Table 22: BD-rate results for subtest C3, test 10, window size 32 rangTabLPS 16×8, new initialization

### Subtest C4

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Samsung JCTVC-H0112 | Single parameter | -0.3% | 0.1% | 0.1% | -0.3% | 0.1% | -0.1% | -0.3% | 0.8% | 0.7% |
|  | Multi- parameter | -0.7% | -0.4% | -0.3% | -0.7% | -0.7% | -0.8% | -0.7% | -0.1% | 0.0% |

Related documents:

JCTVC-H0112: "CE1, subtest C4: Multi-parameter probability estimation for CABAC", E. Alshina, A. Alshin, J. H. Park (Samsung)

Based on: JCTVC-G764, "CE1 (subset B): Multi-parameter probability up-date for CABAC", A. Alshin, E. Alshina, J. H. Park (Samsung)

Test description: Probability up-date in CABAC is modified here to use direct calculation assuming exponential memory decay instead of LUT. Window size in exponential memory decay model is chosen as a power of 2 in order to provide division-free probability up-date. Two variants will be studied: with single and multi-parameter PMU. In single parameter model memory size is increased compared to CABAC in order to provide more accurate prediction. Multi-parameter models use “short distance probability up-date” for predetermined number of bins after entropy reset and switches to semi-sum of two predictors after that.

Cross-checker: Canon (JCTVC-H0135)

Related non-CE documents: JCTVC-H0113

Summary of tests results:

Table 23: Performance test results for one parameter probability estimation model (M=32)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.5% | -0.1% | 0.0% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | -0.4% | 0.0% | 0.0% | -0.3% | -0.3% | -0.2% |  |  |  |
| Class C | -0.4% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |  |  |  |
| Class D | -0.4% | -0.1% | -0.2% | -0.4% | -0.4% | -0.4% |  |  |  |
| Class E | -0.3% | 0.4% | 0.5% | -0.3% | -0.1% | -0.1% |  |  |  |
| Overall | -0.4% | 0.0% | 0.0% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class F | -0.2% | 0.0% | 0.2% | -0.3% | 0.1% | -0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| Class A (8bit) | -0.5% | 0.0% | -0.1% | -0.5% | -0.4% | -0.4% | -0.4% | 1.6% | 1.4% |
| Class B | -0.3% | 0.2% | 0.1% | -0.3% | 0.0% | -0.1% | -0.2% | 0.2% | 0.2% |
| Class C | -0.3% | -0.2% | 0.0% | -0.3% | -0.3% | -0.4% |  |  |  |
| Class D | -0.2% | -0.3% | 0.1% | -0.3% | -0.1% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | 0.1% | -0.3% | -0.1% | -0.3% | -0.3% | 0.8% | 0.7% |
| Class F | -0.2% | 0.1% | 0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.6% | 0.7% | -0.3% | 0.8% | 0.9% |  |  |  |
| Class C | -0.1% | 0.0% | 0.3% | -0.2% | 0.2% | 0.2% |  |  |  |
| Class D | -0.2% | 0.1% | 0.3% | -0.2% | 1.5% | 0.3% |  |  |  |
| Class E | -0.2% | 0.4% | 0.1% | -0.2% | -0.3% | -0.3% |  |  |  |
| Overall | -0.2% | 0.3% | 0.4% | -0.2% | 0.6% | 0.3% |  |  |  |
| Class F | -0.1% | 0.3% | 0.2% | -0.2% | -0.3% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 24: Performance test results for multi-parameter probability estimation model (M0=16, M1=128)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -0.8% | -0.7% | -0.9% | -0.9% | -0.8% |  |  |  |
| Class B | -0.8% | -0.7% | -0.7% | -0.8% | -0.9% | -0.9% |  |  |  |
| Class C | -0.8% | -0.7% | -0.7% | -0.7% | -0.8% | -0.8% |  |  |  |
| Class D | -0.8% | -0.4% | -0.5% | -0.7% | -0.8% | -0.8% |  |  |  |
| Class E | -0.7% | -0.8% | -0.6% | -0.8% | -0.9% | -0.8% |  |  |  |
| Overall | -0.8% | -0.7% | -0.6% | -0.8% | -0.8% | -0.8% |  |  |  |
| Class F | -0.5% | -0.6% | -0.3% | -0.6% | -0.4% | -0.6% |  |  |  |
| Enc Time[%] | 105% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| Class A (8bit) | -0.9% | -0.6% | -0.7% | -0.9% | -1.0% | -1.3% | -0.9% | 0.6% | 0.6% |
| Class B | -0.7% | -0.6% | -0.5% | -0.7% | -0.8% | -1.1% | -0.7% | -0.6% | -0.5% |
| Class C | -0.6% | -0.5% | -0.4% | -0.6% | -0.9% | -0.8% |  |  |  |
| Class D | -0.5% | -0.3% | -0.2% | -0.5% | -0.7% | -0.9% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.7% | -0.5% | -0.4% | -0.7% | -0.9% | -1.0% | -0.7% | -0.1% | 0.0% |
| Class F | -0.5% | -0.3% | -0.2% | -0.5% | -0.7% | -0.8% |  |  |  |
| Enc Time[%] | 103% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.7% | -0.3% | 0.1% | -0.8% | -0.5% | -0.5% |  |  |  |
| Class C | -0.5% | -0.3% | 0.1% | -0.5% | -0.4% | -0.8% |  |  |  |
| Class D | -0.4% | 0.0% | 0.0% | -0.4% | 0.1% | -0.5% |  |  |  |
| Class E | -0.5% | 0.4% | -0.1% | -0.6% | -1.0% | -1.2% |  |  |  |
| Overall | -0.5% | -0.1% | 0.0% | -0.6% | -0.4% | -0.7% |  |  |  |
| Class F | -0.5% | -0.1% | -0.1% | -0.5% | -1.6% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

## Subtest D

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included):

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Nokia JCTVC-H0232 | D1: normal QPs | 4.4% | 2.8% | 3.1% | 3.8% | 5.5% | 5.6% | 6.2% | 2.0% | 3.7% |
|  | D1: low QPs (1,5,9,13) | 7.7% | 7.9% | 8.2% | 5.7% | 5.4% | 5.2% | 9.0% | 10.5% | 10.1% |
| Qualcomm JCTVC-H0536 | D2 | 1.3% | 0.6% | 0.8% | 1.5% | 0.3% | 0.5% | 1.7% | 1.9% | 2.2% |

### Subtest D1

JCTVC-H0232: "CE1.D1: Nokia report on high throughput binarization", J. Lainema, K. Ugur, and A. Hallapuro (Nokia)

Based on: JCTVC-G569, "Single entropy coder for HEVC with a high throughput binarization mode", J. Lainema, K. Ugur and A. Hallapuro (Nokia)

Test description: The aim of JCTVC-G569 is to increase throughput of the CABAC coefficient coding. In high throughput mode, it utilizes CABAC bypass mode for all transform coefficient data and applies HM 4.0 CAVLC coefficient coding engine as the binarization method. In addition to the high throughput mode, the proposed method supports full HM CABAC coding engine, referred to as high efficiency binarization mode. There is a slice level indication identifying which binarization scheme is to be applied.

Crosschecker – Cisco (JCTVC-H0212)

Related non-CE documents: JCTVC-H0233, JCTVC-H0458, JCTVC-H0510, JCTVC-H0554

Summary of tests results

Table 25: BD-rate results for normal QPs {22, 27, 32, 37}

Table 26: BD-rate results usin low QPs {1, 5, 9, 13}

### Subtest D2

JCTVC-H0536: "CE1 D2: Adaptive initialization for CABAC with fixed probability contexts", L. Guo, J. Sole, X. Wang, and M. Karczewicz (Qualcomm)

Based on: JCTVC-G833, "Non-CE1: Adaptive initialization for CABAC with fixed probability contexts”, L. Guo, J. Sole, X. Wang, M. Karczewicz (Qualcomm)

Test description: JCTVC-G833 proposes an adaptive initialization method for CABAC with fixed probability contexts. At the beginning of each slice, a context can be initialized in one of two ways. In one case, a context is initialized using pre-defined (m,n) values (i.e., original HM method), or initialized using a state value selected by the encoder. For each context, the encoder signals the selection of initialization method to the decoder as well as the new state value (if the second method is selected). No probability/state update is performed in the encoding/decoding process.

Crosschecker - RIM (JCTVC-H0652)

Summary of tests results:

Table 27: BD-rate results for subtest D2