| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 |  |  | Document: JCTVC-H0032 |  |
| --- | --- | --- | --- | --- |
| Title: | CE2: Summary report of Core Experiment on Chroma RQT depth |  |  |  |
| Status: | Input Document |  |  |  |
| Purpose: | Core Experiment Summary |  |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto(primary) Xun Guo Xianglin Wang |  |  | Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp xun.guo@mediatek.com xianglin@qualcomm.com |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 2, Chroma RQT depth. Eight organizations have been registered in CE2, and four tools have been evaluated.

# Introduction

In the current HM/WD, the transform depth of corresponding luma block is simply reused for chroma, i.e., luma and chroma components share the same transform depth. However, since chroma components generally have smoother textures than luma, it may not appropriate to always apply the same RQT depth of luma on chroma. To resolve this issue, Core Experiment 2 (CE2) [1] was established at the last Geneva meeting, and three proposals [2]-[4] were included in the description of CE2. In addition, a harmonized solution of the three proposals is defined.

The objectives of this CE is to investigate the impact of limiting chroma RQT depth on the coding efficiency and complexity.

# Participants List

| Company | Name | Email | Proponent | Crosschecker |
| --- | --- | --- | --- | --- |
| HHI | Philipp Helle | philipp.helle@hhi.fraunhofer.de |  |  |
| HHI | Simon Oudin | simon.oudin@hhi.fraunhofer.de |  |  |
| HHI | Benjamin Bross | benjamin.bross@hhi.fraunhofer.de |  |  |
| HiSilicon | Xiaozhen Zheng | zhengxiaozhen@huawei.com |  | X |
| KAIST | Bumshik Lee | lbs@kaist.ac.kr |  | X |
| MediaTek | Xun Guo | xun.guo@mediatek.com | X | X |
| MediaTek | Xin Zhao | x.zhao@mediatek.com | X | X |
| MediaTek | Xiang Li | x.li@mediatek.com | X | X |
| MediaTek | Shawmin Lei | shawmin.lei@mediatek.com | X | X |
| Mitsubishi | Kazuo Sugimoto | Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp | X | X |
| Mitsubishi | Akira Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp | X | X |
| NHK | Atsuro Ichigaya | ichigaya.a-go@nhk.or.jp | X |  |
| Panasonic | Youji Shibahara | shibahara.youji@jp.panasonic.com |  | X |
| Qualcomm | Xianglin wang | xianglin@qualcomm.com | X | X |
| Qualcomm | Liwei Guo | liweig@qualcomm.com | X | X |

# Tools under Test

The following tools are tested:

| test | Proposal Document | Proponent | Tool description | Cross-checker & Document # |
| --- | --- | --- | --- | --- |
| 4.1 JCTVC-G283 | JCTVC-H0317 | MediaTek PKU | - Signaling Max chroma RQT depth for intra - Chroma TU split for inter same as done in luma with max depth=1 - no 32x32 transform for chroma | Qualcomm JCTVC-H0543 |
| 4.2 JCTVC-G442 | JCTVC-H0235 | Mitsubishi NHK | - Implicit chroma RQT depth depending on intra and inter PU mode - allow 32x32 transform for chroma | MediaTek JCTVC- H0319 |
| 4.3 JCTVC-G980 | JCTVC- H0538 | Qualcomm | - Signaling Max chroma RQT depth for intra - Chroma TU split for inter same as done in luma with max depth=1 - allow 32x32 transform for chroma | Mitsubishi JCTVC- H0236 |
| 4.4.0 | JCTVC- H0308 | Harmonization | - Signaling Max chroma RQT depth for intra and inter - no 32x32 transform for chroma | HiSilicon JCTVC- H0258 |
| 4.4.1 | JCTVC- H0308 | Harmonization | - Signaling Max chroma RQT depth for intra and inter - allow 32x32 transform for chroma | Panasonic JCTVC-H0122 |

# Example of Chroma TU tree

The following examples show chroma TU tree for each test setting, with the LCU size of 64x64.

In the figure,

- "Intra/inter depth=1" means "MaxDepth_intra/inter_chroma=0".

- "Intra/inter depth=2" means "MaxDepth_intra/inter_chroma=1".

- Black lines indicate CU boundaries.

- Red lines indicate TU boundaries.

- Case 1 shows an example with typical CU/TU trees.

- Case 2 shows an example with 64x64 CU without any explicit TU splits.

2Nx2N

Intra

NxN

Intra

2Nx2N

Inter

2NxN

Inter

2Nx2N

Inter

NxN

Inter

Nx2N

Inter

2Nx2N

Inter

2Nx2N

Intra

NxN

Intra

Luma CU/TU tree

Chroma CU/TU tree

HM5

Harmonization

Intra depth = 1

Inter depth = 1

G442

Harmonization

Intra depth = 1

Inter depth = 2

Case 1

Luma CU/TU tree

Luma CU/TU tree

Chroma CU/TU tree

HM5

Harmonization with 32x32

Intra depth = 1

Inter depth = 1

G442

Harmonization with 32x32

Intra depth = 1

Inter depth = 2

Harmonization w/o 32x32

Intra depth = 1

Inter depth = 1

Harmonization w/o 32x32

Intra depth = 1

Inter depth = 2

Case 2

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

The objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate [5] compared with default configuration of HM-5.0 with common test conditions defined in [6] are used.

## Complexity Measurements

For the complexity measurement, the reference software and the reference software with the proposed method implemented are executed on the same machine and the computational time are measured. A time factor are then be calculated between the reference software and the reference software with the proposed method implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft.

# Software and configurations used for each test

As subtest 4.1 and 4.3 can be configured using the software used for subtest 4.4, a common software implementation was used for subtests 4.1, 4.3 and 4.4 with configurations shown below. For subtest 4.2, individual implementation was made for testing.

| test | Software | Chroma RQT depth for intra | Chroma RQT depth for inter | 32x32 TU for chroma |
| --- | --- | --- | --- | --- |
| 4.1 | Harmonized solution on top of HM-5.0 | 1 | 1 | not available |
| 4.3 |  | 1 | 1 | available |
| 4.4.0 |  | 1 | 2 | not available |
| 4.4.1 |  | 1 | 2 | available |
| 4.2 | G442 on top of HM-5.0 | 1 | 1 or 2 depends on PU mode | available |

# Summary of Results

Average BD-rate gain for each test is shown below.

| test | High Efficiency |  |  |  |  |  | Low Complexity |  |  |  |  |  | High 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Intra |  | Random Access |  | Low Delay |  | Random Access |  |
|  | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV |
| 4.1 | 0.02 | -2.47 | -0.05 | -1.32 | -0.15 | -0.02 | 0.02 | -1.92 | -0.01 | -1.39 | -0.02 | -0.93 | 0.02 | -2.59 |
| 4.2 | -0.05 | -2.46 | -0.06 | -1.43 | -0.07 | -1.14 | 0.02 | -2.24 | -0.01 | -1.59 | -0.03 | -2.16 | -0.02 | -4.45 |
| 4.3 | -0.05 | -2.46 | -0.09 | -1.35 | -0.10 | -0.53 | 0.02 | -2.24 | -0.01 | -1.72 | -0.05 | -2.13 | -0.03 | -4.08 |
| 4.4.0 | 0.02 | -2.47 | -0.04 | -1.61 | -0.02 | -0.92 | 0.02 | -1.92 | 0.00 | -1.40 | -0.01 | -1.11 | -0.01 | -2.93 |
| 4.4.1 | -0.05 | -2.46 | -0.05 | -1.67 | -0.04 | -1.21 | 0.02 | -2.24 | 0.00 | -1.61 | -0.02 | -1.38 | -0.01 | -4.29 |

Average run-time for encoding and decoding compared with the anchor are shown below.

| test | High Efficiency |  |  |  |  |  | Low Complexity |  |  |  |  |  | High 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Intra |  | Random Access |  | Low Delay |  | Random Access |  |
|  | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC |
| 4.1 | 100% | 99% | 97% | 100% | 98% | 100% | 101% | 99% | 98% | 100% | 99% | 100% | 98% | 100% |
| 4.2 | 99% | 99% | 96% | 100% | 97% | 100% | 100% | 99% | 97% | 100% | 98% | 100% | 96% | 100% |
| 4.3 | 99% | 100% | 97% | 100% | 97% | 100% | 98% | 100% | 98% | 100% | 98% | 100% | 97% | 99% |
| 4.4.0 | 99% | 100% | 98% | 100% | 98% | 100% | 99% | 100% | 98% | 100% | 99% | 100% | 97% | 99% |
| 4.4.1 | 98% | 100% | 99% | 100% | 99% | 99% | 98% | 100% | 99% | 100% | 99% | 99% | 98% | 100% |

# References

- K. Sugimoto, X. Guo and X. Wang, “Description of Core Experiment 2 (CE2): Chroma RQT depth”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1202, 7th Meeting: Geneva, CH, November, 2011.

- X. Zhao, X. Guo, X. Li, S. Lei, S. Ma and W. Gao, “Residue Quad Tree Depth for Chroma in Intra coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G283, 7th Meeting: Geneva, CH, November, 2011.

- A. Minezawa, K. Sugimoto, S. Sekiguchi, A. Ichigaya and S. Sakaida, “Improvement to chroma TU specification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G442, 7th Meeting: Geneva, CH, November, 2011.

- L. Guo, X. Wang and M. Karczewicz, “Evaluation of Limiting Chroma Transform Depth in RQT on HM4.0”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G980, 7th Meeting: Geneva, CH, November, 2011.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.