| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0034 |
| --- | --- |

| Title: | CE4: Summary report of Core Experiment on Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | K.Sato (Sony) M. Budagavi (TI) M. Coban (Qualcomm) H.Aoki (NEC) X.Li (MediaTek) | Email: | Kazushi.Sato@jp.sony.com madhukar@ti.com mcoban@qualcomm.com hao@bu.jp.nec.com x.li@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document reports results related to Core Experiment 4: Quantization.

This core experiment contains following subtests:

- Subtest 1: QP Coding

- Subtest 2: Quantization Matrices

- Subtest 3: QP signalling Unit Granularity

- Subtest 4: Fine Granularity of Quantization Step

# Subtest 1: QP Coding

## Related Contributions

| Test | Description | Proposal | Cross-Check |
| --- | --- | --- | --- |
| 1.1 | prevQP | Sony: JCTVC-H0186 | NEC: JCTVC-H0054 |
| 1.2 | if(left available) leftQP | NEC: JCTVC-H0054 | Sony: JCTVC-H0186 |
| 1.3 | Intra: depends on ipred mode Inter: ave (leftQP, topQP) | NEC/Canon/Qualcomm/Sony: JCTVC-H0053 | JVC Kenwood: JCTVC-H0205 |
| 1.4 | Weighted ave (leftQP, topQP) | JVCKenwood: JCTVC-H0197 | HiSilicon: JCTVC-H0254 |

## Summary of Results

QP_adaptation is applied for this test with options

-aq 1 -aqr 12

Subtest 1.1 is the anchor of this subtest. Summary of the results with MaxCuDQPDepth = 3 are shown in Table 1.2.1, 1.2.2 and 1.2.3 respectively.

Table 1.2.1: subtest 1.2

Table 1.2.2: Subtest 1.3

Table 1.2.3: Subtest 1.4

## Discussions

At the last meeting it was suggested that prevQP is used at LCU boundaries for rate-control friendliness.

Under this condition, HM-method and proposed methods have been evaluated based on coding efficiency. It is recommended that the proposed method be discussed together with Non-CE4 proposals as follow both in terms of coding efficiency and complexity:

JCTVC-H0189: Non-CE4: Low-complexity dQP Coding

JCTVC-H0204: Non-CE4: Compatible QP prediction with RC and AQ

JCTVC-H0289: Non-CE4: Spatial QP prediction based on motion vector predictors (AMVP)

(JCTVC-H0226: On derivation of luma QP predictor for Wavefront Parallel Processing)

# Subtest 2: Quantization Matrices

## Related Contributions

| Test | Description | Proposal | Cross-Check |
| --- | --- | --- | --- |
| 2.1 | - Prediction (copy, up/down-samplin, DPCM 135-degree processing) - Upsampling w/o residue transmission - Signed/unsigned exp-golomb coding | TI/Sony: JCTVC-H0230 | Canon: JCTVC-H0150 HiSilicon: JCTVC-H0303 |
| 2.2 | - Mapping from smaller QM to larger QM | MediaTek: JCTVC-H0384 | Canon: JCTVC-H0151 HiSilicon: JCTVC-H0306 |
| 2.3 |  | Withdrawn | Withdrawn |
| 2.4 | - Layered coding | HiSilicon: JCTVC-H0314 | Canon: JCTVC-H0152 Sony: JCTVC-H0231 |
| 2.5 | - Raster scanning - Prediction based on top and left neighbors | Qualcomm: JCTVC-H0451 | Canon: JCTVC-H0153 HiSilicon: JCTVC-H0307 |

## Summary of Results

Proposals in this subtest are evaluated based on compression ratio (generated QM bits) and error in QM domain. Table 2.2.1 is a summary of each proposal.

“symmetriy*”and “asymmetry*” stand for matrices to be tested that have been provided by the proponents. In this table generated bits for quantization matrices and error in QM domain are shown.

Table 2.2.1

## Discussions

During the e-mail discussions it has been pointed out that relation between error in QM domain and its impact on subjective quality is not clear. Therefore subjective quality comparison is needed.

It is recommended to

- conduct subjective viewing session to compare lossless and lossy compression methods

- hold BoG to evaluate complexity of each proposal (including buffer requirement) both with lossless and lossy

- conclude this subtest.

Suggested conditions for subjective viewing (needed to be discussed) are:

Intra_LC

Sequence: Kimono, BQMall, Vidyo3

QP: 32/37

Matrix: symmetry1, symmetry6, asymmetry3

# Subtest 3: QP Signaling Unit Granularity

## Related Contributions

| Test | Description | Proposal | Cross-Check |
| --- | --- | --- | --- |
| 3.1.a | - TU-based dQP signalling | Motorola Mobility: JCTVC-H0506 | NEC: JCTVC-H0184 |
| 3.1.b | -TU-level dQP signaling with PU-level constraint | Motorola Mobility: JCTVC-H0508 | NEC: JCTVC-H0191 |
| 3.2 | - TU-size-based dQP signalling | Samsung: JCTVC-H0260 | Motorola Mobility: JCTVC-H0518 |

## Summary of Results

[3.1]

CE4 subtest 1.1 (anchor of subtest 1) is used as anchor. Summary of the results on 3.1.a and 3.1.b are shown in Table 3.2.1.1 and in Table 3.2.1.2, respectively.

Table 3.2.1.1

Table 3.2.1.2

[3.2]

HM-5.0 with common test condition is used as anchor. Summary of the result is shown in Table 3.2.2.1.

Table 3.2.2.1

## Discussions

[3.1]

The purpose of the proposed method is not coding efficiency improvement but finer spatial granularity of QP coding for better rate- or quality-controllability. It is recommended to discuss if this proposal be adopted on top of Sub-LCU-level QP coding already included in WD.

[3.2]

The proposed method improves coding efficiency of Class F sequences. It is recommended to discuss if this proposal be recommended to adopt.

# Subtest 4: Fine Granularity of Quantization Step

## Related Contributions

| Test | Description | Proposal | Cross-Check |
| --- | --- | --- | --- |
| 4.1 | Finer-QP step size | Samsung: JCTVC-H0336 | MediaTek: JCTVC-H0318 |
| 4.2 |  | Withdrawn | Withdrawn |

## Summary of Results

Summary of the result is shown in Table 4.2.1. In the table “dBitrate” stands for bit increase with QP+1 over QP. Smaller dBitrate means that finer rate control is possible.

Table 4.2.1

|  | Results | AIHE | AILC | RAHE | RALC | RAHE-10 | LBHE | LBLC | Aver |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HM-5 Anchor QP+1 | BD-rate-Y | -0.3% | -0.3% | 0.2% | 0.2% | 0.4% | 0.3% | 0.3% | 0.1% |
|  | BD-rate-U | 2.7% | 2.7% | -0.4% | 0.5% | -0.1% | -2.5% | -2.6% | 0.0% |
|  | BD-rate-V | 2.8% | 2.8% | -0.2% | 0.5% | -0.1% | -2.3% | -2.2% | 0.2% |
|  | dBitRate | -11.2% | -10.6% | -17.7% | -16.8% | -16.2% | -19.6% | -18.7% | -15.8% |
|  | Enc. Time | 99% | 98% | 98% | 99% | 98% | 98% | 99% | 98% |
|  | Dec. Time | 99% | 97% | 98% | 98% | 98% | 98% | 99% | 98% |
| Proposal dQP_scale1 | BD-rate-Y | 0.6% | 0.8% | 0.6% | 0.6% | 0.7% | 0.6% | 0.5% | 0.6% |
|  | BD-rate-U | -5.0% | -5.0% | -6.7% | -6.8% | -7.7% | -7.8% | -7.8% | -6.7% |
|  | BD-rate-V | -5.1% | -5.0% | -6.7% | -6.9% | -7.9% | -7.7% | -7.9% | -6.7% |
|  | dBitRate | -4.9% | -4.8% | -8.5% | -8.0% | -8.0% | -9.7% | -9.2% | -7.6% |
|  | Enc. Time | 100% | 99% | 99% | 100% | 100% | 99% | 100% | 100% |
|  | Dec. Time | 99% | 99% | 99% | 99% | 100% | 100% | 101% | 99% |

## Discussions

If fine granularity of quantization step is considered, it is recommended to review the proposal. The proposal supports tradeoff between dQP overhead and fine rate qualrity control by adaptively changing dQP scale in the direct extension of the current QP range where the quantization step size is the half of the current quantization step size.

# References

[1] “Description of Core Experiment 4: Quantization”, JCTVC-G1204, Nov 2011

[2] “Common HM test conditions and software reference configurations”, JCTVC-G1200, Nov 2011