| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 Feburary, 2012 | Document: JCTVC-H0038_r2 WG11 Number: m23834 |
| --- | --- |

| Title: | CE8: Summary report of Core Experiment on non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 In Suk Chong2 Matthias Narroschke3 Yu-Wen Huang4 |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com yuwen.huang@mediatek.com |
| Source: | CE coordinators (Toshiba1, Qualcomm2, Panasonic3, MediaTek4) |  |  |

_____________________________

# Introduction

A summary of core experiment 8 (CE8) on non-deblocking loop filtering is reported. There are six Subtests in CE8, Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering (four proposals), Subtest b: ALF simplification (two proposals), Subtest c: Necessity of sign bits for SAO offsets (one proposal), Subtest d: SAO simplification and line buffer reduction (two proposals), Subtest e: SAO improvement (five proposals), Subtest f: Subjective picture quality assessment of ALF. These are evaluated based on the common test conditions in JCTVC-G1200 and additional conditions in JCTVC-G1208. All mandatory results are verified by cross-checkers.

Some formal subjective viewing on CE8.f was conducted prior to the official San Jose meeting on Jan 31.

An informal subjective viewing will be conducted at “Silicon Valley” on the first day of the official meeting on Feb 1. The detailed information will be announced during the meeting.

(Please refer to the attached subjective viewing report.)

# Document list and cross-check assignment

- Summary document (this document)

  - JCTVC-H0038 [CE coordinators] CE8: Summary report of Core Experiment on non-deblocking loop filtering

- Proponent's and verification/study documents

Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering

  - JCTVC-H0272 [MediaTek/Qualcomm/Toshiba] CE8.a.1: One-stage SAO and ALF with LCU-based syntax

    - Cross-check: JCTVC-H0102 [Sharp/Samsung]

  - JCTVC-H0066 [Cisco] CE8.a.2: ALF with LCU-based syntax

    - Cross-check: JCTVC-H0170 [Toshiba]

    - Cross-check: JCTVC-H0586 [Orange]

  - JCTVC-H0273 [MediaTek/Qualcomm/Samsung] CE8.a.3: SAO with LCU-based syntax

    - Cross-check: JCTVC-H0103 [Sharp]

  - JCTVC-H0274 [MediaTek/Qualcomm/Toshiba] CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntax

    - Cross-check: JCTVC-H0408 [Cisco], JCTVC-H0371 [Panasonic], JCTVC-H0422 [Samsung]

Subtest b: ALF simplification

  - JCTVC-H0413 [Samsung/Sony] CE8.b.1: ALF BA feature computation at upper-left 4 pixels in a 4×4 block

    - Cross-check: JCTVC-H0385 [Qualcomm]

  - JCTVC-H0068 [TI] CE8.b.2: ALF with single filter type

    - Cross-check: JCTVC-H0416 [Samsung]

Subtest c: Necessity of sign bits for SAO offsets

  - JCTVC-H0434 [TI] CE8.c.1: Necessity of Sign Bits for SAO Offsets

    - Cross-check: JCTVC-H0331 [Samsung/MediaTek]

Subtest d: SAO simplification and line buffer reduction

  - JCTVC-H0436 [TI] CE8.d.1: Reduced Number of Band Offsets in SAO

    - Cross-check: JCTVC-H0332 [MediaTek]

  - JCTVC-H0375 [Panasonic] CE8.d.2: SAO Line Buffer Reduction

    - Cross-check: JCTVC-H0334 [MediaTek]

Note: CE8.e.2 (JCTVC-H0174) can also be SAO band offset reduction.

Subtest e: SAO improvement

  - JCTVC-H0276 [MediaTek/Qualcomm] CE8.e.1: Offset coding in SAO

    - Cross-check: JCTVC-H0173 [Canon]

  - JCTVC-H0174 [Canon] CE8.e.2: Results related to section e.2

    - Cross-check: JCTVC-H0229 [Ericsson]

  - JCTVC-H0223 [Ericsson/Panasonic] CE8.e.3: Modified SAO edge offsets

    - Cross-check: JCTVC-H0390 [Qualcomm]

  - JCTVC-H0557 [Motorola Mobility] CE8.e.4: Selection and Coding of SAO Parameters

    - Cross-check: JCTVC-H0335 [MediaTek]

  - JCTVC-H0225 [Ericsson/Canon/Panasonic] CE8.e.5: Combination of CE8.e.2 and CE8.e.3

    - Cross-check: JCTVC-H0337 [MediaTek]

Subtest f: Subjective picture quality assessment of ALF

  - JCTVC-H0714 [CE8(Sony/TI/BBC/Broadcom/FUB/Motorola Mobility/NTT DOCOMO)] CE8.f: ALF subjective testing results

  - JCTVC-H0622 [Sony] CE8.f : Crosscheck report on objective evaluation of visual testing

# Experimental results

Based on the common test conditions in JCTVC-G1200, CE8 specified test conditions as described in JCTVC-G1208. Results of the CE will be evaluated on the basis of BD-measures and/or complexity.

## Subtest 0: Number of passes for designing ALF coefficients

See AHG6 summary report.

## Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering

### Modification points and results

The following table summarizes the modification points of specification from WD5 in red character:

|  | CE8.a.1 | CE8.a.2 | CE8.a.3 | CE8.a.4-1 | CE8.a.4-2 | CE8.a.4-3 (*) |
| --- | --- | --- | --- | --- | --- | --- |
| ALF Coef. Signalling | APS/Slice H | LCU | APS (HM-5.0) | APS/Slice H | APS/Slice H | APS/Slice H |
| ALF CU Control Signalling | Slice H | LCU | Slice H (HM-5.0) | Slice H | Slice H | Slice H |
| ALF Coef. Reuse from Coded Coef. | Yes | Yes | No (HM-5.0) | Yes | Yes | Yes |
| ALF Coef. Quant. | Fixed | Adaptive | Fixed (HM-5.0) | Adaptive | Adaptive | Adaptive |
| ALF Num. of Coeff. | 9 | 9 | 9 (HM-5.0) | 9 | 9 | 9 |
| ALF Coeff. Coding | VLC | CABAC | VLC (HM-5.0) | VLC | VLC | VLC |
| ALF Filter Shapes | 9x9 Cross, 5x5 Star | 9x9 Cross, 5x5 Star | 9x9 Cross, 5x5 Star (HM-5.0) | 9x9 Cross, 5x5 Star | 9x9 Cross, 5x5 Star | 9x9 Cross, 5x5 Star |
| ALF Chroma Filter | Independent of Luma | Reuse from Luma | Independent of Luma (HM-5.0) | Independent of Luma | Independent of Luma | Independent of Luma |
| SAO syntax | LCU-based | Picture-based | LCU-based | LCU-based | LCU-based | Picture-based |
| SAO/ALF Config. | One-stage | Two-stage | Two-stage | One-stage | Two-stage | Two-stage |

(*) The purpose of experiment is to compare CE8.a.2 and CE8.a.4 using picture based SAO.

The following table summarizes the modification points of the encoding algorithm from HM5 in red character:

| Idx | Proposal | SAO Encoding Algorithm |  |  | ALF Encoding Algorithm |  |  | ALF Coef. Derivation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Picture-based, single-slice | LCU-based, single-slice | LCU-based, multi-slice | Picture-based, single-slice | LCU-based, single-slice | LCU-based, multi-slice | 1-pass | RDOQ-like |
| 1 | CE8.a.1P | X (OSALF) |  |  | X (OSALF) |  |  | X |  |
| 2 | CE8.a.1L |  | X (OSALF) |  |  | X (OSALF) |  | X |  |
| 3 |  |  |  | X (OSALF) |  |  | X (OSALF) | X |  |
| 4 | CE8.a.2L | X |  |  |  | X |  |  | X |
| 5 |  | X |  |  |  |  | X |  | X |
| 6 | CE8.a.3P | X |  |  | X |  |  | X |  |
| 7 | CE8.a.3L |  | X |  |  | X |  | X |  |
| 8 |  |  |  | X |  |  | X | X |  |
| 9 | CE8.a.4-1P | X (OSALF) |  |  | X (OSALF) |  |  | X |  |
| 10 | CE8.a.4-1L |  | X (OSALF) |  |  | X (OSALF) |  | X |  |
| 11 |  |  |  | X (OSALF) |  |  | X (OSALF) | X |  |
| 12 | CE8.a.4-2P | X |  |  | X |  |  | X |  |
| 13 | CE8.a.4-2L |  | X |  |  | X |  | X |  |
| 14 |  |  |  | X |  |  | X | X |  |
| 15 | CE8.a.4-3P (*) | X |  |  | X |  |  | X |  |
| 16 | CE8.a.4-3L (*) | X |  |  |  | X |  | X |  |
| 17 |  | X |  |  |  |  | X | X |  |

(*) The purpose of experiment is to compare CE8.a.2 and CE8.a.4 using picture based SAO.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for single-slice encoding in HE and HE10 configurations (without Class F):

| Index | Proposal | (Encoding) | AI-HE | RA-HE | RA-HE10 | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CE8.a.1P (Table 1) | (Picture) | 0.0 108 101 | 0.0 102 99 | 0.5 99 93 | 0.2 101 101 | 0.3 102 101 |
| 9 | CE8.a.4-1P (Table 1) | (Picture) | 0.0 109 102 | 0.1 101 100 | 0.5 99 90 | 0.2 101 102 | 0.2 101 103 |
| 12 | CE8.a.4-2P (Table 4) | (Picture) | 0.1 91 100 | 0.3 97 96 | 0.4 97 91 | 0.5 98 98 | 0.4 97 97 |
| 4 | CE8.a.2L | (LCU) (**) | 0.3 71 96 | 0.6 92 95 | 1.0 91 91 | 0.2 94 97 | 0.0 92 96 |
| 15 | CE8.a.4-3P (Table 7) (*) | (Picture) | 0.1 81 102 | 0.2 91 97 | 0.3 92 88 | 0.1 90 99 | 0.1 91 99 |
| 2 | CE8.a.1L (Table 2) | (LCU) | 0.9 106 93 | 0.9 104 97 | 1.8 100 88 | 1.5 103 92 | 2.0 104 91 |
| 10 | CE8.a.4-1L (Table 2) | (LCU) | 0.8 105 95 | 0.9 103 98 | 1.7 100 88 | 1.4 103 93 | 1.8 103 93 |
| 13 | CE8.a.4-2L (Table 5) | (LCU) | 0.5 86 95 | 1.0 98 96 | 1.5 94 87 | 1.3 98 92 | 1.6 97 91 |
| 4 | CE8.a.2L | (LCU) | 0.3 71 96 | 0.6 92 95 | 1.0 91 91 | 0.2 94 97 | 0.0 92 96 |
| 16 | CE8.a.4-3L (Table 8) (*) | (LCU) | 0.4 73 96 | 0.6 91 95 | 1.0 91 87 | 0.6 94 93 | 0.6 92 93 |
| 6 | CE8.a.3P (Table 1) | (Picture) | 0.0 98 101 | 0.1 102 99 | 0.1 102 102 | 0.3 99 100 | 0.3 102 98 |
| 7 | CE8.a.3L (Table 2) | (LCU) | 0.0 98 101 | 0.4 98 100 | 0.5 102 101 | 0.7 98 98 | 0.7 102 99 |

(*)	 The purpose of experiment is to compare CE8.a.2 and CE8.a.4 using picture based SAO.

(**) CE8.a.2 can hypothetically provide picture based encoding. However, the picture-based encoding has not yet been supported in CE8.a.2 software. Therefore, its LCU-based results are copied to this column for comparison.

Note: Subjective viewing will not be conducted indices highlighted in yellow.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for single-slice encoding in LC configuration (without Class F):

| Index | Proposal | (Encoding) | AI-LC | RA-LC | LDB-LC | LDP-LC |
| --- | --- | --- | --- | --- | --- | --- |
| N/A | Reference (SAO-off vs SAO-on in HM 5.0) | (Picture) | -0.7 104 107 | -1.2 100 103 | -2.2 101 104 | -7.9 101 105 |
| 6 | CE8.a.3P (Table 1) | (Picture) | -0.6 102 104 | -1.0 100 104 | -1.8 101 104 | -7.6 100 105 |
| 7 | CE8.a.3L (Table 2) | (LCU) | -0.6 104 104 | -0.8 99 105 | -1.6 101 104 | -7.0 100 104 |

Note: Subjective viewing will not be conducted indices highlighted in yellow.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for multi-slice encoding in HE configurations (without Class F):

| Index | Proposal | (Encoding) | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- |
| 3 | CE8.a.1L (Table 4) | (LCU) | -0.7 N/A N/A | -1.4 N/A N/A |
| 11 | CE8.a.4-1L (Table 13) | (LCU) | -0.9 104 108 | -1.7 107 108 |
| 14 | CE8.a.4-2L (Table 14) | (LCU) | -0.7 N/A N/A | -1.4 N/A N/A |
| 5 | CE8.a.2L | (LCU) | -2.2 101 111 | -3.6 101 115 |
| 17 | CE8.a.4-3L (Table 9) (*) | (LCU) | -1.9 105 114 | -3.0 106 115 |
| 8 | CE8.a.3L (Table 5) | (LCU) | -1.0 100 100 | -1.5 100 100 |

(*)	 The purpose of experiment is to compare CE8.a.2 and CE8.a.4 using picture based SAO.

Note: Anchor for multi slice encoding is “SAO on + ALF off” (except for index 8, which uses “SAO off + ALF off” as the anchor).

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for multi-slice encoding in LC configuration (without Class F):

| Index | Proposal | (Encoding) | LDB-LC | LDP-LC |
| --- | --- | --- | --- | --- |
| 8 | CE8.a.3L (Table 5) | (LCU) | -1.3 -5.3 -6.6 100 101 | -6.8 -7.3 -8.3 100 101 |

Note: Anchor for multi slice encoding is “SAO off + ALF off”.

### Comments from cross-checkers

To CE8.a.1 (JCTVC-H0272, MediaTek/Qualcomm/Toshiba):

JCTVC-H0102 (Sharp/Samsung) :

The coding results perfectly match with those provided by proponents. The current HM5.0’s ALF uses picture level area classification, so it is difficult to apply in low latency encoding. The evaluated method named LCU base syntax, enables both picture-base encoding and LCU-base encoding, which is applicable in low latency encoding.

To CE8.a.2 (JCTVC-H0066, Cisco):

JCTVC-H0170 (Toshiba) :

Coding efficiency results are matched with the proponent’s results. Relative decoding time is similar to result of proponent’s (within 3% discrepancy). However, relative encoding time differs from result of proponent’s (up to 14% slower than proponents’ results for AI). We could not identify the reason, but since it is AI which can encoder faster than the other coding conditions, the difference of the environments might affect the results. In JCTVC-H0066, a LCU based ALF syntax and technique are proposed.  The proposed scheme can provide sub-frame encoder delay by signalling one single set of filter coefficients for each LCU. In each LCU, it is adaptively decided whether using one of the previous filter coefficients sets or signalling new coefficients. In addition, adaptive codebook technique to specify the previous filter, modification to quantization accuracy of filter coefficients and coefficient prediction are introduced. Filter coefficients are commonly used for luma and chroma, where chorma filtering can only be applied when luma filtering is applied.

JCTVC-H0586 (Orange) :

Verified coding efficiency results are strictly identical to the ones measured by the proponents. Furthermore, the source code has been studied and matches the description of JCTVC-H0066.

To CE8.a.3 (JCTVC-H0273 MediaTek/Qualcomm):

JCTVC-H0103 (Sharp) :

The coding results perfectly match with those provided by proponents. The current HM5.0’s SAO uses quad-tree partitioning at picture-level, so it is difficult to apply in low latency encoding. The evaluated method named LCU base syntax, enables both picture-base encoding and LCU-base encoding, which is applicable in low latency encoding.

- The merge process is summarized as the following

  - If row merge is used, some consecutive LCUs parameters are copied from upper LCU

    - the number of consecutive LCUs is derived from upper LCU parameters.

  - Otherwise, if left merge is used, the consecutive LCUs parameter are copied from left LCU

    - the number of consecutive LCUs is derived a syntax.

  - Otherwise, if up merge is used, the LCU parameter is copied from upper LCU

  - Otherwise, the LCU parameter is decoded from bitstream.

To CE8.a.4 (JCTVC-H0274, MediaTek/Qualcomm/Toshiba):

JCTVC-H0408 (Cisco) :

The following experiments/tables of JCTVC-H0274 were cross-checked:

- OSALF with LCU-based syntax, picture-based encoding, single-slice per picture, and anchor using JCTVC-G1200 (Table 1)

- OSALF with LCU-based syntax, LCU-based encoding, single-slice per picture, and anchor using JCTVC-G1200 (Table 2)

- OSALF with LCU-based syntax, LCU-based encoding, multi-slice per picture, and anchor using JCTVC-G1200 with multi-slice per picture, SAO-on, and ALF-off (Table 13)

The cross-check results match those of document JCTVC-H0274. The timing results are also comparable, considering the variations of the clusters.

Also, the test results of Table 3 are identical to those of Table 13, but with a different anchor (SAO-off, and ALF-off). It has been verified that the test results (right-hand side) of the Table 3 and Table 13 spreadsheets are identical

JCTVC-H0371 (Panasonic) :

Due to the request from the proponents, parts of the experiment are restarted with a fix for the source code and the results are not yet available. The restarted part corresponds to the test settings where multiple slices per picture are enabled. Current version of the input document therefore reports the results for single slice per picture test setting only. The proponents requested cross-check for the following operation mode and test configurations:

- Experiment 1: Two Stage ALF with LCU-based SAO and ALF syntax, picture-based SAO and ALF encoding, single-slice per picture, and anchor using JCTVC-G1200.

- Experiment 2: Two Stage ALF with LCU-based SAO and ALF syntax, LCU-based SAO and ALF encoding, single-slice per picture, and anchor using JCTVC-G1200.

- Experiment 3: Two Stage ALF with LCU-based SAO and ALF syntax, LCU-based SAO and ALF encoding, multi-slice per picture, and anchor using JCTVC-G1200 with multi-slice per picture and disabling SAO and ALF.

The results (coding gain and runtimes) reported by the proponents are confirmed for Experiment 1.

JCTVC-H0422 (Samsung) :

Xxxx

## Subtest b: ALF simplification

### Modification points and results

The following figure explains the difference between WD5/HM5 and CE8.b.1:

The number of operations is the same as that of HM 5.0. However, the pixels required for BA feature computation is the same as those for 5x5 star shape of the top left pixel, while HM 5.0 requires 7 more pixels outside the 5x5 star shape of the top left pixel.

| Proposal | Num coef. | AI-HE | RA-HE | RA-HE10 | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- | --- | --- |
| CE8.b.1 | 9 (same as WD 5) | 0.0 100 99 | 0.1 101 101 | 0.2 100 100 | 0.1 100 100 | 0.1 100 100 |

The following description explains the difference between WD5/HM5 and CE8.b.2:

A single filter shape (combination of cross and star) is proposed instead of switching two filter shapes (cross or star) in WD5/HM5.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) in HE and HE10 configurations (without Class F):

| Proposal | Num coef. | AI-HE | RA-HE | RA-HE10 | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- | --- | --- |
| WD 5 | 9 |  |  |  |  |  |
| CE8.b.2 9x7 cross + center 3x3 square | 10 | -0.1 102 100 | 0.1 98 99 | 0.1 99 100 | 0.2 97 99 | 0.2 99 99 |
| CE8.b.2 7x5 cross + center 5x5 star | 10 | 0.0 102 99 | 0.2 99 101 | 0.0 104 107 | 0.3 100 98 | 0.1 101 97 |
| CE8.b.2 7x7 cross + center 5x5 star | 11 | 0.0 102 100 | 0.1 96 98 | 0.0 104 107 | 0.1 100 101 | 0.0 100 100 |

### Comments from cross-checkers

To CE8.b.1 (JCTVC-H0413, Samsung/Sony):

JCTVC-H0385 (Qualcomm) :

Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

To CE8.b.2 (JCTVC-H0068, TI):

JCTVC-H0416 (Samsung) :

Four filters types highlighted in JCTVC-H0068 were cross-checked: “snowflake(star)-5x5” and “cross-9x9”, both with 9 coefficients that are used in HM 5.0; and “asymmetric cross 9x7 with 3x3 square center” and “asymmetric snowflake(star)-7x5”, both with 10 coefficients. Rate-PSNR numbers match exactly to the proponent’s results. As for software study, the two 10 coefficients shapes design conform to the no-ALF line buffer principle in the adopted JCTVC-G212.

## Subtest c: Necessity of sign bits for SAO offsets

### Modification points and results

The following description explains the difference between WD5/HM5 and CE8.c.1:

It is proposed to remove sign coding of edge offset and set the offset values to be 0 or positive values for category 1 and 2, and 0 or negative values for category 3 and 4. Same scheme is applied for all color components.

The following table summarizes the coding results (Y BD-rate, U BD-rate, V BD-rate, encoding time and decoding time in %) in HE and HE10 configurations (without Class F):

| Proposal | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
| CE8.c.1 | 0.0 0.0 0.0 99 99 | 0.0 0.1 0.1 101 100 | 0.0 0.8 0.5 103 107 | 0.0 0.3 0.4 100 101 | 0.0 0.2 0.2 100 99 |
| Encoder only restriction (reference) | 0.0 0.0 0.0 99 99 | 0.0 -0.1 0.0 99 95 | 0.0 0.1 0.0 101 103 | 0.0 0.0 0.0 98 97 | 0.0 -0.1 -0.2 98 95 |

### Comments from cross-checkers

To CE8.c.1 (JCTVC-H0434, TI):

JCTVC-H0331 (Samsung/MediaTek) :

Performance and visual quality are verified for CE8.c.1, which is sign removal in edge offset coding. Both normative and non-normative solutions were testes. Average luma BD-rate change in verified tests is 0.0% for both normative and non-normative design, but there is some chroma BD-rate increase. The variation of encoding and decoding time compared to anchor is under noise level.

An alternative solution is proposed by Samsung and MediaTek in JCTVC-H0114. Both JCTVC-H0434 and JCTVC-H0114 can solve the salt and pepper noise problem. Comparison of JCTVC-H0434 and JCTVC-H0114 can be summarized as follows.

Summary of test results for 8 bits internal depth

|  | Encoder-only |  |  |  | Normative syntax change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JCTVC-H0434 |  | JCTVC-H0114 |  | JCTVC-H0434 |  | JCTVC-H0114 |  |
|  | 0EO(0)8 |  | -1EO(0)3 |  | 0EO(0)8 |  | -1EO(0)3 |  |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| AI-HE | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% |
| RA-HE | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | -0.4% |
| LD-HE | 0.0% | 0.3% | 0.0% | 0.2% | 0.0% | 0.3% | -0.1% | -0.5% |
| All | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% | -0.1% | -0.4% |

Summary of test results for 10 bits internal depth

|  | Encoder-only |  |  |  | Normative syntax change |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JCTVC-H0434 |  | JCTVC-H0114 |  | JCTVC-H0434 |  | JCTVC-H0114 |  |
|  | 0EO(0)32 |  | -5EO(0)11 |  | 0EO(0)32 |  | -5EO(0)11 |  |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| RA-HE | 0.0% | 0.6% | 0.0% | 0.0% | 0.0% | 0.7% | 0.0% | 0.1% |

Our observations are:

- For encoder only change, JCTVC-H0114 performs better than JCTVC-H0434, especially for Inter test cases. So total restriction of SAO sharpening effect is not beneficial performance wise.

- Normative changes of EO syntax in JCTVC-H0434 do not provide performance improvement (sometimes causes performance degradation).

- Maximum performance drop observed in verified test was 0.6% for luma and 5.0% for chroma.

- Maximum performance drop observed in JCTVC-H0114 was 0.1% for luma and 1.9% for chroma

## Subtest d: SAO simplification and line buffer reduction

### Modification points and results

The following figure explains the difference between WD5/HM5 and CE8.d.1:

It is proposed to divide intensity level into 4 bands as shown in the following figure. Selected band is coded as SAO type. For each band the intensity level range is divided into 8 sub-bands. Therefore, only 8 offsets need to be coded and stored in the buffer.

It is further proposed to extend the range of the first and of the last sub-band as shown in the following figure.

The following description explains the difference between WD5/HM5 and CE8.d.2:

The proposed line memory reduction method exploits the high correlation between the “partial pixel classification results” of neighboring pixels, given by the functions: xSign(P1,CENTER−P1,UP) and xSign(P2,CENTER−P2,UP), for reducing on-chip memory. The proposed method results in 50% reduction in the encoder and decoder line memory. Algorithm is described as follows:

If 45-Degree, 90-Degree or 145-Degree Edge-Offset pixel classifiers are selected, then the following is applied at the 4th line above LCU-boundary (2nd line above LCU boundary for chroma):

- If xSign(P1,CENTER−P1,UP) is equal to xSign(P2,CENTER−P2,UP), then xSign(P2,CENTER−P2,UP) is stored in the line memory as a representative of both.

- If xSign(P1,CENTER−P1,UP) is different from xSign(P2,CENTER−P2,UP), then value “0” is stored in the line memory as a representative of both.

Where xSign(X)=1if(X>0)0if(X==0)−1if(X<0).

The following table summarizes the achieved buffer reduction:

| Number | Proponent | Buffer reduction compared to HM5.0 |
| --- | --- | --- |
| JCTVC-H0436 CE8.d.1 (band offset reduction) | Texas Instruments | Storage of 8 band offsets instead of 16 (50%) |
| JCTVC-H0436 CE8.d.1 (band offset reduction + sub band extension) | Texas Instruments | Storage of 8 band offsets instead of 16 (50%) |
| JCTVC-H0375 CE8.d.2 | Panasonic | Encoder: Line memory reduction from 0.75 to 0.375 lines per color component (50%) Decoder: Line memory reduction from 0.25 to 0.125 lines per color component (50%) |
| JCTVC-H0174 CE8.e.2 (3 band offset reduction, or 1 band offset) | Canon | Storage of 8 band offsets instead of 16 (50%), but need to signal center value with 5 bit (in total 43% reduction compared to WD5) |

The following table summarizes the coding results (Y BD-rate, U BD-rate, V BD-rate, encoding time and decoding time in %) for HE and HE10 configurations (without Class F):

| Number | Proponent | AI-HE | RA-HE | RA-HE10 | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- | --- | --- |
| JCTVC-H0436 CE8.d.1 (band offset reduction) | Texas Instruments | 0.0 -0.2 -0.1 98 98 | 0.0 -0.3 -0.2 99 100 | 0.0 -0.4 -0.4 106 109 | 0.0 -0.5 -0.5 101 101 | 0.0 -0.4 -0.7 98 98 |
| JCTVC-H0436 CE8.d.1 (band offset reduction + sub band extension) | Texas Instruments | 0.0 0.0 0.1 94 94 | 0.0 0.0 0.1 100 102 | 0.0 0.1 0.0 104 106 | 0.0 -0.1 0.2 101 102 | 0.0 -0.1 0.0 97 97 |
| JCTVC-H0375 CE8.d.2 | Panasonic | 0.0 0.0 0.0 100 100 | 0.0 0.0 0.0 100 100 | 0.0 0.0 0.0 100 100 | 0.0 -0.2 0.0 100 100 | - |
| JCTVC-H0174 CE8.e.2-A (3 band offset mode) | Canon | 0.1 -1.2 -1.4 100 100 | 0.0 -1.5 -1.6 100 100 | 0.0 -1.7 -1.9 100 102 | -0.1 -2.6 -2.8 100 99 |  |
| JCTVC-H0174 CE8.e.2-B (1 band offset mode) | Canon | 0.0 -0.2 -0.3 99 99 | 0.0 -0.4 -0.3 100 99 | 0.0 -0.5 -0.5 100 99 | 0.0 -0.5 -0.4 100 99 |  |

Note: Subjective viewing will not be conducted on proposal highlighted in yellow.

Note: CE8.e.2 (JCTVC-H0174) can also be consider as band offset buffer reduction.

### Comments from cross-checkers

To CE8.d.1 (JCTVC-H0436, TI):

JCTVC-H0332 (MediaTek) :

Basically, results are confirmed. In Test 1, the number of transmitted band offsets is reduced from 16 to 8, so the buffer size to store offsets is reduced by half. The measured BD-rate is 0.0% for luma and -0.4% for chroma. In Test 2, the ranges of the first and last bands are extended. The BD-rate becomes 0.0% for both luma and chroma. The crosschecker would like to point out that the buffer size can be reduced by adding level constraints on the number of offsets per picture, as proposed in JCTVC-G218, JCTVC-H0272, JCTVC-H0273, and JCTVC-H0274. When the buffer size is reduced by half by adding the constraint for the SAO in HM-5.0, no coding efficiency loss is observed.

To CE8.d.2 (JCTVC-H0375, Panasonic):

JCTVC-H0334 (MediaTek) :

The line memory for SAO is reduced by 50% with unchanged coding efficiency and run times. All data are confirmed.

## Subtest e: SAO improvement

### Modification points and results

The following figure explains the difference between WD5/HM5 and CE8.e.1:

Syntax is the same as that of WD 5.

## Offset codeword design

Offset codeword for EO mode is newly designed to represent the statistics shown in JCTVC-G222.

The offset coding in BO is not changed in this proposal. The offset coding in EO is as follows:

If ( ( offset – b ) < 0 ), index = – ( ( offset – b ) << 1 ) – a

Else, index = ( ( offset – b ) << 1 ) – 1 + a

The index is used as the input of exp-Golomb code table. The parameters, a and b, are shown in the following table for different EO categories, where b denotes the offset value of the highest probability and a controls the priorities of two offset values of the same magnitude.

Table 2.  Parameters of the codeword design

|  | 10-bit pixels | 8-bit pixels |
| --- | --- | --- |
| EO category 1 | b = 2, a = 1 | b = 1, a = 1 |
| EO category 2 | b = 1, a = 1 | b = 1, a = 0 |
| EO category 3 | b = -1, a = 0 | b = -1, a = 1 |
| EO category 4 | b = -2, a = 0 | b = -1, a = 0 |

## Offset prediction

Prediction of offset values from the neighboring region is newly introduced for EO mode.

Prediction of offset values from the adjacent band (i.e., predict the second band offset from the first band offset, and so on) is newly introduced for BO mode.

To predict the offset value, the neighboring region information is used. If the current region and the left region in the same depth select EO, category 1 and category 2 of the current region will be predicted by corresponding offsets of the left region, and in the current region the category 3 and category 4 will be predicted by category 2 and category 1 offset magnitudes of opposite signs. If the current region applies BO, the offset of the second band will be predicted by that of the first band, and the offset of the 16th band will be predicted by that of the 15th band.

The following figure explains the difference between WD5/HM5 and CE8.e.2:

Different SAO Band Offset types are proposed in order to be more adaptive to the pixel values repartition. These 3 proposed types of Band Offsets are replacing the current Band Offset classifications illustrated in the figure below. Compared to HM5.0, the number of offsets is changed and is reduced from 16 to 8. The range of a band in BO_8o_1 is 8, which is the same as WD5. The ranges of a band in BO_8o_2 and BO_8o_3 are 4 and 2, respectively. The center of pixel value of each proposed Band Offsets is transmitted. This center is coded by considering a subdivision of the intensity interval into 32 ranges.

|  | HM5.0 | Exp A | Exp B |
| --- | --- | --- | --- |
| BO_0 | On |  |  |
| BO_1 | On |  |  |
| BO_8o_ 1 |  | on | on |
| BO_8o_2 |  | on |  |
| BO_8o_3 |  | on |  |

The following figure explains the difference between WD5/HM5 and CE8.e.3:

The first aspect is to modify the derivation process of edge index To enable the use of 8 edge offsets, where 4 of the edge offsets are identical with WD. The additional offsets enable modifications of edges using same neighbourhood as WD but without to require that two neighbouring pixels shall have equal value as in WD.

edgeIdx = 20 + Sign(-2*recPicture[xC+I, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])+

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

where hPos, vPos, recPicture and Sign are identical to the definition in WD.

When the current line is the 4th line for luma component and 2nd line for chroma components above the LCU boundary, 4 edges identical with WD is used which enable same line memory requirement for LCU based processing as WD.

The second aspect is to add a switch between transmitting 8 edge offset or the current 4 edge offset in WD5. The figure below explains the characteristics of each edge offset.

The third aspect is to predict the edge offset of the local maxima as the negative of the edge offset of the local minima.

4 offsets

8 offsets

The following description explains the difference between WD5/HM5 and CE8.e.4:

# Modification in the application of the EO offset

The EO offset is restricted to be of the same sign (where the sign is inferred from the relative position of the current pixel and a bilinear interpolation from the two neighboring pixels according to the edge direction). This offset is applied per class but with the following modification from WD5. Consider an interpolated current pixel value (I) that is generated using a weighted combination of its two neighbors (L, R) as shown below.  The offset is applied to the current pixel (C) as in WD5 unless the application of the offset results in a value that passes I.  In this case, the output value (O) is set to I

In another experiment, the EO offset is allowed to have both signs. In this case, the above applies but when the offset is negative, the offset magnitude is applied in the direction away from the interpolated value.

# Modification in the entropy coding of the EO offset

Since the EO offset is restricted to be of the same sign, the distribution of the EO offsets is expected to be different than that of the same positive range in the unrestricted case because the direction (sign) is now taken into account according to class type. It has been observed (see below) that the +1 offset tends to be more frequent than the zero offset over EO types (especially at low QPs). Taking this into account, the codewords for the +1 and 0 offsets are switched for EO types.

# Modification in the number of edge offset classes

The number of potential classes has expanded to 8. With a proper mapping, a certain number of classes per edge type can be achieved. In this experiment, the total number of edge offsets is either 4 or 8 is studied.

# Modification in the entropy coding of the SAO types

The codeword for the representation of SAO type is changed to make the bit cost for signalling the SAO types more even, as shown below.

SAO type		code

SAO off		1

SAO E0		0000

SAO E1		0001

SAO E2		0010

SAO E3		0011

SAO B0		010

SAO B1		011

The following description explains the difference between WD5/HM5 and CE8.e.5:

Harmonization of CE8.e.3 and CE8.e.2.

The following table summarizes the coding results (Y BD-rate, U BD-rate, V BD-rate, encoding time and decoding time in %) for HE and HE10 configurations:

| Proposal | AI-HE | RA-HE | RA-HE10 | LDB-HE | LDP-HE |
| --- | --- | --- | --- | --- | --- |
| CE8.e.1-1** | 0.0 -0.1 -0.1 100 100 | -0.1 -0.1 -0.2 100 100 | 0.0 0.0 0.0 100 100 | -0.1 0.1 0.0 100 100 | -0.1 -0.2 -0.4 101 101 |
| CE8.e.1-2*** | 0.0 0.0 -0.1 101 100 | -0.1 -0.1 -0.1 100 100 | 0.0 0.0 0.0 100 100 | 0.0 0.2 -0.1 100 100 | 0.0 -0.1 -0.2 100 99 |
| CE8.e.2-A (3 band offsets) | 0.1 -1.2 -1.4 100 100 | 0.0 -1.5 -1.6 100 100 | 0.0 -1.7 -1.9 100 102 | -0.1 -2.6 -2.8 100 99 |  |
| CE8.e.2-B (1 band offset) | 0.0 -0.2 -0.3 99 99 | 0.0 -0.4 -0.3 100 99 | 0.0 -0.5 -0.5 100 99 | 0.0 -0.5 -0.4 100 99 |  |
| CE8.e.3 | 0.0 -0.4 -0.4 X 100 | -0.1 -1.0 -1.0 X 102 | -0.1 -1.3 -1.3 100 104 | -0.2 -1.7 -1.9 X 102 | -0.1 -1.2 -1.7 X 102 |
| CE8.e.4 | 0.0 -0.3 -0.4 103 10.6 | -0.1 -0.4 -0.5 101 105 | 0.0 -0.3 -0.3 102 106 | -0.1 -0.5 -0.7 101 106 | -0.1 -0.6 -0.9 101 108 |
| CE8.e.5 | 0.1 -1.5 -1.7 99 100 | -0.1 -2.3 -2.4 100 102 | -0.1 -2.8 -3.0 100 101 | -0.3 -4.0 -4.2 100 101 | -0.3 -3.4 -4.2 99 101 |

Note: Subjective viewing will not be conducted on proposal highlighted in yellow.

The following table summarizes the coding results (Y BD-rate, U BD-rate, V BD-rate, encoding time and decoding time in %) for LC configurations:

| Proposal | AI-LC | RA-LC | LDB-LC | LDP-LC |
| --- | --- | --- | --- | --- |
| Reference (SAO-off vs SAO-on in HM 5.0) | -0.7 -2.5 -3.2 104 107 | -1.2 -4.0 -4.5 100 103 | -2.2 -8.7 -10.1 101 104 | -7.9 -11.9 -12.7 101 105 |
| CE8.e.1-1** | -0.7 -2.6 -3.3 101 103 | -1.2 -4.0 -4.6 100 102 | -2.3 -8.6 -9.9 100 103 | -8.0 -11.6 -12.8 100 104 |
| CE8.e.1-2*** | -0.7 -2.6 -3.3 101 103 | -1.2 -3.9 -4.5 100 103 | -2.2 -8.4 -9.8 100 103 | -8.0 -11.6 -12.7 100 104 |
| CE8.e.2-A (3 band offsets) | -0.6 -3.3 -4.2 ? ? | -1.2 -6.1 -6.9 ? ? | -2.4 -11.9 -13.4 ? ? | -8.0 -14.7 -15.9 ? ? |
| CE8.e.2-B (1 band offset) | -0.7 -2.7 -3.3 ? ? | -1.2 -4.5 -5.0 ? ? | -2.2 -9.5 -10.8 ? ? | -7.9 -12.3 -13.5 ? ? |
| CE8.e.3 | -0.7 -2.8 -3.4 100 105 | -1.4 -5.2 -5.8 100 106 | -2.8 -10.9 -12.7 100 106 | -8.9 -13.5 -14.9 100 108 |
| CE8.e.4 | -0.7 -3.0 -3.7 ? ? | -1.2 -4.3 -5.0 ? ? | -2.3 -9.1 -10.7 ? ? | -8.1 -12.1 -13.4 ? ? |
| CE8.e.5 | -0.7 -3.5 -4.4 100 105 | -1.4 -7.0 -7.8 100 105 | -2.9 -13.5 -15.3 99 105 | -9.0 -16.1 -17.8 99 107 |

Note: Subjective viewing will not be conducted on proposal highlighted in yellow.

*CE8.e.3 and CE8.e.5 has results with ALF off cases as a supplemental data.

**CE8.e.1-1 can significantly reduce salt and pepper noises. (The SAO in HM-5.0 only optimizes PSNR and can have this problem in rare cases.)

***CE8.e.1-2 can totally remove salt and pepper noises. (The SAO in HM-5.0 only optimizes PSNR and can have this problem in rare cases.)

### Comments from cross-checkers

To CE8.e.1 (JCTVC-H0276, MediaTek/Qualcomm):

JCTVC-H0173 (Canon) :

Two experiments were cross-checked. The first experiment A corresponds to the CE8.e.1 description where the offset coding for Edge Offset is changed compared to HM5.0. The second experiment B is an additional non-normative method applied on encoder side above experiment A. The BDR results reported here perfectly match those provided by MediaTek/Qualcomm in document JCTVC-H0276.

To CE8.e.2 (JCTVC-H0174, Canon):

JCTVC-H0229 (Ericsson) :

Encoding time is unreliable but is expected to be similar as the anchor. Decoding time is computed on a single PC. Visual inspection indicated that the proposal according to Experiment B performs subjectively similar as the anchor.

To CE8.e.3 (JCTVC-H0223, Ericsson/Panasonic):

JCTVC-H0390 (Qualcomm) :

Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

To CE8.e.4 (JCTVC-H0557, Motorola Mobility):

JCTVC-H0335 (MediaTek) :

Simulation results are confirmed.

To CE8.e.5 (JCTVC-H0225, Ericsson/Canon/Panasonic):

JCTVC-H0337 (MediaTek) :

No additional line buffer compared with HM-5.0 is required. The average coding efficiency gain is 0.1% for luma and 3% for chroma under the JCTVC-G1200 common conditions. All data are confirmed.

## Subtest f: Subjective picture quality assessment of ALF

Setup of CE8.f viewing is described in section 9.4 of the attached CE8 description.

# References

[1]	“Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, November 2011.

[2]	“Description of CE8: Non-deblocking loop filtering”, JCTVC-G1208, Geneva, November 2011.