| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0040_r2 WG11 Number: m23353 |
| --- | --- |

| Title: | CE10: Summary report of core experiment on deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Xun Guo Byeungwoo Jeon Matthias Narroschke | Email: | andrey.norkin@ericsson.com xun.guo@mediatek.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This contribution is a summary report of Core Experiment 10 on deblocking filtering. The goal of the proposals is to enhance deblocking filtering techniques in the HEVC Test Model. This is not limited to improving coding efficiency and subjective quality but also reducing complexity and harmonization of the various schemes that are technically feasible. The CE comprises four subtests. All the proposals except those in Subtest 4 are based on HM5.0. Proposals in Subtest 4 are based on the version of HM5.0 with implemented deblocking filter parameters signaling provided by Toshiba and distributed in among the CE 10 participants. The experiments are performed according to the common test conditions provided in JCTVC-G1200. Additionally, all the proposals provide results for LD-P configurations. All results are verified by cross-checkers.

# Document list

## Summary document (this document)

- JCTVC-H0040 [CE coordinators] CE12: Summary report of core experiment on deblocking filtering

## Proposals and cross-verification documents

Subtest 1: Subjective and objective quality improvements

- [JCTVC-H0310, MediaTek] CE10 Subtest1: Chroma Deblocking Filter

  - [JCTVC-H0298, Intel] CE10 Subtest1: Cross verification of MediaTek’s chroma deblocking filter (JCTVC-H0310)

- [JCTVC-H0311, MediaTek] CE10 Subtest1: Simplified Chroma Deblocking Filter in JCTVC-H0310

  - [JCTVC-H0299, Intel] CE10 Subtest1: Cross verification of MediaTek’s simplified chroma deblocking (JCTVC-H0311)

- [JCTVC-H0147, Panasonic] CE10.1: Symmetric rounding in the deblocking filter

  - [JCTVC-H0262, Sony] CE10 Subtest1: Cross-verification for Panasonic's proposal JCTVC-H0147

Subtest 2: Line memory reduction

- [JCTVC-H0280, MediaTek, Sony] CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries

  - [JCTVC-H0575, Ericsson] CE10.2: cross-verification of Sony's and MediaTek's proposal on deblocking line memory reduction

  - [JCTVC-H0149, Panasonic] CE10.2: Cross check results for the deblocking filter line buffer reduction of Sony and MediaTek

- [JCTVC-H0472, Ericsson] CE10.2: Level restrictions on tiles for line buffer reduction

  - [JCTVC-H0264, Sony] CE10 Subtest2: Cross-verification for Ericsson's proposal

Subtest 3: Deblocking filter simplifications

- [JCTVC-H0146, Panasonic] CE10.3: Simplified delta calculation in the deblocking filter

  - [JCTVC-H0261, Sony] CE10 Subtest3: Cross-verification for Panasonic's proposal JCTVC-H0146

- [JCTVC-H0108, LG] CE10 Subtest3: Simplified BS calculation process in deblocking filter

  - [JCTVC-H0576, Ericsson] CE10.3: Cross-verification of proposal on deblocking Bs simplification from LG

- [JCTVC-H0473, Ericsson] CE10.3: Deblocking filter simplifications: Bs computation and strong filtering decision

  - [JCTVC-H0618, LG] Cross-verification of Ericsson's proposal on deblocking filter simplifications (JCTVC-H0473)

- [JCTVC-H0396, Qualcomm] CE10 Subtest 3: Deblocking Filter Simplifications

  - [JCTVC-H0209, SKKU/SKT] CE10.3: Cross-verification Results of Qualcomm's Simplification of Deblocking (JCTVC- H0396) by SKT/SKKU

- [JCTVC-H0155, SKT/SKKU] CE10.3: Simplification of First Delta Calculation in Deblocking by SKT/SKKU

  - [JCTVC-H0459, Qualcomm] CE10.3: Cross Check of SKT/SKKU Proposal JCTVC-H0155

- [JCTVC-H0156, SKT/SKKU] CE10.3: Simplification of Second Delta Calculation in Deblocking by SKT/SKKU

  - [JCTVC-H0157, ETRI] CE10.3: Cross verification of SKT/SKKU's Simplification of Second Delta Calculation in Deblocking (JCTVC-H0156)

- [JCTVC-H0084, TI] CE10 Subtest 1: Test results on unification of luma and chroma filtering

  - [JCTVC-H0263, Sony] CE10 Subtest1: Cross-verification for TI's proposal JCTVC-H0084

Subtest 4: Controlling strength of deblocking filter at slice level

- [JCTVC-H0148, Panasonic] CE10.4: Deblocking filter using adaptive weighting factors

  - [JCTVC-H0350, Sony] CE10 Subtest4: Cross-verification for Panasonic's proposal JCTVC-H0148

- [JCTVC-H0574, Ericsson] CE10.4: deblocking parameters signaling

  - [JCTVC-H0474, Qualcomm] CE10.4: Cross check of Ericsson's proposal JCTVC-H0574

- [JCTVC-H0398, Qualcomm] CE10 Subtest 4: Transform Dependent Deblocking Filter Parameter Adjustment

  - [JCTVC-H0656, Toshiba] CE10 Subtest 4: Cross-check of Qualcomm's Transform Dependent Deblocking Filter Parameter Adjustment JCTVC-H0398

Note: Proposal H0084 has been moved from Subtest 1 to Subtest 3 on request of proponents.

# Experimental results

Based on common test conditions [1], results for all mandatory test cases and two additional LD-P configurations shall be provided for CE10 proposals. The following evaluation criteria are used:

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Subjective video quality

In the tables below, the following abbreviations are used:

AI – all intra

RA – random access

LD B – low delay (B)

LD P – low delay (P)

## Subtest 1: Subjective and objective quality improvements

Table 1 Cross-verification assignment for Subtest 1

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| H0310, MediaTek | Intel (H0298) |
| H0311, MediaTek | Intel (H0299) |
| H0147, Panasonic | Sony (H0262) |

### Subtest 1: Deblocking filter modifications

Table 2 Parts that have been modified compared to HM5.0 are marked with “x”

| Proponent | H0310, MediaTek | H0311, MediaTek | H0147, Panasonic |
| --- | --- | --- | --- |
| Line-wise filtering decisions (on/off) | Added filtering decision for chroma | Added filtering decision for chroma |  |
| Decision weak/strong filter |  |  |  |
| Luma filter operation (weak) |  |  | Introduced symmetric rounding in the “delta”-calculation |
| Luma filter operation (strong) |  |  |  |
| Chroma filter operation |  |  | Introduced symmetric rounding in the “delta”-calculation |
| Bs derivation | Added derivation of Bs = 2 for chroma |  |  |
| Tc values |  |  |  |
| Beta values |  |  |  |
| Optimization of original source code parts |  |  |  |
| Remarks |  |  |  |

### Subtest 1: summary of proposals

H0310, MediaTek: Chroma filtering 1

HM5.0, chroma components are de-blocked only if any of the adjacent blocks is intra.

Separate deblocking filtering decisions to chroma deblocking are proposed:

|  | p30 | p20 | p10 | p00 | q00 | q10 | q20 | q30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p31 | p21 | p11 | p01 | q01 | q11 | q21 | q31 |  |
|  | p32 | p22 | p12 | p02 | q02 | q12 | q22 | q32 |  |
|  | p33 | p23 | p13 | p03 | q03 | q13 | q23 | q33 |  |

d_chroma = |p01 – p11| + |q01 – q11| + |p02 – p12| + |q02 – q12|, where pij and qij are chroma pixels

The chroma samples will be filtered if the below expression is true:

(d_chroma < Beta_Chroma) && (On the 8x8 chroma grid).

H0310, MediaTek: Chroma filtering 2

Chroma components in HM5.0 are de-blocked only if any of the adjacent blocks is intra.

It is proposed to modify chroma filtering as in the following (reusing d, calculated on luma samples).

d_luma = |p20 – 2*p10 + p00| + |q20 – 2*q10 + q00| + |p23 – 2*p13 + p03| + |q23– 2*q13 + q03|, where pij and qij are luma pixels.

The corresponding chroma samples will be filtered if the below expression is true:

(d < Beta_Chroma) && (On the 8x8 chroma grid).

- For I slices, Beta_Chroma is defined as a fixed value 128.

- For B/P slices, Beta_Chroma is the same as Beta_Luma.

H0147, Panasonic: Rounding in deblocking filter

It is proposed to modify rounding in calculation of Δ’s in deblocking filter with the intention of making it symmetric for positive and negative values.

Rounding in HM5.0 Δ=9⋅q0i−p0i−3⋅q1i−p1i+8>>4 is shown in the figure below.

It is proposed to change rounding by adding a value a=7 in the case of a positive Δk and a different value b=8 in the case of a negative Δk:

Δ=9⋅q0i−p0i−3⋅q1i−p1i+a>>4;Δk≥09⋅q0i−p0i−3⋅q1i−p1i+b>>4;Δk<0

In the implementation, it is also proposed to combine rounding with subsequent clipping in the following way:

Different a and b are used in calculation of different deltas:

|  | Number of right shift bits | a | b |
| --- | --- | --- | --- |
| Luma weak filter for p0i and q0i | 4 | 7 | 8 |
| Luma weak filter for p1i and q1i | 1 | 0 | 1 |
| Chroma filter for p0i and q0i | 3 | 3 | 4 |

### Subtest 1: Results

Table 3 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0310, MediaTek | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| H0311, MediaTek | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| H0147, Panasonic | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |

Table 4 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0310, MediaTek | -0.4% | -0.6% | -0.6% | -0.6% | 0.0% | -0.9% | -1.2% | -1.2% | -0.6% |
| H0311, MediaTek | -0.3% | -1.2% | -1.1% | -1.0% | -0.2% | -1.6% | -2.2% | -2.2% | -1.5% |
| H0147, Panasonic | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |

Table 5 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0310, MediaTek | -0.5% | -0.6% | -0.7% | -0.8% | 0.0% | -0.9% | -1.1% | -1.3% | -0.5% |
| H0311, MediaTek | -0.3% | -1.2% | -1.1% | -0.6% | -0.1% | -1.4% | -1.6% | -1.3% | -1.3% |
| H0147, Panasonic | -0.1% | 0.0% | 0.2% | -0.1% | -0.1% | 0.0% | -0.1% | -0.3% | 0.0% |

## Complexity – relative decoding time against the anchor

Table 6 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0310, MediaTek | 99% | 100% | 100% | 100% | 99% | 100% | 100% | 101% | 99% |
| H0311, MediaTek | 99% | 99% | 99% | 100% | 99% | 99% | 100% | 101% | 101% |
| H0147, Panasonic | 99% | 99% | 100% | 99% | 100% | 99% | 99% | 99% | 100% |

### Subtest 1: comments from cross-checkers

- [JCTVC-H0310, MediaTek] CE10 Subtest1: Chroma Deblocking Filter

  - [JCTVC-H0298, Intel] The BD rate results exactly match the data provided by MediaTek, and the encoding and decoding time are similar with MediaTek’s data. Informal subjective viewing showed similar visual quality with HM5.0. Some areas in the proposal results showed better visual quality and some other areas showed slightly worse visual quality. It is hard to say it is caused by the proposal or by the change of motion prediction.

- [JCTVC-H0311, MediaTek] CE10 Subtest1: Simplified Chroma Deblocking Filter in JCTVC-H0310

  - [JCTVC-H0299, Intel] The BD rate results exactly match the data provided by MediaTek, and the encoding and decoding time are similar with MediaTek’s data. Informal subjective viewing showed similar visual quality with HM5.0. Some areas in the proposal results showed better visual quality and some other areas showed slightly worse visual quality. It is hard to say it is caused by the proposal or by the change of motion prediction.

- [JCTVC-H0147, Panasonic] CE10.1: Symmetric rounding in the deblocking filter

  - [JCTVC-H0262, Sony] The BD-bitrates are completely identical with ones provided by Panasonic. The encoding and decoding time are similar with Panasonic’s results. The source code seems to be as described in H0147. The visual quality is similar with HM5.0 in sequences described in JCTVC-G1210. The implement cost is low.

## Subtests 2: Line memory reduction

Table 7 Cross-verification assignment Subtest 3

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| H0280, MediaTek/Sony | Panasonic (H0147), Ericsson (H0575) |
| H0472, Ericsson | Sony (H0264) |

### Subtest 2: Deblocking filter modifications

Table 8 Parts that have been modified compared to HM5.0

| Proponent | H0280, MediaTek/Sony | H0472, Ericsson |
| --- | --- | --- |
| Number of lines for writing | 2 | 3 |
| Number of lines for reading | 3 | 4 |
| Applied to LCU boundaries or all horizontal boundaries? | LCU boundaries | none of them |
| Modifications of deblocking decision | Yes | no |
| Modifications of filtering operations | Yes | no |
| Deblocking line buffer reduction | 1 for Luma | two times |
| Remarks |  |  |

### Subtest 2: summary of proposals

H0280, MediaTek/Sony: Reduction of deblocking line buffer by one line

Two adjacent blocks forming a block boundary.

Deblocking in HM5.0 reads 4 pixels above the block boundary and writes 3 pixels.

It is proposed to modify deblocking (strong filtering) of horizontal LCU boundaries so that deblocking reads 3 pixels above the boundary and writes 2 pixels.

In strong filtering decisions, it is proposed to modify a condition

|p3-p0| + |q0 - q3| < (β>>3) 			HM5.0

|p2-p0| + |q3-q0| < (β>>3).			Proposal

In strong deblocking filtering, it is proposed to NOT modify pixel p2 and to change calculation of pixel p1 as in the following.

p1 = (p2 + p1 + p0+ q0 +8)>>4			HM5.0

p1 = (4*p2 + 5*p1 + 4*p0+ 3*q0 +8)>>4	Proposal

H0472, Ericsson: Level restrictions on tiles for line buffer reduction

It is proposed to introduce restrictions on the maximum tile width when levels are defined. For a certain level the tile width shall not exceed a certain number of LCU blocks. If the picture width exceeds this number, it is split in vertical tiles.

In Error: Reference source not found, an example of minimum and maximum tile sizes is introduced.

Example on restrictions on Tile size

| Level | MaxTileWidthInLCU | MinTileWidthInLCU | MinTileHeightInLCU |
| --- | --- | --- | --- |
| 5 and above | 30 | 3 | 3 |

LCU

#0

LCU

#1

LCU

#6

LCU

#7

LCU

#14

Vert. tile boundary

Tile #1

Line buffer

Column buffer

(saved in the memory)

decoded LCU’s

LCU’s to be decoded

    - Subtest 2: Results

Table 9 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0280, MediaTek, Sony | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| H0472, Ericsson | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |

Table 10 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0280, MediaTek, Sony | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | -0.1% |
| H0472, Ericsson | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% | -0.2% | -0.2% | -0.2% |

Table 11 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0280, MediaTek, Sony | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| H0472, Ericsson | -0.1% | 0.0% | -0.2% | -0.4% | 0.0% | -0.1% | 0.1% | -0.2% | -0.1% |

## Complexity – relative decoding time against the anchor

Table 12 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0280, MediaTek, Sony | 99% | 99% | 100% | 100% | 99% | 99% | 100% | 102% | 99% |
| H0472, Ericsson | 100% | 100% | 100% | 100% | 99% | 100% | 100% | 100% | 100% |

### Subtest 2: comments from cross-checkers

- [JCTVC-H0280, MediaTek, Sony] CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries

  - [JCTVC-H0575, Ericsson] Part 1 of the proposal is checked in this contribution. The BD-rate results of Part 1 are identical to those provided by the proponent and the decoding times are similar. The software was found matching the description. The subjective quality was studied on LC configurations and was similar to the anchors on most of the studied sequences. On a couple of sequences one could observe some trailing artifacts that were not present in the anchor. However, is it difficult to say if these artifacts appear because of the proposal or because of different motion prediction.

  - [JCTVC-H0149, Panasonic] The proposal JCTVC-H0280 consists of two technical changes compared to HM5.0. First, one line buffer is reduced by modifying the deblocking filter. Second, the so called luminance virtual boundary of adaptive loop filtering is shifted to three pixels above the horizontal LCU boundary to match the proposed modified deblocking. The combination of both technical changes is cross checked in this contribution. The BD-bit rate results as well as the run time ratios reported by Sony and MediaTek are confirmed.

- [JCTVC-H0472, Ericsson] CE10.2: Level restrictions on tiles for line buffer reduction

  - [JCTVC-H0264, Sony] The BD-bitrates are completely identical with ones provided by Ericsson. The encoding and decoding time are similar with Ericsson’s results. The source code is not changed and the configuration is changed. The subjective quality is similar with HM5.0 in sequences described in JCTVC-G1210. It is concerned that reading pixels in previous tile (tile-#1) from frame memory (DRAM) in the process of tile-#2 provides additional complicated control and frame memory (DRAM) access.

## Subtest 3: Deblocking filter simplifications

Table 13 Cross-verification assignment Subtest 3

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| H0146, Panasonic | H0261, Sony |
| H0108, LG | H0576, Ericsson |
| H0473, Ericsson | H0618, LG |
| H0396, Qualcomm | H0209, SKKU |
| H0155, SKT/SKKU | H0459, Qualcomm |
| H0156, SKT/SKKU | H0157, ETRI |
| H0084, TI | Sony (H0263) |

### Subtest 3: Deblocking filter modifications

Table 14 Parts that have been modified compared to HM5.0 are marked with “x”

| Proponent | H0146 | H0108 | H0473 | H0396 Part 1 | H0155 | H0156 | H0084, TI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Decision weak/strong filter |  |  | x |  |  |  |  |
| Luma filter operation (weak) | x |  |  | x | x | x | x |
| Luma filter operation (strong) |  |  |  |  |  |  |  |
| Chroma filter operation |  |  |  | x |  |  | x |
| Bs derivation |  | x | x |  |  |  |  |
| Tc values |  |  |  |  |  |  |  |
| Beta values |  |  |  |  |  |  |  |
| Optimization of original source code parts |  |  |  |  |  |  |  |
| Remarks | Simplified delta calculation for outer sample deblocking. Shorter critical path. Parallel deblocking of q1,q0,p0,p1 | Removal max Bs process, Reduce # of Bs decision process | one loop over 4-pel segments, less variables in strong filtering decision |  | Simplified first delta calculation (no multiplication), unification of the luma and chroma filters | No additional filter for second delta calculation (p1 and q1) | Chroma filter shares the same delta calculation with luma weak filter |

### Subtest 3: summary of proposals

HM5.0 deblocking

Δ = 9* (q0 – p0) – 3 (q1 – p1) + 8 ) >> 4

if (abs (Δ) < ThrCut )

{

Δ = Clip3 (–tc, tc, Δ )

p0 = Clip( p0 + Δ)

q0 = Clip ( q0 – Δ)

if (Deblock_p1)

{

Δp1 = Clip3 (–tc, tc,  ((p2 + p0 + 1)>>1 ) – p1 + Δ ) >>1  )

p1 = Clip( p1 + Δp1)

}

if (Deblock_q1)

{

Δq1 = Clip3 (–tc, tc,  ((q2 + q0 + 1)>>1 ) – q1 – Δ ) >>1  )

q1 = Clip ( q1 + Δq1)

}

}

H0146, Panasonic:

The proposed changes:

Δp1 = Clip3 (–tc, tc, ((p2 + p0 + 1)>>1 ) – p1 + Δ ) >>1  )

Δq1 = Clip3 (–tc, tc, ((q2 + q0 + 1)>>1 ) – q1 – Δ ) >>1  )

Compared to HM5.0:

1. Calculation of Δp1 and Δq1 does not depend on calculation of Δ.

2. Modification of p1 and q1 does not depend on the signal on the other side of the block boundary, only smoothing at p1 and q1.

H0156, SKT/SKKU

The proposed changes:

Δp1 = Δ/2

Δq1 = –Δ/2

Compared to HM5.0:

Second delta calculation uses only the first delta without additional pixel values.

H0396, Qualcomm

Luma Weak filter HM5.0:   	Δ = ( 9*( q0 – p0) –3*(q1–p1) + 8) >> 4

Luma Weak filter proposed:    = ( 3*(q0 – p0) – (q1 – p1) + 4 ) >> 3

Chroma filter proposed: 	same as luma

Fig 1. (a) step (b) HM5.0 filters (c) proposed weak filter

Fig 2. (a) Step (b) HM5.0 luma filter (c) HM5.0 chroma filter, (d) proposed chroma filter

H0084, TI

Luma Weak filter HM5.0:   Δ = (9*( q0 – p0) –3*(q1–p1) + 8)>>4

Luma Weak filter proposed : Δ = (13*( q0 – p0) - 4*( q1–p1) + 16) >> 5

Chroma filter HM-5.0 :    Δ = ((q0 – p0)<<2) + p1 –q1 + 4 )>>3;

Chroma filter proposed :   Δ = (13*( q0 – p0) - 4*( q1–p1) + 16)>>5

Chroma filter is the same as luma.

Shape the output of filtering the step function is somewhat between Fig 1 (c) and HM5.0 (Fig 1 (b)) .

H0155, SKT/SKKU

Luma Weak filter HM5.0:   Δ = (9*( q0 – p0) –3*( q1–p1) + 8)>>4

Luma Weak filter proposed : Δ = ((q0 – p0) << 2) + p1 –q1 + 4 )>>3

It is proposed to use HM5.0 chroma filter (see Fig. 2(c)) for luma filtering.

H0108, LG: Bs modification

1. Bs calculation in HM5.0

- 1st step: BS decision process on             (b) 2nd step: BS unification process on
4 line basis except inside of 8x8               8 line basis

Proposed Bs calculation:

Part 1. Removal of 2nd step (BS unification process)

Part 2. Changing Bs decision process from every 4-pel block to every other 4-pel block in order to reduce the number of Bs calculations.

H0473, Ericsson: Bs and strong filtering decision modifications

Part 1. Proposed Bs calculation: 2nd step unification removed:

It is

proposed to perform loop filtering separately on every 4-pel blocks.

Part 2.	Proposed modifications to strong filtering:

HM5.0 deblocking:

dp0 = |p20 – 2p10 + p00|

dq0 = |q20 – 2q10 + q00|

dp3 = |p23 – 2p13 + p03|

dq3 = |q23 – 2q13 + q03|

Strong filter decision for p0

Proposed:

dp = |p20 – 2p10 + p00| + |p23 – 2p13 + p03|

dq = |q20 – 2q10 + q00| + |q23 – 2q13 + q03|

d = dp + dq

Strong filter decision

d < (β>>2) && ( |p30 – p00| + |q30 – q00|) < (β>>2) …

Same d as in weak filter decisions is used

    - Subtests 3: Results

Table 16 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0146, Panasonic | -0.2% | 0.0% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% | 0.5% | -0.1% |
| H0108, LG | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| H0473, Ericsson | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| H0396, Part 1 Qualcomm | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | 0.3% | -0.2% |
| H0155, SKT/SKKU | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| H0156, SKT/SKKU | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.5% | 0.0% |
| H0084, TI | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% |

Table 17 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0146, Panasonic | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | -0.1% | 0.0% |
| H0108, LG | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% |
| H0473, Ericsson | 0.0% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| H0396, Part 1 Qualcomm | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | -0.2% |
| H0155, SKT/SKKU | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.3% | 0.1% |
| H0156, SKT/SKKU | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.2% | 0.0% |
| H0084, TI | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |

Table 18 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0146, Panasonic | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | -0.1% |
| H0108, LG | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| H0473, Ericsson | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| H0396, Part 1 Qualcomm | 0.0% | -0.1% | -0.1% | -0.4% | 0.1% | 0.0% | 0.3% | 0.3% | -0.1% |
| H0155, SKT/SKKU | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| H0156, SKT/SKKU | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 0.1% |
| H0084, TI | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |

## Complexity – relative decoding time against the anchor

Table 19 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0146, Panasonic | 97% | 100% | 101% | 100% | 100% | 101% | 99% | 99% | 99% |
| H0108, LG | 98% | 99% | 98% | 97% | 100% | 98% | 98% | 99% | 98% |
| H0473, Ericsson | 100% | 100% | 100% | 100% | 100% | 100% | 101% | 100% | 100% |
| H0396, Part 1 Qualcomm | 100% | 100% | 101% | 101% | 100% | 101% | 101% | 101% | 101% |
| H0155, SKT/SKKU | 99% | 98% | 99% | 99% | 98% | 100% | 97% | 100% | 100% |
| H0156, SKT/SKKU | 99% | 98% | 99% | 99% | 98% | 97% | 99% | 98% | 100% |
| H0084, TI1 | 100% | 99% | 100% | 102% | 100% | 100% | 100% | 100% | 100% |

1 Decoding times on a single PC are taken from the cross-checker’s contributions.

### Subtest 3: comments from cross-checkers

- [JCTVC-H0146, Panasonic] CE10.3: Simplified delta calculation in the deblocking filter

  - [JCTVC-H0261, Sony] The BD-bitrates are completely identical with ones provided by Panasonic. The encoding and decoding time are similar with Panasonic’s results. The source code seems to be as described in H0146. H0146 is preferable for SW, but it is necessary to check the visual quality carefully because slight losses were confirmed in Sony’s viewing.

- [JCTVC-H0108, LG] CE10 Subtest3: Simplified BS calculation process in deblocking filter

  - [JCTVC-H0576, Ericsson] The BD-rates match those provided by the proponent. The decoding times are similar to those provided by the proponent. The subjective viewing has been performed. When watching the sequences in motion, one can notice differences between the anchor and the proposal. However, it is difficult to say whether the proposal is consistently worse than the anchor.

- [JCTVC-H0473, Ericsson] CE10.3: Deblocking filter simplifications: Bs computation and strong filtering decision

  - [JCTVC-H0618, LG] The BD-rates are completely matched with ones from Ericsson and shows small difference of decoding time, but it is marginal enough. According to the informal visual test, it’s hard to find out any differences between anchor and the proposal.

- [JCTVC-H0396, Qualcomm] CE10 Subtest 3: Deblocking Filter Simplifications

  - [JCTVC-H0209, SKKU/SKT] PSNR and bitrate results were identical to the results provided by Qualcomm. Since the encoding execution was operated on several PCs, the measured encoding time slightly fluctuated, however, the decoding tests were runs on single PC, and the decoding time was similar to what Qualcomm provided. [

- [JCTVC-H0155, SKT/SKKU] CE10.3: Simplification of First Delta Calculation in Deblocking by SKT/SKKU

  - [JCTVC-H0459, Qualcomm] The encoding timings are approximate and indicate that they are comparable with the HM5 anchor. The decoding timings are measured on a single CPU and are also similar to the anchor. The software changes correspond to the proposal.

- [JCTVC-H0156, SKT/SKKU] CE10.3: Simplification of Second Delta Calculation in Deblocking by SKT/SKKU

  - [JCTVC-H0157, ETRI] The code and results are verified and are conformant to the results stated by SKT/SKKU.

- [JCTVC-H0084, TI] CE10 Subtest 1: Test results on unification of luma and chroma filtering

  - [JCTVC-H0263, Sony] The BD-bitrates are completely identical with ones provided by TI. The encoding and decoding time are similar with TI’s results. The source code seems to be as described in H0084. It is necessary to check the visual quality carefully because slight losses were confirmed in Sony’s viewing.

## Subtest 4: Controlling strength of deblocking filter at slice level

Table 20 Cross-verification assignment Subtest 4

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| H0148, Panasonic | H0350, Sony |
| H0398, Qualcomm | H0656, Toshiba |
| H0574, Ericsson | H0474, Qualcomm |

### Subtest 4: Deblocking filter modifications

Table 21 Parts that have been modified compared to Toshiba’s implementation of deblocking parameters signaling (based on HM5.0)

| Proponent | H0148, Panasonic | H0398, Qualcomm | H0574, Ericsson |
| --- | --- | --- | --- |
| Number of additional parameters compared to HM5.1 | 6 added | 1 (SPS) 1 modified (APS, slice) 3 (APS, slice; or 1 if no intra/inter dependency) | 1 added 1 modified |
| Total number of parameters | 2+6 | 1 (SPS) 4 (APS, slice; or 2 if no intra/inter dependency) | 3 |
| What additional parameters are signaled | 1 On/off-flag 5 Weights to delta values of luma filters (3 for strong, 2 for weak) | deblocking_control_present_flag (SPS) 2 up to 4 params to control tC offset based on transform size | side_beta_offset_div4 beta_offset instead of beta_offset_div2 |
| Parameters change typically (in the provided software): every frame/every sequence/same for all sequences in the test set | On a frame basis Encoder control minimizing MSE for subjectively important luminance samples. Weights are coded together with each frame regardless of the bit rate they need. | Per scene of the sequence or finer | On a frame basis Encoder control minimizing RD-cost |
| Modifications of Bs | No |  | No |
| Modification of deblocking decisions |  |  | No |
| Modification of filtering operations | yes |  | No |
| Remarks |  |  |  |

### Subtest 4: summary of proposals

H0148, Panasonic

HM5.0:

Δ = Clip3 (–tc, tc, ( 9* (q0 – p0) – 3 (q1 – p1) + 8 ) >> 4 )

p0 = Clip( p0 + Δ)

q0 = Clip ( q0 – Δ)

Proposed :

It is proposed to multiply Δ-values (after clipping operation) with scaling factors, which are sent in the slice header. Five different scaling factors are used depending on the pixel position relative to the block boundary (weak and strong filters use different scaling factors at every pixel position).

Δ

α4Δ

Δp1

Δq1

Δ

α5Δ

Quantized scaling factors α1… α5 are sent in the slice header.

Weak filter: α4, α5 for p1, q1,   	0 ≤ αi ≤ 1.2 (in the encoder)

Strong filter: 3 scaling factors, 		0 ≤ αi ≤ 1.3 (in the encoder)

Strong filter is modified in order to produce Δ values.

Example:

Δq2,i=p0i+q0i+q1i−5⋅q2i+2⋅q3i

p2i'=Clipp2i<<6+Δp2,i⋅α1+32>>6

Estimation of parameters:

Scaling factors are estimated in the encoder for every frame, based on MSE (with additional subjective criterion), quantized with a step 1/64 and sent in the slice header.

H0398, Qualcomm

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| ..... |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if (deblocking_filter_control_present_flag) { |  |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| Note that if deblocking_filter_control_present_flag=0, then deblocking_filter_in_aps_enabled_flag=0 and aps_deblocking_filter_flag=0 and disable_deblocking_filter_flag=0 and beta_offset_div2=0 and tc_offset_div2=0 are inferred. |  |
| } |  |
| ... |  |
| } |  |

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| if (deblocking_filter_control_present_flag) { |  |
| inherit_dbl_params_from_APS_flag | u(1) |
| if (!inherit_dbl_params_from_APS_flag){ |  |
| disable_deblocking_filter_flag | u(1) |
| if (!disable_deblocking_filter_flag) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| tc_offset_intra_div2 | se(v) |
| tc_offset_intra_delta | se(v) |
| if ( slice_type != I) { |  |
| tc_offset_inter_div2 | se(v) |
| tc_offset_inter_delta | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| ..... |  |

Semantics:

tc_offset_intra_div2, tc_offset_intra_delta, tc_offset_inter_div2, tc_offset_inter_delta (APS, slice header): usage of these syntax units is illustrated with the following pseudo-code.

- transform_type = NxN (= 4x4 or 8x8 or 16x16 or 32x32)

  - INTRA_TC_OFFSET = (2×tc_offset_intra_div2) + tc_offset_NxN_intra_delta

  - INTER_TC_OFFSET = (2×tc_offset_inter_div2) + tc_offset_NxN_inter_delta

  - BETA_OFFSET = 2×beta_offset_div2

- Tc_offset_NxN_intra_delta = tc_offset_intra_delta × factor_NxN

- Tc_offset_NxN_inter_delta = tc_offset_inter_delta × factor_NxN

- Constant values used in the computation (not signaled):

  - Factor_4x4 = 0

  - Factor_8x8 = 1

  - Factor_16x16 = 2

  - Factor_32x32 = 3

- IF ( Intra Mode ) THEN TcOffset = DEFAULT_INTRA_TC_OFFSET(=2) + INTRA_TC_OFFSET; ELSE TcOffset = INTER_TC_OFFSET

- BetaOffset = BETA_OFFSET

- tC: Q = Clip3(0, MAX_QP + DEFAULT_INTRA_TC_OFFSET, QP + TcOffset )   (luma and chroma deblocking)

- β: Q = Clip3(0, MAX_QP, QP + BetaOffset )    (luma deblocking only)

The following offsets are used (set up in the config. file)

| TU size | Intra (excl. default offset = 2) | Inter |
| --- | --- | --- |
| 4×4 | -4 | 0 |
| 8×8 | -2 | +1 |
| 16×16 | 0 | +2 |
| 32×32 | +2 | +3 |

H0574, Ericsson:

Proposed:

1. Replace beta_offset_div2 with beta_offset

2. Add a parameter side_beta_offset_div4, which adjusts decision between filtering one

| inherit_dbl_params_from_APS_flag | u(1) |
| --- | --- |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset | se(v) |
| tc_offset_div2 | se(v) |
| side_beta_offset_div4 | se(v) |
| } |  |
| } |  |

Semantics:

IndexTC = Clip3(0, MAX_QP+2, QP + Tc_IntraOffset + tc_offset_div2<<1 );

IndexB = Clip3(0, MAX_QP, QP + beta_offset );

IndexSideBeta = Clip3(0, MAX_QP, iQP + side_beta_offset_div4<<2);

Beta = betatable[IndexB]*iBitdepthScale;

BetaSide = betatable_8x8[iIndexSideBeta]*iBitdepthScale;

iTc = tctable [iIndexTC]*iBitdepthScale;

SideThreshold = (iBetaSide+(iBetaSide>>1))>>3;

A frame-wise estimation for parameters beta_offset, tc_offset_div2 and side_beta_offset_div4 is implemented in the encoder based on the cost function Cost = SSD + λ*bits.

### Subtest 4: Results

Table 22 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0148, Panasonic | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% | -0.2% |
| H0398, Qualcomm | -0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| H0574, Ericsson | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% | -0.4% | -1.1% | -0.2% |

Table 23 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0148, Panasonic | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.2% | 0.1% | 0.3% | 0.0% |
| H0398, Qualcomm | -0.2% | -0.2% | 0.1% | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| H0574, Ericsson | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% |

Table 24 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0148, Panasonic | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% | 0.3% | 0.1% | 0.0% |
| H0398, Qualcomm | -0.2% | -0.2% | 0.1% | -0.1% | 0.0% | -0.3% | 0.0% | -0.1% | 0.0% |
| H0574, Ericsson | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.2% | -0.2% | 0.0% |

## Complexity – relative decoding time against the anchor

Table 25 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  | 10 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P | RA |
| H0148, Panasonic | 101% | 101% | 102% | 102% | 100% | 102% | 101% | 100% | 101% |
| H0398, Qualcomm | 100% | 99% | 100% | 99% | 99% | 99% | 101% | 99% | 100% |
| H0574, Ericsson | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

### Subtest 4: comments from cross-checkers

- [JCTVC-H0148, Panasonic] CE10.4: Deblocking filter using adaptive weighting factors

  - [JCTVC-H0350, Sony] The BD-bitrates are completely identical with ones provided by Panasonic. The encoding and decoding time are similar with Panasonic’s results. The source code seems to be as described in H0147. A few sequences described in JCTVC-G1210 are improved in visual quality and other cases are similar compared with HM5.0.

- [JCTVC-H0574, Ericsson] CE10.4: deblocking parameters signaling

  - [JCTVC-H0474, Qualcomm] The BD-rates are identical to those provided by the proponent. The software was studied and was found in general matching the proposal.

- [JCTVC-H0398, Qualcomm] CE10 Subtest 4: Transform Dependent Deblocking Filter Parameter Adjustment

  - [JCTVC-H656, Toshiba]

# Subjective quality

A subjective quality test (informal viewing by experts) will be held during the San Jose meeting.

# Conclusions

In CE10, the proposals from four subtests are evaluated based on the common test conditions. Additionally, all the proposals were required to provide results for LD-P configurations (both HE and LC). The results are summarized based on BD-rates and decoding time. The informal subjective viewing by experts will be held in San Jose in order to assess how the proposals affect the subjective quality.

# References

- F. Bossen, Common test conditions and software reference configurations, JCTVC-G1200, Geneva, Nov. 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, Description of Core Experiment 10: Deblocking filtering, JCTVC-G1210, Geneva, Nov. 2011.