| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0051 |
| --- | --- |

| Title: | Burst transmission of I_PCM blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents simulation results of the Burst I_PCM method proposed in JCTVC-G118. The method allows encoders to send I_PCM sample data of successive I_PCM block without termination associated with entropy coding process. Thereby it significantly enhances the throughput of transmitting PCM sample data of successive I_PCM blocks while reducing some bits of side-information. Simulation results for common test sequences demonstrate that the Burst I_PCM does not affect the BD-rate and time results of HM-5.0-dev-highlevel rev1712. Additional simulation results for a synthesized sequence demonstrate that it reduces the header bitrates of loss-less coded videos by 32kbps or 2.4% without affecting encoding and decoding times. It is recommended that the Burst I_PCM is adopted into HM-6.0 in order to enhance the current HM design.

- Introduction

In the last JCTVC meeting, JCTVC-G118 presented a concept-level idea of burst transmission of I_PCM blocks (hereafter referred to as the Burst I_PCM). The Burst I_PCM allows encoders to transmit PCM sample data of successive I_PCM blocks without termination associated with entropy coding process. Thereby it significantly enhances the throughput of PCM sample data transmission of successive I_PCM blocks while reducing some bits of side-information.

Section 2 reviews the Burst I_PCM and explains its benefit in terms of the data transmission throughput and side-information reduction. Section 3 reports simulation results using HM-5.0-dev-highlevel (rev. 1712) that integrates the restriction in the maximum I_PCM size (32x32) proposed in JCTVC-G112.

- Burst I_PCM transmission

At first, we describe how the current HM transmits successive I_PCM blocks in the same depth of a CTB and explain its problem. Then we describe the Burst I_PCM and how it handles the problem.

2.1	Successive I_PCM encoding in HM

The coded representation of an I_PCM block comprises of variable-length data of PU header (pred_type, pcm_flag, and pcm_alignment_zero_bit) and fixed-length data of PCM sample (pcm_sample_luma and pcm_sample_chroma). When successive CUs in the same layer of a CTB are I_PCM blocks, the variable-length data and fixed-length data of the successive I_PCM blocks are interleaved. Figure 1(a) shows an example that a certain layer of CTB contains one non-I_PCM block and three I_PCM blocks. Figure 1(b) illustrates the bistream of Figure 1(a). As it can be deduced from Figure 1 (b), throughput of transmitting PCM sample data of three I_PCM blocks is affected due to the presence of the variable-length data in the second and third PU headers. Specifically, the current I_PCM signaling need to terminate PCM sample data transmission at each of I_PCM blocks. Therefore it affects the throughput of transmitting PCM sample data of successive I_PCM blocks and increases some bits of side-information.

Figure 1: (a) Example of CTB that contains three successive I_PCM blocks. (b) HM bistream of (a). (c) Proposed bitstream of (a). Note that PU header (pred_type, pcm_flag, and pcm_alignment_zero_bit) is variable length data while PCM sample is fixed length data.

2.2	Burst I_PCM transmission

In the PU header of the first I_PCM block, the Burst I_PCM sends the number of subsequent I-PCM blocks that successively follow the first I-PCM block in the same layer of a CTB. Then the Burst I_PCM sends PCM sample data of the successive I_PCM blocks specified by that number plus 1. Figure 1 (c) illustrates the bitstream of Figure 1 (a) encoded. Notice that PCM sample data of the two subsequent I_PCM blocks (block 2 and 3) immediately follow that of the first I_PCM block, that is, it skips signaling of the PU headers of the two subsequent I_PCM blocks. This allows encoder to send PCM sample data of the three successive I_PCM blocks without termination associated with entropy coding process. In this way, the Burst I_PCM can enhance the throughput of PCM sample data transmission of successive I_PCM blocks while reducing some bits of side-information.

The number of subsequent I-PCM blocks is specified as num_subsequent_pcm syntax. (Note that num_successive_pcm_minus1 may be an alternative good name.) The bit representation of num_subsequent_pcm syntax is based on TU binarization as shown in Table 1. The bit representation of a num_subsequent_pcm value is directly appended to the output bitstream of that EncodeTerminate process in a way similar to pcm_alignment_zero_bit. This design avoids having changes of the current specification of the DecodeTerminate process for pcm_flag. In addition, since it requires no arithmetic encoding and decoding processes, there are no complexities in encoding and decoding processes for num_subsequent_pcm syntaxes.

Table 1: Binary representation of num_subsequence_pcm syntax.

| Num_subsequent_pcm | Bit representation |  |  |
| --- | --- | --- | --- |
|  | 1st | 2nd | 3rd |
| 0 | 0 |  |  |
| 1 | 1 | 0 |  |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 1 |

2.3	Complexity analysis of burst I_PCM transmission

As it can be inferred from Section 2.2, the additional complexity introduced by the Burst I_PCM is negligibly small. What HM decoder needs to do additionally is two things, 1) Paring of num_subsequent_pcm and 2) Conditional skipping of parsing of PU headers (and end_of_slice when FGS is used). The paring of num_subsequent_pcm is less complex as explained in Section 2.2. The conditional skipping is also less complex since the condition depends only the value of the number of the successive number of I_PCM blocks indicated by a num_subsequent_pcm value. Specifically, HM decoder skips parsing of PU headers num_subsequent_pcm times.

The Burst I_PCM gives some freedom in encoder implementation. For one implementation that is not able to count the number of successive I_PCM blocks, the implementation can always send num_subsequent_pcm=0. This may implies a potential disadvantage that results in some side-information increases. However, the side-information increase is expected to be negligible because of the presence of pcm_alignment_zero_bit. For example, if we assume the uniform distribution of the remainder of total bits divided by 8, the expectation of the side-information increase is roughly 1bit. (0.125*8 + 0.125*0 + 0.125*0  … + 0.125*0). For another implementation that counts the number of successive I_PCM blocks, its counting process is also less complex since it counts the successive I-PCM blocks in the same layer of a CTB. (Indeed the counting process is less complex than process for checking neighboring available PUs. In our HM implementation, the counting process is written in about 100 lines.)

# Simulation results

3.1	Experimental setup

The Burst I_PCM described in Section 2 is implemented into the HM-5.0-dev-highlevel rev. 1712 which integrates the restriction in the maximum I_PCM size (32x32) proposed in JCTVC-G112. ("JCTVC-H0051-HM-5.0-dev-highlevel-rev1712-burstPCM.patch" is the patch.) Their 64-bit encoder and decoder executables were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platform listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Simulation for common test sequences were conducted according to JCTVC-G1200 using parameters PCMEnabledFlag=1, PCMLog2MaxSize=5, and PCMLog2MinSize=3. The results are described in 3.2. Since I_PCM mode is not selected in the common test sequences with the common coding conditions, additional simulation results for a synthesized sequence using lower QP values are described in 3.3.

3.2	Results for common test sequences

An attached Excel sheet, JCTVC-H0051-results.xls, shows the detailed results of the Burst I_PCM. Table 2 shows the summary results. (Notice that since I_PCM mode is not selected, the bit rates and PSNR values of the original and modified HM-5.0-dev-highlevel rev. 1712 results are identical. ) It is observed that the Burst I_PCM does not affect encoding and decoding times as it is expected from the complexity analysis in Section 2.3.

Table2: Summary of simulation results

3.3	Additional results for synthesized noisy sequence

This subsection provides additional results for a synthesized sequence depicted in Figure 2. The synthesized sequence is Gaussian noise, which is difficult to compress by using prediction, transform coding, and entropy encoding. Some videos can partially contain textures whose nature is similar to Gaussian noise. The synthesized sequence is generated by using JCTVC-E057-NEC-Sandstorms-Software available from [4].

Figure 2: Synthesized sequence, Sandstorms, (CIF 30Hz 30frames)

Table 3 and Table 4 show HE and LC RA coding results for Sandstorms by using QP values 0, 4, 8, and 12. (An attached txt "JCTVC-H0051-sandstorms-enc-batch" describes MS-DOS script to run the HE and LC RA coding for Sandstorms.) "Header kbps" column shows the header bitrates derived by subtracting the uncompressed bitrates from the compressed bitrates (i.e., header bitrates = compressed bitrates - 30(Hz) x 8(bit) x 352 x 288 x 3 / 2). From Table 3 and Table 4, it is observed that the Burst I_PCM significantly reduces the header bitrates by about 32kbps or 2.4% while not affecting the encoding and decoding times.

The significant reduction in header bitrates is explained as follows: The current HM adopts the restriction in the maximum I_PCM size (32x32) proposed in JCTVC-G112. When a 64x64-LCU region is subject to I_PCM coding, it needs to split the 64x64-LCU into 4 CUs and encode each of them separately. That requires some PU headers in sending the last three I_PCM CUs. The Burst I_PCM skips signaling of the PU headers and results in significant reduction in header bit rates. (Notice that the Burst I_PCM also enhances the throughput of PCM sample data transmission of successive I_PCM blocks.)

Table 5 and Table 6 show additional HE and LC RA coding results that the encoder forces num_subsequent_pcm=0, i.e., the encoder trasmits each of I_PCM blocks separately as the current HM does. (The separate I_PCM encoding is enabled by setting macros "SERIAL_ENCODING" to 1. ) As expected from the analysis in Section 2.3, it is observed from Table 5 and 6 that the separate I_PCM encoding does not change the bitrates due to the presence of pcm_aslignment_zero_bit.

Table3: HE RA coding results. (All are loss-less coded and PSNR values are not shown.)

Table4: HE LC coding results. (All are loss-less coded and PSNR values are not shown.)

Table5: HE RA coding results using serial encoding. (All are loss-less coded and PSNR values are not shown.)

Table6:LC RA coding results using serial encoding. (All are loss-less coded and PSNR values are not shown.)

# Conclusions

This contribution has described the Burst I_PCM. Simulation results have demonstrated that the Burst I_PCM reduces the header bitrates of loss-less coded videos by 2.4% without affecting encoding and decoding times. It is recommended that the Burst I_PCM is adopted into HM-6.0.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Chono et al., "Potential enhancement of signaling of I_PCM blocks," JCTVC-G118, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- R. Hattori et al., "Fast bypass mode for CABAC," JCTVC-F177, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- T. Hellman et al., "Changing luma/chroma coefficient interleaving from CU to TU level," JCTVC-G112, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- http://phenix.int-evry.fr/jct/doc_end_user/documents/5_Geneva/wg11/JCTVC-E057-v5.zip

Appendix A: Changes in WD text

Modified portions in 7.3.4, 7.3.5, and 7.3.7 are highlighted with red font and yellow color background.

### 7.3.4. Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| XLCU = HorLumaLocation( CurrTbAddr ) |  |
| YLCU = VerLumaLocation( CurrTbAddr ) |  |
| NumPCMBlock = 0 |  |
| moreDataFlag = coding_tree( XLCU, YLCU, Log2TbSize, 0 ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| if( ( CurrTbAddr = = firstTbInTileAddr ) && tile_boundary_independence_flag ) |  |
| rbsp_trailingbits() |  |
| } while( moreDataFlag ) |  |
| } |  |

7.3.5 Coding Tree syntax

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress NumPCMBlock == 0 ) { |  |
| if( !entropy_coding_mode_flag && slice_type != I ) |  |
| cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| else if( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| if( cu_qp_depta_enabled_flag && log2CUSize == log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if(NumPCMBlock == 0) |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| else |  |
| NumPCMBlock-- |  |
| if(NumPCMBlock == 0) { |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| } |  |
| else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CUSize <= Log2MaxIPCMCUSize && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | ue(v) |
| NumPCMBlock= num_subsequent_pcm + 1 |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( j = 0; j < NumPCMBlock; j++ ){ |  |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |
| NumPCMBlock-- |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N. When the pcm_flag is not present, it shall be inferred to be equal to 0.

### num_subsequent_pcm specifies the number of subsequent I_PCM coding units with the current log2CUSize that successively follow the current I_PCM coding unit. The values of pcm_flags of the subsequent I_PCM coding units are set to 1. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive. When the num_subsequent_pcm is not present, it shall be inferred to be 0.

The bit representation of a num_subsequent_pcm value is specified in Table X.

Table X: Bit representation of num_subsequence_pcm syntax.

| num_subsequent_pcm | Bit representation |  |  |
| --- | --- | --- | --- |
|  | 1st | 2nd | 3rd |
| 0 | 0 |  |  |
| 1 | 1 | 0 |  |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 1 |

In "9.2	CABAC parsing process for slice data", the last paragraph

"In case the request for a value of a syntax element is processed for the syntax element mb_type and the decoded value of mb_type is equal to I_PCM, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.2.1.2."

is replaced by

"In case the request for a value of a syntax element is processed for the syntax element pcm_flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any num_subsequent_pcm and pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.2.1.2."

Appendix B: Simplified version of Appendix A.

With a new definition of pcm_unit () below, 7.3.5 and 7.3.7 can described in more simplified way.

7.3.5 Coding Tree syntax

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress NumPCMBlock == 0 ) { |  |
| if( !entropy_coding_mode_flag && slice_type != I ) |  |
| cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| else if( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| if( cu_qp_depta_enabled_flag && log2CUSize == log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if(NumPCMBlock == 0) |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| else |  |
| pcm_unit( x0, y0, log2CUSize ) |  |
| if(NumPCMBlock == 0) { |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| } |  |
| else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CUSize <= Log2MaxIPCMCUSize && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | ue(v) |
| NumPCMBlock= num_subsequent_pcm + 1 |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_unit( x0, y0, log2CUSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.11	Pcm unit syntax ()

| pcm_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock-- |  |
| } |  |