Modifications to "JCTVC-G1103_d9.doc"

- Modifications to syntax 7.3.4, 7.3.5, and 7.3.7. (highlighted with red font and yellow color background)

- Addition of a new syntax table 7.3.9 Pcm_unit( )

- Specification of a new syntax element after pcm_flag

- Modification of a paragraph in 9.2 CABAC parsing process for slice data (highlighted with red font and yellow color background)

- Add a new descriptor tu(n) for the new syntax element num_subsequent_pcm in 7.2.

- Use tu(3) for num_subsequent_pcm; remove the table from the description of num_subsequent_pcm.

- Add a subsection in section 9 "Parsing process for truncated unary codes".

### Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| XLCU = HorLumaLocation( CurrTbAddr ) |  |
| YLCU = VerLumaLocation( CurrTbAddr ) |  |
| NumPCMBlock = 0 |  |
| moreDataFlag = coding_tree( XLCU, YLCU, Log2TbSize, 0 ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| if( CurrTbAddr = = firstTbInTileAddr ) |  |
| rbsp_trailingbits() |  |
| } while( moreDataFlag ) |  |
| } |  |

### Coding tree syntax

| coding_tree( x0, y0, log2CUSize, cuDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress && log2CUSize > Log2MinCUSize && NumPCMBlock == 0 ) { |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( cu_qp_delta_enabled_flag && log2CUSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1, cuDepth + 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1, cuDepth + 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if( NumPCMBlock == 0 ) |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| else |  |
| pcm_unit( x0, y0, log2CUSize ) |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) && NumPCMBlock == 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

### 7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CUSize >= Log2MinIPCMCUSize && log2CUSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock= num_subsequent_pcm + 1 |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_unit( x0, y0, log2CUSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_lc_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.9	Pcm unit syntax ()

| pcm_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock-- |  |
| } |  |

pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be inferred to be equal to 0.

### num_subsequent_pcm specifies the number of subsequent I_PCM coding units with the current log2CUSize that successively follow the current I_PCM coding unit in the same depth of a TB. The values of pcm_flags of the subsequent coding units are set equal to 1. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent I_PCM coding units are identical. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive.

Table X: Bit representation of num_subsequence_pcm syntax.

pcm_alignment_zero_bit is a bit equal to 0.

In "9.2	CABAC parsing process for slice data", the last paragraph

"In case the request for a value of a syntax element is processed for the syntax element mb_type and the decoded value of mb_type is equal to I_PCM, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.2.1.2."

is replaced by

"In case the request for a value of a syntax element is processed for the syntax element pcm_flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and num_subsequent_pcm and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.2.1.2."

In "7.2	Specification of syntax functions and descriptors", add a new descriptor tu(v) between te(v) and u(n) as follows:

–	te(v): truncated Exp-Golomb-coded syntax element with left bit first. The parsing process for this descriptor is specified in subclause Error: Reference source not found.

–	tu(v): truncated unary coded syntax element with left bit first. The parsing process for this descriptor is specified in subclause Error: Reference source not found.

–	u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

In "9	Parsing process", the following paragraph

"This process is invoked when the descriptor of a syntax element in the syntax tables in subclause Error: Reference source not found is equal to ue(v), me(v), se(v), te(v) (see subclause Error: Reference source not found), or ae(v) (see subclause Error: Reference source not found)."

is replaced by

"This process is invoked when the descriptor of a syntax element in the syntax tables in subclause Error: Reference source not found is equal to ue(v), me(v), se(v), te(v), tu(v) (see subclause Error: Reference source not found), or ae(v) (see subclause Error: Reference source not found)."

In "9.1	Parsing process for Exp-Golomb codes", the following paragraph

"Syntax elements coded as ue(v), me(v), or se(v) are Exp-Golomb-coded. Syntax elements coded as te(v) are truncated Exp-Golomb-coded. "

is replaced by

"Syntax elements coded as ue(v), me(v), or se(v) are Exp-Golomb-coded. Syntax elements coded as te(v) are truncated Exp-Golomb-coded. Syntax elements coded as tu(v) are truncated unary coded."

In "9.1	Parsing process for Exp-Golomb codes", the following paragraph

"–	Otherwise (the syntax element is coded as te(v)), the range of possible values for the syntax element is determined first. The range of this syntax element may be between 0 and x, with x being greater than or equal to 1 and the range is used in the derivation of the value of the syntax element value as follows.

–	If x is greater than 1, codeNum and the value of the syntax element is derived in the same way as for syntax elements coded as ue(v).

–	Otherwise (x is equal to 1), the parsing process for codeNum which is equal to the value of the syntax element is given by a process equivalent to:

b = read_bits( 1 ) 	(9-1)
codeNum = !b

"

is replaced by

"–	Otherwise, if the syntax element is coded as te(v), the range of possible values for the syntax element is determined first. The range of this syntax element may be between 0 and x, with x being greater than or equal to 1 and the range is used in the derivation of the value of the syntax element value as follows.

–	If x is greater than 1, codeNum and the value of the syntax element is derived in the same way as for syntax elements coded as ue(v).

–	Otherwise (x is equal to 1), the parsing process for codeNum which is equal to the value of the syntax element is given by a process equivalent to:

b = read_bits( 1 ) 	(9-2)
codeNum = !b

–	Otherwise (the syntax element is coded as tu(v)), the value of the syntax element is derived by invoking the mapping process as specified in subclause 9.1.2 with codeNum as the input.

"

Add the following subsection after subclause 9.1.1.

"9.1.2	Mapping process for truncated unary codes

Input to this process is codeNum as specified in subclause Error: Reference source not found.

Output of this process is a value of a syntax element coded as tu(v).

The syntax element value synVal is derived from a process equivalent to the following pseudo-code.

synVal = 0 
for (i = 0; i < codeNum; i++){
	bit = read_bits( 1 ) 
	if( !bit ) 
		break
	synVal ++
}

"