| いJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0053 |
| --- | --- |

| Title: | CE4 Subtest 1: Rate control friendly spatial QP prediction based on intra prediction (test 1.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Masaaki Kobayashi, Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com +81-3-3758-2111 kobayashi.masaaki517@canon.co.jp shima.masato@canon.co.jp +1 858 658 3973 mcoban@qualcomm.com martak@qualcomm.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | NEC Corporation, Canon Inc., Qualcomm Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of rate control friendly spatial QP prediction based on intra prediction, which has been proposed in JCTVC-G1028 and is tested as the test 1.3 in CE4 Subtest 1. For CUs coded with angular intra prediction, the method employs a neighbouring CU within the current LCU along intra prediction direction for QP prediction. For other CUs, the method predicts QP as the average of reference QPs selected from above and left CUs within the current LCU. The experimental results show that, under the test condition of CE4 Subtest 1, the proposed QP prediction achieves coding efficiency improvement of 0.5% for AI-HE and AI-LC configurations, 0.3% for RA-HE, RA-LC and LB-LC configurations, and 0.2% for LB-HE and RA-HE10 configurations, compared with the AVC-style QP prediction.

# Introduction

At the last meeting, results of CE4 Subtest 1 on QP coding [1] were reviewed. Among spatial QP prediction methods listed in CE4 Subtest 1 at the 7th JCT-VC meeting, the test 1.3.c (JCTVC-G067) method [2] based on intra prediction direction and prediction mode showed the best R-D performance. However, two major concerns were raised on: (1) adaptability to rate control to avoid CPB overflow and (2) complexity. JCTVC-G1028 [3] was then presented as a solution for those concerns by introducing two conceptual modifications into the test 1.3.c as follows.

- QP reference across LCU boundary is prohibited. This concept is included in test 2.3.c (JCTVC-F332) [4] of the CE4 Subtest 2 at the 6th JCT-VC meeting. Previously-coded QP is employed for reference if no CUs are available within LCU, i.e., if the current CU is located at the top left of the current LCU. This modification introduces LCU-level adaptability to rate control requirement to avoid CPB overflow.

- For simplification, prediction mode dependent QP prediction, which is 2.3.f (JCTVC-F300) [5] of the CE4 Subtest 2 at the 6th JCT-VC meeting, is removed.

CE4 Subtest 1 [6] was then again established at the 7th JCT-VC meeting in order to evaluate the performance of alternative QP prediction methods including JCTVC-G1028. JCTVC-G1028 is listed in CE4 Subtest 1 as the test 1.3. This contribution reports the experimental results of the test 1.3 method.

# Algorithm description

For deriving sample prediction information, the proposed method employs the top-left-most CU and the top-left-most PU of each set of CUs where QP is assigned, which is referred to as quantisation group of coding units in the current WD [7] and hereafter denoted as QGCU in this contribution. Along the derived intra prediction direction, QP of left adjacent QGCU or above adjacent QGCU is set to the predicted QP. Figure 1 illustrates CU, PU and QGCUs employed for the proposed QP prediction. In order to ensure LCU-level adaptability to rate control requirement to avoid CPB overflow, each QGCU which is outside of the LCU is regarded as unavailable, as shown in Figure 2.

Brief algorithm of the proposed method is described as follows.

- If the top-left-most CU in the current QGCU is intra-coded, the predicted QP is derived along intra prediction direction of the top-left-most PU with the JCTVC-G357 method [8], as shown in Figure 3.

- Otherwise if at least either of the above QGCU or the left QGCU is available within the LCU, predicted QP is calculated by averaging QPs of those QGCUs available.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

Detailed textual algorithm description in the form of WD text is shown in Appendix A.

Figure 1: CU, PU and neighbouring QGCUs employed for QP prediction of the proposed method.

Figure 2: QP reference relationship between QGCUs.

Figure 3: QP prediction along intra prediction direction.

# Experiment

## Setup

The proposed QP prediction algorithm has been implemented onto the base software for CE4 Subtest 1 [6]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 1 test conditions, with all applicable MinCUDQPSize values, i.e. 8x8, 16x16, 32x32 and 64x64.

The computing platform used for the experiments is shown below.

- OS: Linux 64-bit

- CPU: Intel Xeon X5670 2.93GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 48GiB

## Results

Tables 1 and 2 show the summary of experimental results. The brief summary of complete results is shown in Table 1. Tables 2 shows the summary of experimental results with MinCUDQPSize equal to 8x8, i.e., with the mandatory configuration of CE4 Subtest 1. More detailed results are shown in the attached spreadsheets.

Table 1: Brief summary of complete results.

| MinCUDQP | BD-rates |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC | RA HE-10 |
| 8x8 | -0.52% | -0.45% | -0.32% | -0.34% | -0.25% | -0.28% | -0.21% |
| 16x16 | -0.22% | -0.17% | -0.17% | -0.14% | -0.17% | -0.13% | -0.12% |
| 32x32 | -0.04% | -0.03% | -0.04% | -0.05% | -0.07% | -0.07% | -0.08% |
| 64x64 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% |
| MinCUDQP | dQP code amount ratio |  |  |  |  |  |  |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC | RA HE-10 |
| 8x8 | -11.48% | -12.12% | -6.85% | -6.68% | -4.70% | -4.52% | -5.12% |
| 16x16 | -9.21% | -9.26% | -5.39% | -5.32% | -4.46% | -4.22% | -4.04% |
| 32x32 | -4.15% | -4.08% | -2.65% | -2.54% | -2.60% | -2.56% | -2.03% |
| 64x64 | 0.00% | 0.00% | 0.02% | 0.01% | 0.04% | 0.01% | -0.02% |

Table 2: Summary results of test 1.3 against test 1.1 with MinCUDQPSize equal to 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.4% | -0.4% | -0.4% | -8.1% | -0.3% | -0.3% | -0.3% | -8.4% |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.4% | -10.7% | -0.3% | -0.3% | -0.3% | -11.1% |  |  |  |  |
| Class C | -0.6% | -0.5% | -0.5% | -12.5% | -0.5% | -0.5% | -0.5% | -12.9% |  |  |  |  |
| Class D | -0.5% | -0.4% | -0.5% | -12.4% | -0.4% | -0.4% | -0.4% | -12.6% |  |  |  |  |
| Class E | -0.7% | -0.7% | -0.7% | -12.4% | -0.8% | -0.7% | -0.8% | -14.5% |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.5% | -11.5% | -0.5% | -0.4% | -0.5% | -12.1% |  |  |  |  |
|  | -0.5% | -0.5% | -0.5% |  | -0.5% | -0.4% | -0.4% |  |  |  |  |  |
| Class F | -0.2% | -0.4% | -0.3% | -8.2% | -0.2% | -0.3% | -0.3% | -9.2% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.1% | -0.3% | -4.9% | -0.4% | -0.2% | -0.2% | -5.0% | -0.1% | -0.1% | 0.1% | -4.0% |
| Class B | -0.3% | -0.2% | -0.2% | -6.1% | -0.3% | -0.1% | 0.0% | -5.8% | -0.3% | -0.3% | -0.2% | -6.1% |
| Class C | -0.4% | -0.1% | -0.4% | -7.9% | -0.4% | -0.1% | -0.2% | -7.7% |  |  |  |  |
| Class D | -0.3% | -0.6% | -0.5% | -7.7% | -0.4% | -0.6% | -0.3% | -7.6% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.4% | -6.9% | -0.3% | -0.2% | -0.2% | -6.7% | -0.2% | -0.2% | -0.1% | -5.1% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.2% | -0.2% |  | -0.2% | -0.2% | -0.1% |  |
| Class F | -0.2% | -0.3% | -0.3% | -5.9% | -0.3% | -0.4% | -0.2% | -6.3% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -3.9% | -0.2% | 0.0% | -0.3% | -3.2% |  |  |  |  |
| Class C | -0.3% | -0.4% | -0.3% | -5.9% | -0.4% | -0.4% | -0.1% | -5.6% |  |  |  |  |
| Class D | -0.3% | -0.7% | 0.3% | -5.7% | -0.3% | -0.6% | -1.0% | -5.5% |  |  |  |  |
| Class E | -0.1% | -0.4% | -0.3% | -3.0% | -0.3% | 0.2% | -1.3% | -4.1% |  |  |  |  |
| Overall | -0.2% | -0.4% | -0.1% | -4.7% | -0.3% | -0.2% | -0.6% | -4.5% |  |  |  |  |
|  | -0.2% | -0.4% | -0.1% |  | -0.3% | -0.3% | -0.6% |  |  |  |  |  |
| Class F | -0.2% | -0.2% | -0.2% | -3.4% | -0.1% | 0.9% | 0.5% | -3.2% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |  |  |  |  |

# Remarks

The current software implementation contains conditional branches depending on the PredMode of the top-left-most CU and availability of neighbouring QGCUs. Although its implementation cost does not look negligible at a glance, branches can be removed with some look-up table, and so on.

# Conclusion

This contribution has reported the experimental results of results of rate control friendly spatial QP prediction based on intra prediction, which is test 1.3 of CE4 Subtest 1. The experimental results show that, under the test condition of CE4 Subtest 1, the proposed QP prediction achieves coding efficiency improvement of 0.5% for AI-HE and AI-LC configurations, 0.3% for RA-HE, RA-LC and LB-LC configurations, and 0.2% for LB- LB-HE and RA-HE10 configurations, compared with the AVC-style QP prediction. The computational cost of the proposed method is negligible. It is recommended that the presented spatial QP prediction based on intra prediction be incorporated into the next version of HM as the QP prediction method of HEVC.

# References

[1] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-F904, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[2] H. Aoki, et al., “CE4 Subtest 1: Spatial QP prediction based on intra prediction and prediction mode (test 1.3.c),” JCT-VC document JCTVC-G067, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[3] H. Aoki, et al., “Non-CE4: Rate control friendly spatial QP prediction,” JCT-VC document JCTVC-G1028, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[4] M. Coban and M. Karczewicz, “CE4 Subtest 2: QP prediction from spatially neighboring CUs (test 2.3.b, 2.3.c),” JCT-VC document JCTVC-F332, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[5] M. Kobayashi and M. Shima, “CE4 Subtest 2: Delta QP prediction results of test 2.2.b and 2.3.f,” JCT-VC document JCTVC-F300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[6] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, July 2011.

[7] B. Bross, et al., “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC document JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, July 2011.

[8] J. Xu et al, “Non-CE4: Improvement of Intra prediction based QP prediction,” JCT-VC document JCTVC-G357, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 5

The modified or added portions are highlighted with yellow colour.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantisation parameter, QPY, of the left neighbor quantisation group of coding units in the current slice. If the left neighbor quantisation group in the current slice is not available, QPY,PREV is the luma quantisation parameter, of the previous quantisation group in decoding order in the current slice. For the first quantisation group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED. .

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Default quantisation parameter prediction process

Inputs to this process are

–	a variable CurrTbAddr specifying the treeblock address of the treeblock covering the current quantisation group of coding units,

–	a variable tbAddrAbove specifying the treeblock address of the treeblock covering the above neighbouring quantisation group of coding units,

–	a variable tbAddrLeft specifying the treeblock address of the treeblock covering the left neighbouring quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If all of the following conditions are true, then QPY, PRED is derived as defined in the equation 8-xx3.

- The above neighbouring quantisation group of coding units in the current slice is available.

- tbAddrAbove is equal to CurrTbAddr.

- The left neighbouring quantisation group of coding units in the current slice is available.

- tbAddrLeft is equal to CurrTbAddr.

QPY,PRED = ( ( ( QPY,ABOVE + QPY,LEFT + 1 ) / 2               	(8-53)

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available and tbAddrAbove is equal to CurrTbAddr, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available and tbAddrLeft is equal to CurrTbAddr, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the previous quantisation group of coding units in decoding order in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Intra quantisation parameter prediction process

Inputs to this process are

–	a syntax element pcm_flag specifying if the top-left coding unit of the current quantisation group of coding units is coded in PCM mode,

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable CurrTbAddr specifying the treeblock address of the treeblock covering the current quantisation group of coding units,

–	a variable tbAddrAbove specifying the treeblock address of the treeblock covering the above neighbouring quantisation group of coding units,

–	a variable tbAddrLeft specifying the treeblock address of the treeblock covering the left neighbouring quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- pcm_flag is equal to 1.

- intraPredMode is equal to 0 (Intra_Planar) or 3 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 1 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 2 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-7.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is greater than 4 and less than 18, the above neighbouring quantisation group of coding units in the current slice is available, and tbAddrAbove is equal to CurrTbAddr, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is greater than 21 and less than 34, the left neighbouring quantisation group of coding units in the current slice is available, and tbAddrLeft is equal to CurrTbAddr, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.