| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0056 |
| --- | --- |

| Title: | AHG13: On interlace indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

- Abstract

This contribution presents the following approach and technical proposals toward interlace support:

- Define interlace indication and other post-processing indications separately by considering which information should be conveyed at a sequence level or at a picture level; if unified sequence/picture level SEI messages that convey all post-processing information are defined, signal the interlace indication in those SEI messages.

- Modifications to the AHG13 text: Move VUI flag to SPS and reduce the bit length of the field of interlace indication SEI message from 4 to 1;

- Modified access unit delimiter RBSP to indicate dynamically changing source material properties.

It is recommended that JCT-VC discusses the design of interlace indication and creates text during the 8th JCTVC meeting to be integrated into the CD.

- Background

In the last JCTVC meeting, AHG13 Interlace Indication was established to support interlace format in HEVC. The starting candidate text specifies a SEI message, interlaced_field_indication(), and a VUI flag, interlaced_field_indication_present_flag: The SEI message conveys the information about interlaced fields of the associated coded pictures, and the presence of the new SEI message in each of access units is specified by the interlaced_field_indication_present_flag value.

The following opinions on the starting candidate text were expressed :

- Move the VUI flag to SPS in order to simplify the signaling of the type of a coded sequence;

- Reserve more bits of the field of interlaced_field_indication() to define other SEI messages that indicate other ways of interpreting the decoded arrays;

- Use AVC style indication ( conditional signaling of bottom_field_flag in slice headers );

- Define an unified sequence level SEI message that contains information at a sequence level about characteristics of the sequence (e.g., color space, progressive/interlace, etc);

- Define an unified picture level SEI message that contains information at a picture level about characteristics of the picture (e.g., top/bottom indication for interlaced video, stereoscopic side-by-side/mono indication for frame-compatible stereo video, etc);

- Define a picture-level signaling means other than SEI message to indicate dynamically changing source material properties.

# Proposed approach toward interlace support

We think that the definition of the unified sequence-level and picture-level SEI messages is an elegant design. However, we think that interlace indication and other post-processing indications are separate issues. In addition, which video characteristics, excluding progressive/interlace, should be static in a coded sequence are not well defined at this time, at least from the current HM design perspective. Therefore we propose the following approach:

- Define interlace indication and other post-processing indications separately by considering which information should be conveyed at a sequence level or at a picture level;

- If the unified sequence/picture level SEI messages are defined, convey the interlace indication as well as other post-processing indications in those SEI messages.

# Modifications to AHG13 interlace indication

As discussed in the last JCTVC meeting, we think that for interlace support, an essential information at a sequence level is whether a coded sequence is progressive video or interlace video, and essential information at a picture level is whether a coded picture is top field or bottom field if the coded sequence is interlace. The AHG13 text supports the signaling of the essential information at both sequence and picture level. In order to improve its usability, we propose two modifications to the AHG 13 text as explain in the following subsections. (Note that if the unified sequence/picture level SEI messages are defined, field_flag and interlaced_field_indication syntaxes described below should be conveyed in those SEI messages.)

3.1	Move interlaced_field_indication_present_flag to SPS

Since the current HM does not handle decoded pictures coded with different picture resolutions, it is impossible to use picture-level adaptive frame/field control for given a sequence. Therefore we propose to move the VUI flag to SPS and change its definition to specify the type of the associated coded sequence and the presence of interlaced_field_indication() in each of access units. The SPS flag can be specified as follows:

field_flag specifies whether coded pictures of the associated coded sequence consist of frames only or fields only. If the value of field_flag is 0, the coded pictures of the associated coded sequence consist of frames only and a variable FieldPicOnlyFlag is set to 0. Otherwise (the value of field_flag is 1), the coded pictures of the associated coded sequence consist of fields only and the variable FieldPicOnlyFlag is set to 1. The values of field_flags in the associated seq_parameter_set_rbsps of a coded sequence shall be identical.

3.2	Modification to interlaced_field_indication()

If field_flag is 1, the associated coded sequence is interlace and what need to be signaled in each of interlaced_field_indication() is only the field of the associated coded picture. We propose reducing the bit length of the field from 4 to 1 as follows:

A.1.X	Interlaced field indication SEI message syntax

| interlaced_field_indication( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| interlaced_field_indication | 5 | u(1) |
| } |  |  |

- Table D-1 – Interpretation of interlaced_field_indication

| Value | Indicated field of picture |
| --- | --- |
| 0 | top field |
| 1 | bottom field |

# Modified access unit delimiter for signaling of dynamically changing source material properties

Access unit delimiter RBSP may be used to indicate the type of slices present in the associated coded picture and to simplify the detection of the boundary between access units. The access unit delimiter RBSP does not specify any normative decoding process, and thus it may also indicate the associated coded picture structure that can be different picture-by-picture.

For example, if a coded sequence contains  interlace videos, the structure of some of the coded pictures is top field and that of the remaining coded pictures is bottom field. The following shows the (tentative) modifications (highlighted with red font and yellow background) to the current access unit delimiter RBSP in order to signal frame, top field, and bottom field:

7.3.2.5	Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| primary_pic_type | u(3) |
| primary_pic_struct | u(2) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.4.2.5 Access unit delimiter RBSP semantics

The access unit delimiter may be used to indicate the type of slices present in a primary coded picture and to simplify the detection of the structure of a primary coded picture as well as the boundary between access units. There is no normative decoding process associated with the access unit delimiter.

primary_pic_type indicates that the slice_type values for all slices of the primary coded picture are members of the set listed in Table 7-2 for the given value of primary_pic_type.

primary_pic_struct indicates the structure of the primary coded picture specified by Table  7-3.

Table 7-2 – Meaning of primary_pic_type

| primary_pic_type | slice_type values that may be present in the primary coded picture |
| --- | --- |
| 0 | 2 |
| 1 | 0, 2 |
| 2 | 0, 1, 2 |

Table 7-3 – Meaning of primary_pic_struct

| primary_pic_struct | Indicated picture structure |
| --- | --- |
| 0 | frame |
| 1 | top field |
| 2 | bottom field |
| 3 | unspecified |

Notice that the access unit delimiter RBSP indicates dynamically changing source material properties. Thus, for example, it can be further modified (highlighted with blue font and green background) to handle the use of frame-compatible stereoscopic video as follows:

7.3.2.5	Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| primary_pic_type | u(3) |
| primary_pic_struct | u(2) |
| primary_frame_packing | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 7-4 – Meaning of primary_frame_packing

| primary_frame_packing | Interpretation |
| --- | --- |
| 0 | mono |
| 1 | stereo side-by-side |
| 2 | stereo top-bottom |
| 4-8 | reserved |

# Conclusions and recommendations

This contribution has presented the following approach and technical proposals toward interlace support:

- Define interlace indication and other post-processing indications separately by considering which information should be conveyed at a sequence level or at a picture level; if unified sequence/picture level SEI messages that convey all post-processing information are defined, signal the interlace indication in those SEI messages.

- Modifications to the AHG13 text: Move VUI flag to SPS and reduce the bit length of the field of interlace indication SEI message from 4 to 1;

- Modified access unit delimiter RBSP to indicate dynamically changing source material properties.

It is recommended that JCT-VC discusses the design of interlace indication and creates text during the 8th JCTVC meeting to be integrated into the CD.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).